3GPP TSG-RAN WG2 #99	R2-1708346

Berlin, Germany, 21st – 25th August 2017

Agenda Item:	10.3.2.4

Source:	Ericsson

Title:	Maximum number of RLC retransmissions in PDCP duplication

Document for:	Discussion, Decision

# Introduction

RAN2 has agreed that, for DC, radio link failure due to the maximum number of RLC retransmissions is kept as in LTE and each RLC entity triggers the event independently of each other. It is still open if the same behaviour applies for CA.

In June NR AH, the following related agreements and FFS were noted:

Agreements

1.	FFS in CA, as a baseline RLF is not triggered when reaching the maximum number of retransmission for a PDCP duplicate

2.	SNs of the two duplicate legs should be independently assigned

# Discussion

For data duplication in the CA case, RAN2 agreed to use a similar framework as for the DC case. Data duplication happens at the PDPC level. The PDCP entity delivers the PDCP PDU to two RLC entities. Each of this RLC entities will set its own headers including SN. From a logical point of view, when duplication is configured for the CA case, it is the same as configuring DC.

RLF due to reaching the maximum number of RLC retransmissions is a mechanism to ensure that the RRC protocol is informed when RLC AM protocol cannot ensure lossless transmissions i.e. when there are issues in the Radio Link.

From one angle, it can be argued that if one PDCP PDU has been received over one of the legs, it does not need to be transmitted any longer via the second leg. Thus, if the RLC PDU(s) associated to a PDCP PDU has/have been transmitted over one link a number equal to the “maximum number of RLC retransmissions” and, if that PDCP PDU has been received over the other link, there is no need to trigger a radio link failure. Discarding the associated RLC PDU(s) could be sufficient. To achieve this, RLC peers need to communicate and the impact is explained in [1]. Furthermore, to avoid RLC RLF triggering in practice, a high maximum number of retransmission value, which in LTE is 32, can be configured, which is considered a simple and effective approach, if this behaviour is wanted.

From another angle, PDCP data duplication does not have any relation with the fact that the RLC is not able to provide the committed service i.e. lossless transmissions. Reaching the maximum number of RLC retransmissions means that the RLC cannot provide lossless transmissions. Thus, this event should always be notified regardless of what upper layers are doing. If upper layers are not notified, the RLC entity will keep sending other RLC PDUs through a link which is not working, wasting resources and creating interference.

For CA, when PDCP duplication is configured, maximum number of RLC retransmissions triggers a RLF, similar as for the DC case.

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	For CA, when PDCP duplication is configured, maximum number of RLC retransmissions triggers a RLF, similar as for the DC case.

# References

R2-1708329, PDCP and RLC behaviour for PDCP data duplication, Ericsson, RAN2#99, Berlin, Germany, 21st – 25th August 2017