3GPP TSG-RAN WG2 #99	R2-1708347

Berlin, Germany, 21st – 25th August 2017

Agenda Item:	10.3.2.1

Source:	Ericsson

Title:	RLC Specification State Variable Naming

Document for:	Discussion, Decision

# Introduction

This contribution discusses about RLC specification state variable naming that was changed from LTE.

# Discussion

In recent RLC specification draft, there has been major change in variable naming compared to LTE. The new naming removes information that was previously associated with the variable names. For instance, from the variable name “RX_Next”, which is former “VR(R)”, it is impossible to say what the variable associated with is. On the contrary, from LTEs corresponding variable name VR(R), first R is indicating that the variable is associated with the receiver and later R is indicating that it is receive state variable. Disadvantage of LTE-styled variable naming is that it requires learning of the variable names as they initially may not seem intuitive.

In order to make the variable names intuitive, we propose to have variable names associated with the window position. This improves readability of the specification as the reader may associate the variable with the window behaviour.

Table 1: Variable name comparison

| LTE | Description | Current naming | Proposed change |
| --- | --- | --- | --- |
| VT(A) | Acknowledgement state variable | TX_Next_Ack | Tx_Lower_Window_Edge |
| VT(S) | Send state variable | TX_Next | Tx_Upper_Window_Edge |
| POLL_SN | Poll send state variable | POLL_SN | POLL_SN |
| VR(R) | Receive state variable | RX_Next | Rx_Lower_Window_Edge |
| VR(X) | t-Reordering state variable | RX_Next_Status_Trigger | Rx_Reordering_Sequence_Number |
| VR(MS) | Maximum STATUS transmit state variable | RX_Highest_Status | Rx_Max_Status_Sequence_Number |
| VR(H) | Highest received state variable | RX_Next_Highest_Rcvd | Rx_Upper_Window_Edge |

For the reader’s consideration, example is provided with the old variable naming and new variable naming

Section 5.1.3.2.3: Actions when an AMD PDU is placed in the reception buffer:

When an AMD PDU with SN = x is placed in the reception buffer, the receiving side of an AM RLC entity shall:

- if x >= RX_Next_Highest_Rcvd

-update RX_Next_Highest_Rcvd to x+ 1;

- if all bytes of the RLC SDU with SN = x are received:

- reassemble the RLC SDU from AMD PDU(s) with SN = x, remove RLC headers when doing so and deliver the reassembled RLC SDU to upper layer;

- if x = RX_Highest_Status,

- update RX_Highest_Status to the SN of the first RLC SDU with SN > current RX_Highest_Status for which not all bytes have been received;

-if x = RX_Next:

-update RX_Next to the SN of the first RLC SDU with SN > current RX_Next for which not all bytes have been received;

Section 5.1.3.2.3: Actions when an AMD PDU is placed in the reception buffer:

When an AMD PDU with SN = x is placed in the reception buffer, the receiving side of an AM RLC entity shall:

- if x >= Rx_Upper_Window_Edge

- update Rx_Upper_Window_Edge to x + 1;

- if all bytes of the RLC SDU with SN = x are received:

- reassemble the RLC SDU from AMD PDU(s) with SN = x,

remove RLC headers when doing so and deliver the reassembled RLC SDU to upper layer;

- if x = Rx_Max_Status_Sequence_Number,

- update Rx_Max_Status_Sequence_Number to the SN of the first RLC SDU with SN > current Rx_Max_Status_Sequence_Number for which not all bytes have been received;

- if x = Rx_Lower_Window_Edge:

- update Rx_Lower_Window_Edge to the SN of the first RLC SDU with SN > current Rx_Lower_Window_Edge for which not all bytes have been received;

When comparing two variable naming schemes, it may be noticed that in the later, the readability is significantly improved. For this reason, we propose:

Variable names proposed in Table 1 are taken into use in TS 38.322.

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	Variable names proposed in Table 1 are taken into use in TS 38.322.