3GPP TSG-RAN WG2 #99	Tdoc R2-1708349

Berlin, Germany, 21st – 25th August 2017

Agenda Item:	10.3.1.6

Source:	Ericsson

Title:	BSR formats in NR

Document for:	Discussion, Decision

# Introduction

Uplink scheduling is a key functionality to meet a broad range of use cases including enhanced mobile broadband, massive MTC, critical MTC, and additional requirements defined during the RAN requirements study. Details to how this functionality can be designed, compared to the baseline framework of LTE is discussed in contributions.

In #RAN2 98 some agreements related to BSR were reached.

Agreements

- The number of LCGs will be increased up to 8.

- The concept of periodicBSR-timer and retx-BSRtimers are reused and are configured per MAC entity

- As a baseline, the concept of logicalChannelSR-ProhibitTimer is reused in NR. It is allowed to configured infinite value for this timer.

- The logicalChannelSR-Mask is supported

In RAN2#AdHoc held on 27th – 29th June 2017, some agreements related to the BSR triggers were made

Agreements

=>	No new BSR triggers in addition to LTE are needed

Being based on the LTE BSR format as a baseline for NR, this contribution discusses the further potential enhancements for BSR format to enable the further progress.

# Discussion

## General Aspects

The concept of a logical channel group (LCG) was included in LTE to reduce the BSR signalling overhead in case a UE have many radio bearers established simultaneously. Currently up to four LCGs can be supported per UE in LTE and it has been agreed that NR shall support up to eight LCGs. The grouping of radio bearers (or logical channels) is established at radio bearer setup by the gNB based on QoS attributes of the radio bearers such as QoS Class Identifier (QCI). It has been agreed that NR reuses LTE BSR framework as the baseline, in other words, NR BSR is also triggered per LCG.

NR BSR is also triggered per LCG.

With more LCGs, the long BSR format of LTE, carries only up to 4 LCGs and will not be able to accommodate all LCGs buffer status in NR. Neither can the short BSR be reused since at least three bits will be needed to ID the LCGs and thereby leaving only 5 bits for the BSR which is inadequate.

In NR, the LTE BSR formats must be updated to effectively accommodate BSs for up to eight LCGs.

LTE supports 3 types of BSRs including Regular BSR, Periodic BSR as well as Padding BSR. For Padding BSR, if more than one LCG has data available for transmission in the TTI, a  Truncated BSR can be used which comprises BS of the LCG with the highest priority logical channel with data available for transmission. Different types of BSRs are applicable to different use cases. These use cases are still valid for NR.  Therefore, NR shall support all three types of BSRs as like LTE.

NR supports 3 types of BSRs including Regular BSR, Periodic BSR and Padding BSR.

Meanwhile, LTE has defined 3 BSR formats for different purposes, including short BSR, long BSR and truncated BSR. NR shall support all three formats of BSRs as like LTE.

NR supports short, long, and truncated BSR formats as like LTE

## Updated BSR formats

### Short BSR format

In LTE, two bits are used to indicate the LCG Id when using the short BSR format. The agreement of extending the number of LCGs up to eight will require 3 bits to uniquely identify the LCG. There are two options to update the short BSR format accordingly.

In the first option, as shown in Figure 1, the size of BS field can be decreased to 5 bits, to keep the short BSR as 1-byte long as like LTE. This would mean fewer levels of granularity for BS. A separate new BS value needs to be defined correspondingly.

Figure 1. Format of short BSR (option 1)

In the second option, as shown in Figure 2, together with the new size of BS field, i.e., 1-byte long, the size of short BSR is increased to 2 bytes to maintain the byte alignment. However, there are still leave 5 bits unused. These bits could e.g. be used to further extend the granularity of the BS in future releases.

Figure 2. Format of short BSR (option 2)

Between the two options, we think Option 1 is a better choice. It is advantageous to keep the same size of  the short BSR as LTE, and don’t introduce additional overhead. This is especially useful when there is a small grant received, e.g. during random access with a UE on the cell border.  It is also worth to note that the size of BS field in short BSR would be 5 bits, this give fewer granularities compared to that in LTE. This is acceptable since the short BSR is typically triggered when there is a small grant, the granularity of BS is less important compared to the bits saving.

The short BSR is 1 byte, consisting of a 3-bit LCG ID field and a 5-bit BS field.

### Long BSR format

In LTE, the long BSR include four BS fields corresponding to LCG IDs #0 through #3. All UEs that have more than one LCG with data available for transmission in the TTI where the BSR is transmitted must report Long BSR regardless whether they have data availble for all four LCGs. This may lead to redundant BSRs especially for UEs with limited uplink connection, in fact, the UEs may have fewer LCHs/LCGs (less than four)) that need to be configured.

It is further noted that 6 bits may not be sufficient to convey a buffer status with the granularity required for the wide range of services expected to be support by NR. The prime targets of NR include extreme MBB rates (10 Gbps), but also much lower rates e.g. from less extreme MBB or URLLC, and moreover an increased mix of high and low rate services. This could make the buffer sizes for the different LCGs to change rapidly and an increased granularity of the buffer sizes would be of great benefit for the scheduler. One way to encode such granularity would be to byte-align the fields and thereby simply extend the size of the Buffer Status field to 8 bits. For the long BSR format, it is suggested to increase BS field to 8 bits, which is not contradict with the BS field with 5 bits for short BSR. The long BSR is typically triggered when there is a sufficient grant, in this case, the accuracy of the BS report is more important.

The long BSR has a 1 byte BS field for each reported LCG ID

Following the same design principles, the long BSR format in NR would be like what is shown in Figure 3.  The issue of BSR overhead may become more problematic.

A Long BSR format consisting of one BS field for each of the eight LCGs has unnecessarily large overhead in cases when few LCGs are configured.

Figure 3. An example of updated long BSR carrying BS fields for 8 LCGs

Considering UEs may have different number of active services/LCHs, NR shall allow the number of LCGs to be configured per UE. Even for the same UE, the number of active services/LCHs may change from time to time. In this way, UEs are not required to be always configured with up to 8 LCGs. Then, a UE only report buffer status for its configured LCGs, which would be less than 8 LCGs, most of the time. The size of a long BSR may be different between UEs. Therefore, we suggest

The number of LCGs is configured per UE, and a long BSR can consist of BSs for less than 8 LCGs

As a correspondence, the long BSR format shown in Figure 3 can be updated to allow a variable size. At the same time, the MAC subheader must be also updated to include a L field as the MAC CE will be of varying length, indicating the actual size of the long BSR. Correspondingly, the L field indicates the number of LCGs that are configured and reported in the long BSR. The actual formats are shown in Figure 4.

Figure 4. Formats of the MAC subhead and the long BSR

### Truncated BSR format

In LTE, for Padding BSR:

-	if the number of padding bits is equal to or larger than the size of the Short BSR plus its subheader but smaller than the size of the Long BSR plus its subheader:

-	if more than one LCG has data available for transmission in the TTI where the BSR is transmitted: report Truncated BSR of the LCG with the highest priority logical channel with data available for transmission;

-	else report Short BSR.

-	else if the number of padding bits is equal to or larger than the size of the Long BSR plus its subheader, report Long BSR.

As can be seen, the truncated BSR carries only the BS information of the LCG with the highest priority logical channel.

As we proposed in above sections, NR keeps the same size of short BSR as LTE. It is straight forward to reuse the same rules for the Truncated BSR format.

As like LTE, the Truncated BSR uses the short BSR format

# Conclusion

In section 2 we made the following observations:

Observation 1	NR BSR is also triggered per LCG.

Observation 2	In NR, the LTE BSR formats must be updated to effectively accommodate BSs for up to eight LCGs.

Observation 3	A Long BSR format consisting of one BS field for each of the eight LCGs has unnecessarily large overhead in cases when few LCGs are configured.

Based on the discussion in Section 2 we propose the following:

Proposal 1	NR supports 3 types of BSRs including Regular BSR, Periodic BSR and Padding BSR.

Proposal 2	NR supports short, long, and truncated BSR formats as like LTE

Proposal 3	The short BSR is 1 byte, consisting of a 3-bit LCG ID field and a 5-bit BS field.

Proposal 4	The long BSR has a 1 byte BS field for each reported LCG ID

Proposal 5	The number of LCGs is configured per UE, and a long BSR can consist of BSs for less than 8 LCGs

Proposal 6	As like LTE, the Truncated BSR uses the short BSR format