3GPP TSG-RAN WG2 #99	Tdoc R2-1708352

Berlin, Germany, 21st – 25th August 2017	update of R2-1707175

Agenda Item:	10.3.1.8

Source:	Ericsson

Title:	General HARQ aspects of SPS UL

Document for:	Discussion, Decision

# Introduction

In RAN2-98AH, the following agreements were made in the chairman’s notes for SPS [1]:

=>	Modelling in the MAC for grant-free will be discussed after the difference between the two schemes is better understood pending RAN1 progress.  RAN2 will aim to have a unified MAC operation for common functionalities between grant-free and UL SPS with understanding that there can be differences after input from RAN1.

=>	RAN2 understands that to support UL SPS similar to LTE a mode of operation in which RRC configuration (with no initial PHY resources) with L1 activation/deactivation needs to be supported.  RAN2 will continue discussion on UL SPS, with LTE functionality.

=>	A common RRC signalling can be design to allow the configuration of different UL transmissions schemes.

Further, in the LS from RAN1 [2], two different types of UL transmission without grant have been agreed to be specified in RAN1. Type 1 is different from type 2 at least on the point that any L1 signalling is not required, and type 2 has some similarity with LTE UL SPS at least on the point that L1 signalling is used for activation/deactivation. RAN1 also agree for both types to support HARQ operations with repetitions, while further study is needed whether to configure multiple resources, and also to decide a timing reference for type 2. In addition, it is agreed in RAN1 that

•	NR supports more than 1 HARQ process for UL transmission without grant

Aslo, it is agreed in RAN1#85 that

•	NR should support at least asynchronous hybrid ARQ in the DL and UL to avoid fixed timing relationship between initial transmission and re-transmission

In this paper and paper [2], we discuss MAC operation for common functionalities related with HARQ operation. More specifally, this paper deals with general HARQ aspects of NR SPS, while a reliability enhancement feedback mode for NR SPS is provided in [2]. Some further SPS enhancements are discussed in [3] and our views on grant-free versus SPS (i.e., type 1 versus type 2) are in [4].

# Discussion

## Baseline HARQ feedback operation

In LTE UL SPS, the retransmissions could be performed on dynamic granted resources, in which gNB has the possibility to modify the grant and free up the configured resources for new low-latency data.

In addition, in LTE, for an UL transmission with grant, the UE can interpret whether an UL grant with NDI not-toggled is received or not as HARQ feedback. If an UL grant with NDI not-toggled (i.e., retransmission UL-grant) is received, then this indicates a failure in the gNB and UE can retransmit according to the UL-grant.

Similarly, for SPS UL, a reception of an UL-grant for retransmission should be supported and considered as a negative HARQ feedback. Due to the lack of a PHICH-like channel in NR, the feedback for UL transmissions has to be sent in PDCCH. But, to always send an explicit positive or negative feedback in PDCCH for each UL transmission induces a large signaling load in the downlink. For reduced load of the DL control channel, the negative feedback for UL transmissions can be skipped and replaced with UL-grant for retransmission without substantially degrading the UL transmission performance.

In order to differentiate the resource allocation information for SPS and dynamic scheduling, in LTE, an SPS-C-RNTI is used to mask resource allocation information over PDCCH. The UL-grant with NDI=1 and scrambled with SPS-C-RNTI is used to indicate the retransmission for SPS UL transmission. As type-2 according to RAN1 agreement is LTE SPS, a similar UL-grant format can be re-used as the baseline.

As in LTE SPS UL, retransmission for SPS UL transmission based on an UL grant is supported.. The UL-grant format in LTE is used as the baseline.

After UL transmission, data has to be kept in the HARQ buffer in case an UL grant for retransmission is received, and consequently no new data can be transmitted on the same HARQ process. To support transmission of any new data that can arrive in the UE buffer shortly after a first transmission, multiple HARQ processes should be supported. This will allow for low latency and increased throughput of SPS UL data. Multiple HARQ processes for SPS UL is also the case for LTE.

As in LTE SPS UL, multiple HARQ processes for SPS UL are supported in NR.

## Impact of non-explicit-HARQ feedback

In LTE, a fixed timing (n+4 or n+3) is configured so that UE knows implicitly when the feedback on PHICH will be sent and proper actions are taken depending on the feedback in PHICH. If ACK is sent on PHICH, then the UE can overwrite the buffer and transmit new data on SPS UL. In NR, unless e.g. a PDCCH indicating ACK is sent, UE keeps data in the buffer and no new data can be sent in SPS UL with the same HARQ PID. Note that, this issue is not relevant in dynamic scheduling since the transmission of the new data is triggered by an UL grant with NDI-toggled from the gNB.

In NR SPS, as one straightforward solution, UE can also keep the data in the buffer after the transmission and use other HARQ PIDs for new data transmsion. Once all HARQ PIDs are used and we need to reallocate a new HARQ PID, data buffer is flushed for new data. However, this puts a limitation that the UL-grant for retransmission needs to be sent before all HARQ PIDs are exhausted. Such a limitation might not be desired as SPS can be configured with short duration by mini-slot and higher numerology and with a small number of HARQ processes. As one example, we can configure only one HARQ process with a small periodicity for the UE. This is to target the use case with infrequent and small data transmission in URLLC. As in other example with dynamic TDD, in a pattern with UL-frame in most of the time, the allocated HARQ processes can quickly run-out.

As a more flexbile and forward-compatible solution to support sending new data in UL SPS, a maximum time T can be explicitly configured for the UE to wait for an UL grant for retransmission. This means that the UE keeps the data in the buffer, and waits for a retransmission UL grant during a maximum time T. If no UL grant is received with the same HARQ process ID when the timer expires, the UE assumes that the data is successfully delivered and it can reuse the HARQ process for new data. If a retransmission UL grant is received and it overlaps in time with the configured SPS grant, then the same behavior from LTE is adopted, that is, the dynamic UL grant overrides the configured SPS grant [6]. Morever, such a maximum time T can also be configured to wait for the positive-feedback as presented in the companion paper [3].

A timer should be reset after a transmission or retransmission, counting down from the time T.

In SPS UL, a time T after an UL transmission on a HARQ process is configured to wait for a retransmission UL grant for the same HARQ process.

If no retransmission UL grant is received within the time T, then UE considers the transmission successful and the HARQ process can be overwritten by new data.

This proposal implies that if the UE receives a UL grant while timer T is running, it shall perform a retransmission. This also applies to SPS. See the following figure as an example.

Figure 1  Illustration of time T

## HARQ PID

In LTE SPS and Fast UL framework (Rel-14) with multiple HARQ processes, the HARQ PID is not indicated. Formulas have been specified (TS 36.321) to derive the HARQ PID from the absolute system frame number (SFN) and subframe number (which is known in both eNB and UE). In other words, the HARQ PID is synchronously set, and eNB and UE are in-sync on which process ID to use on each TTI.

For configured uplink grants, the HARQ Process ID associated with this TTI is derived from the following equation for asynchronous UL HARQ operation:

HARQ Process ID = [floor(CURRENT_TTI/semiPersistSchedIntervalUL)] modulo numberOfConfUlSPS-Processes,

where CURRENT_TTI=[(SFN * 10) + subframe number] and it refers to the subframe where the first transmission of a bundle takes place.

This synchronicity may restrict both gNB’s flexibility in scheduling retransmissions and UE’s ability to send new data.

If “semiPersistSchedIntervalUL” is large, then for the same HARQ process, we have time to schedule a retransmission between two TTIs that have the same HARQ process IDs. Indeed, as long as the SPS is not configured in every slot, we can dynamically schedule a retransmission in between two SPS resources.

The problem arises when the SPS is configured in every slot and there is a clash between the HARQ process ID in the retransmission grant and the HARQ process ID in the configured SPS grant. As in LTE, the retransmission is on the next TTI that has the same HARQ PID as the initial transmission according to the formula. This introduces latency, as shown in the following example. In this example, the quickest retransmission according to gNB and UE’s processing timing can be available earlier than the scheduled TTI for that particular HARQ PID.

The retransmission can also be dynamically scheduled in another TTI. See the following figure as an example. The data in HARQ process 7 can potentially be overwritten or moved to the next available HARQ process 8. One issue is the TTI that is associated with HARQ PID=1 and comes next before any feedback/UL-grant from gNB is available. As per Proposal 4, a retransmission, which may be redundant, is performed.

As one solution of the above problems, we can configure the number of HARQ processes so that the time between two TTIs that have the same PID according to the formula is matched to the gNB and UE processing time. Following the example shown in the above, we can configure the number of HARQ processes to be equal to six.

“numberOfConfUlSPS-Processes” can be configured by RRC per UE to match the processing time between initial transmisions and retranmissions.

In the LS from RAN1 [2], it has been agreed to support HARQ operations with repetitions for SPS. The formula should also be able to handle the repetition factor K and assigning the HARQ PID according to the first transmission within the reptitions.  One example is shown in the following:

For configured uplink grants, the HARQ Process ID associated with this TTI is derived from the following equation for asynchronous UL HARQ operation:

HARQ Process ID = [floor(CURRENT_SUBFRAME/semiPersistSchedIntervalUL/Repetition factor K)] modulo numberOfConfUlSPS-Processes,

where CURRENT_SUBFRAME=[(SFN * 10) + subframe number] and it refers to the subframe where the first transmission of a bundle takes place.

Note that in LTE the repetition is supported under the case where the duration of the repetition is smaller than the periodicity of the UL SPS occasions. This is not anymore the case in NR.

In the following example, the interval is equal one, repetition factor is three and number of HARQ process is two.

“Repetition factor K” can be added into the formula to support reptition.

In addition, the formula-approach has some restrictions in supporting repetition. The starting time of the first copy of the K repetitions is restricted and this introduces an intial alignment delay. See the following Figure 2 as the example.

Figure 2 Inflexible repetition starting time

In order to support a flexible starting time, we can configure several formulas to the UE that support all possible starting time and utilize other resources, e.g., a different frequency allocation or a different cyclic shifts in DMRS, to signal the actual used formula to the gNB, i.e., the starting time of the repetition sequence.  See the following figure as an example.

Frequency resource or different cyclic shifts in DMRS can be used to indicate the starting timing of the repetition sequence.

However, this essentially means that the capacity is reduced.

Lastly, the formula need to address dynamic TDD case. Lets assume that transmission of HARQ PID#X has failed and the UE needs to retransmit it. In order to have the same HARQ PID the UE need to retransmit the data on the next occasion where the subframe# gives the same HARQ PID. However there is no guarantee that subframe will be an UL subframe. As one approach to tackle the problem, the formula can be re-defined that the HARQ process ID is computed from SPS occasions instead of the absolute subframe number.  One example is shown in the following:

HARQ Process ID = [floor(current_SPS occasion / Repetition factor K)] modulo numberOfConfUlSPS-Processes,

where current_SPS occasion is an interger number counting the number of the SPS UL opportunities from the first time an SPS UL is available after (re)-configuration.

This replacement also enable the configuration of SPS periodicity smaller than one subframe to support mini-slot and different sub-carrier spacing.

The subframe number in the formula is replaced by a counter of the UL SPS occasions to support dynamic TDD and periodicity smaller than one subframe.

In summary, in order to support functionalities introduced in NR, such as dynamic TDD, repetition, periodicity shorter than one subframe, etc. We propose that

In SPS UL, HARQ PID is derived from UL SPS occasions, frequency resources and DMRS resources. In addition, the repetition is supported.

### Limitation of formula based approach

For NR, asynchronous HARQ feedback has been agreed in RAN1#85 such that "NR should support at least asynchronous hybrid ARQ in the DL and UL to avoid fixed timing relationship between initial transmission and re-transmission”. A commonly-agreed formula to specify the HARQ PID to use at each time is essentially a synchrnoized hybrid ARQ. One can argue that gNB can configure different timing for UEs with “numberOfConfUlSPS-Processes”, and avoids a fixed timing relationship between initial transmission and re-transmission. However, this still indicates that the timing is fixed for a particular UE over a period of time. Further, it requires that if gNB wants to have a different timing, it needs a new RRC configuration.

In a dynamic TDD case where the consecutive UL frames are long separated apart in time, this introduces unnecessary latency for retransmission due to the waiting time for the next SPS occasion with the same HARQ PID. One can also argue that we can configure  “numberOfConfUlSPS-Processes” to have a smaller number to match the processing time at gNB and UE. This, however, also requires a RRC configuration.

Lastly, as argued above, in order to support repetition, either cyclic shifts in DMRS or frequency resources are utilized to signal a different starting position. This leads to the capacity loss.

Formula-based HARQ PID does not support full asynchronous HARQ, introduces delay in dynamic TDD and capacity loss when configured with repetition.

# Conclusion

For general HARQ aspects of SPS UL in NR, we made the following observation:

Observation 1	“numberOfConfUlSPS-Processes” can be configured by RRC per UE to match the processing time between initial transmisions and retranmissions.

Observation 2	“Repetition factor K” can be added into the formula to support reptition.

Observation 3	Frequency resource or different cyclic shifts in DMRS can be used to indicate the starting timing of the repetition sequence.

Observation 4	The subframe number in the formula is replaced by a counter of the UL SPS occasions to support dynamic TDD and periodicity smaller than one subframe.

Observation 5	Formula-based HARQ PID does not support full asynchronous HARQ, introduces delay in dynamic TDD and capacity loss when configured with repetition.

We propose the following:

Proposal 1	As in LTE SPS UL, retransmission for SPS UL transmission based on an UL grant is supported.. The UL-grant format in LTE is used as the baseline.

Proposal 2	As in LTE SPS UL, multiple HARQ processes for SPS UL are supported in NR.

Proposal 3	In SPS UL, a time T after an UL transmission on a HARQ process is configured to wait for a retransmission UL grant for the same HARQ process.

Proposal 4	If no retransmission UL grant is received within the time T, then UE considers the transmission successful and the HARQ process can be overwritten by new data.

Proposal 5	In SPS UL, HARQ PID is derived from UL SPS occasions, frequency resources and DMRS resources. In addition, the repetition is supported.

# References

RAN2 Adhoc 2017 June, Qingdao, Chairman notes

R1-1711686, LS on NR UL SPS / UL transmission without UL grant

R2-1708353, Enhanced HARQ feedback mode in SPS

R2-1708351, SPS enhancements in NR

R2-1708350, Grant Free and Semi-Persistent Scheduling in NR

R1-1714445, On Pre-emption Indication for Uplink