3GPP TSG-RAN WG2 #99                                                                                     R2-1708360

Berlin, Germany, 21 - 25 Aug 2017

Source:	CATT

Title:	Cross check on APDC

Agenda Item:	9.3

Document for:	Discussion and Decision

Introduction

In past several RAN2 meetings, Zlib/Deflate and APDC solution were proposed to be the candidate UDC solutions and were evaluated. In last RAN 76 meeting, it is concluded that additional information on APDC solution should be provided to RAN2, and RAN2 will do corresponding analysis on APDC solution and RAN #77 in Sep.17 will decide between DEFLATE-based solution and APDC solution.

In this paper, APDC simulation results based on the source codes provided by Qualcomm are provided and some observations are given.

Discussion

- 2.1    APDC results

The APDC performance is evaluated in this part with the source code from QualComm. We assumed to use the default parameter in the source code. For APDC_COMPRESSION_MEMORY_SIZE definition, e.g. for 8K buffer, we could set APDC_COMPRESSION_MEMORY_SIZE to 8192, and for 32K buffer, we could set the APDC_COMPRESSION_MEMORY_SIZE to 32768.

However, according to the APDC design shown in TR 36.754, valid value range for Lookback length (14bit) of CPCR header field is from 4 to 16383, which means APDC can support the compression buffer size up to 16Kbyte buffer.

Observation 1: APDC could not support 32K byte buffer.

Hence, in this evaluation, we only provided the 8k size buffer simulation, i.e. set APDC_COMPRESSION_MEMORY_SIZE to 8192.

Table-1: APDC results based on disclosed source codes

| Input file | Original Size（Byte） | Results in TR 36.754 [1] |  | 8Kbyte buffer of APDC |  | Differences |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Compressed Size（Byte） | Compression Efficiency | Compressed Size（Byte） | Compression Efficiency |  |
| Input traffic 1: FTP data-client-CMCC | 1211 | 548 | 54.74% | 553 | 54.34% | -0.40% |
| Input traffic 2: FTP data-server-CMCC | 1782 | 884 | 50.39% | 885 | 50.34% | -0.05% |
| Input traffic 3: SIP signalling-CMCC UE 1 | 51020 | 7337 | 85.61% | 8208 | 83.91% | -1.70% |
| Input traffic 4: SIP signalling-CMCC | 32680 | 5827 | 82.16% | 6332 | 80.62% | -1.54% |
| Input traffic 5: SIP signalling-CMCC | 46688 | 6561 | 85.94% | 7377 | 84.20% | -1.74% |
| Input traffic 6: Video data-CMCC (duration: ~6s) | 13450 | 5105 | 62.04% | 5246 | 61.00% | -1.04% |
| Input traffic 7: Web surfing-CMCC | 2381720 | 767990 | 67.75% | 851605 | 64.24% | -3.51% |
| Input traffic 8: Long period Video data-CMCC (duration: ~6min) | 1371861 | 295658 | 78.44% | 319995 | 76.67% | -1.77% |
| Input traffic 9: Video data-MTK (duration: ~1hr) | 2453749 | 638347 | 73.98% | 651029 | 73.47% | -0.51% |
| Input traffic 10: Long period ftp-MTK | 879630 | 216910 | 75.34% | 216959 | 75.34% | 0.00% |
| Input traffic 11: Multiple IP flows-QC | 5319100 | 1312299 | 75.32% | 1417499 | 73.35% | -1.97% |

From the results, using disclosed codes, APDC can still achieve significant UDC gain although there are some performance losses compared with the results in TR.

Observation 2: APDC can achieve significant UDC gain by using disclosed compression source codes.

Observation 3: Some differences can be observed (0~3.51%) compared with the APDC simulation results in TR 36.754.

- 2.2    Comparison of solutions

In this part, performance comparison among solutions is provided. Note that the static Huffman encoding with 1byte UDC header in Deflate solution is considered; the APDC results are simulated by CATT based on the disclosed source codes; the RoHC results are copied from TR 36.754[1]. The best efficiency is highlighted in yellow respectively for 8Kbyte buffer and 32Kbyte buffer. For the comparison of 32k buffer cases, 8Kbyte buffer results of APDC are used due to absence of 32Kbyte buffer results.

Table-2 Performance comparison

| Input file | 8Kbyte buffer compression efficiency |  |  | 32Kbyte buffer compression efficiency |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Deflate | APDC | RoHC | Deflate | APDC | RoHC |
| Input traffic 1: FTP data-client-CMCC | 49.96% | 54.34% | 73.30% | 49.96% | 54.34% | 73.30% |
| Input traffic 2: FTP data-server-CMCC | 44.61% | 50.34% | 59.70% | 44.61% | 50.34% | 59.70% |
| Input traffic 3: SIP signalling-CMCC UE 1 | 86.50% | 83.91% | 5.40% | 87.95% | 83.91% | 5.40% |
| Input traffic 4: SIP signalling-CMCC | 83.79% | 80.62% | 5.10% | 84.87% | 80.62% | 5.10% |
| Input traffic 5: SIP signalling-CMCC | 86.85% | 84.20% | 4.40% | 88.25% | 84.20% | 4.40% |
| Input traffic 6: Video data-CMCC (duration: ~6s) | 62.98% | 61.00% | 21.70% | 62.99% | 61.00% | 21.70% |
| Input traffic 7: Web surfing-CMCC | 65.20% | 64.24% | 23.10% | 70.03% | 64.24% | 23.10% |
| Input traffic 8: Long period Video data-CMCC (duration: ~6min) | 71.26% | 76.67% | 45.10% | 73.75% | 76.67% | 45.10% |
| Input traffic 9: Video data-MTK (duration: ~1hr) | 59.08% | 73.47% | 80.70% | 57.92% | 73.47% | 80.70% |
| Input traffic 10: Long period ftp-MTK | 62.01% | 75.34% | 83.40% | 58.56% | 75.34% | 83.40% |
| Input traffic 11: Multiple IP flows-QC | 71.63% | 73.35% | -- | 73.79% | 73.35% | -- |
| Input traffic 7+8:average mixed | 65.31% | 63.00% | -- | 70.11% | 63.00% | -- |
| Input traffic 7+8:one inserted in another one | 67.42% | 68.77% | -- | 71.40% | 68.77% | -- |
| Input traffic 7+8:random mixed | 65.30% | 64.80% | -- | 70.24% | 64.80% | -- |

Observation 4: For 8Kbyte buffer, Deflate is better in 7 out of 14 cases, while APDC is better in 3 cases and RoHC is better in  4 cases; for 32Kbyte buffer, Deflate is better in 9 out of 14 cases, while APDC is better in 1 case and RoHC is better in 4 cases.

Conclusion

From the APDC evaluation by using disclosed source codes, it is observed that,

Observation 1: APDC could not support 32K byte buffer.

Observation 2: APDC can achieve significant UDC gain by using disclosed compression source codes.

Observation 3: Some differences can be observed (0~3.51%) compared with the results in TR 36.754.

Observation 4: For 8Kbyte buffer, Deflate is better in 7 out of 14 cases, while APDC is better in 3 cases and RoHC is better in  4 cases; for 32Kbyte buffer, Deflate is better in 9 out of 14 cases, while APDC is better in 1 case and RoHC is better in 4 cases.

According to observation 1, the buffer size should be clarified in the TR 36.754 for APDC. Corresponding CR can be found in [2].

Reference

[1]	TR 36.754, Study on UL data compression for E-UTRA.

[2]	R2-1708361, Clarification on Buffer Size for APDC	 CATT