3GPP TSG-RAN WG2 Meeting #99	R2-1708545

Berlin, Germany, 21 - 25 Aug 2017

Agenda item:	9.4.4

Source: 	Huawei, HiSilicon

Title: 	Measurement Report Mechanism for Drones

Document for:	Discussion

Introduction

In RAN2#98 meeting, potential enhancements for UAV interference problem have been discussed, and for measurement report the following agreements have been made:

- Agreements 1 Study the enhancement of measurement report mechanism to address the issue of increased number of strong neighbour cells.

In this contribution, we propose several solutions for measurement report mechanism regarding to increased number of strong neighbour cells.

Discussion

As drone UE flying at altitude, it can detect more neighbour cells. Because the transmission channel is line of sight without obstructions that may block the signal, and the path loss only depends on the distance between eNB and UE, it is possible that most of these neighbour cells may be strong cells. And these cells could trigger measurement reporting frequently, e.g. event A4. Based on the specification, the measurement report should include the best neighbouring cells up to maxReportCells. If there are many neighbor cells triggering the measurement report for the same measID, due to the limited neighbour cell number, i.e. 8 neighbour cells at most, the other neighbour cell may not be included in measurement report only if the leaving condition applicable for this event is fulfilled for the top best cells. So one extreme case is that one cell triggered the measurement report, but its RSRP is lower than the top maxReportCells cells, and then the measurement result of this neighbour cell will not be included in measurement report. Therefore, there are two issues to be resolved:

Issue 1: More triggering cell included in the measurement report

Issue 2: frequent measurement report triggering

To handle the issue 1, and make sure the trigger cell should be included in measurement report, we propose:

Proposal 1: enlarge the value of maxReportCells to accommodate more neighbor cells.

DCM has proposed a solution to handle this issue 2 [1], in which the reporting is triggered if the entering/leaving condition is fulfilled only when multiple applicable cells have met the event triggering criteria. This can reduce the trigger number to a certain extent, but there may be one case in which the number of applicable cells does not reach the required threshold at the same time, so the measurement report may not be transmitted in time. In addition to the aforementioned solution, a max delay restriction should be applied to guarantee a timely report as illustrated in Fig.1. A timer can be started when one cell fulfils the entering/leaving condition, if the timer expires the measurement report should be triggered no matter how many cells fulfilled the condition.

Fig.1 timer for max measurement report delay

Proposal 2: a max time delay after the first cell triggering measurement report can be set for timely measurement report transmission.

Fig.2 different time to trigger depending on altitude

To reduce the trigger number of measurement report, eNB can configure different values for time to trigger which can be used depending on drone UE altitude. In that case when UE is flying above eNB it can use a long time-to-trigger to tolerate the fluctuation of radio link quality of neighbour cells. And when the drone lowers down the height until under the eNB, it can use a normal value of time to trigger as illustrated in Fig.2.

Proposal 3: different values of time-to-trigger can be configured to drone UE depending altitude.

Conclusion

By analysing the measurement issues for drones we have the following proposals:

Proposal 1: enlarge the value of maxReportCells to accommodate more neighbour cells.

Proposal 2: a max time delay after the first cell triggering measurement report can be set for timely measurement report transmission.

Proposal 3: different values of time-to-trigger can be configured to drone UE depending altitude.

Reference

[1]       R2-1704333 Initial views on potential problems and solutions for aerial vehicles, NTT DOCOMO INC.