3GPP TSG-RAN WG2 #99	Tdoc R2-1708581

# Berlin, Germany, 21st – 25th August 2017

Agenda Item:	10.4.1.4.7

Source:	Ericsson

Title:	ANR framework in NR

Document for:	Discussion, Decision

# Introduction

Since the first release of LTE, Automatic Neighbour Relation (ANR) functionality has been introduced to automatically generate the relations between radio network entities. Such relations are used to establish connections between entities, support mobility, load balancing, dual connectivity, etc., and ANR is seen as significantly reducing the planning and operation costs for operators [2]. Therefore, it is our understanding that automatic establishment of relations will continue to be an important feature in NR networks. One can expect that the challenging NR requirements will result in an even higher emphasis on the need for seamless mobility than in legacy RATs, which further increases the importance of automatic relation establishment in NR. In addition, an extensive use of beamforming, together with a lean design of NR will bring new challenges and additional scope to the relation management.

- Detection of neighbor cell for measurement is based on NR-SS

In RAN1#88, it was agreed that;

In this paper, further details of how the UE could aid the neighbour relation establishment process is provided and scenarios wherein the UE fails to identify a neighbouring cell is provided.

# Discussion

## Identifiers for ANR establishment

In LTE, a UE detects cells based on their PSS/SSS, which encode a Physical Cell Identifier (PCI). An important property of these signals is that the UE autonomously detects the neighbour cell IDs from an acquired PSS/SSS, i.e., the network does not need to provide a neighbour cell list. UEs typically detect and measure neighbour cells by sampling a short time window (e.g. 5 ms) on the target frequency (which may be the same or different from serving) and search (possibly offline) for PSS/SSS occurrences within that sample. For each detected PSS/SSS, the UE can also perform a measurement using the CRS corresponding to the PCI. The result of that action is a list of neighbour cell IDs and corresponding measurement sample.

Once the UE performs measurements, mobility events can trigger the transmission of measurement reports. Based on these reports the serving eNB is able to identify that a given PCI belongs to a neighbour eNB in order to trigger a handover preparation procedure. In order to do that, the serving eNB needs to maintain a Neighbour Relation Table that maps locally unique identifiers, i.e. the PCIs in LTE, with globally unique identifiers, i.e. the E-UTRAN Cell Global identifier (E-CGI). E-CGI is defined in LTE as a combination of PLMN ID (Combination of country code and network code) and ECI (combination of eNB ID and cell ID).

The procedure is illustrated in Figure 1. In step 1, the UE sends a measurement report to the serving eNB, containing the PCI of cell B. The eNB then instructs the UE to read the ECGI, the TAC and all available PLMN ID(s) of cell B. The neighbour cell broadcasts its PLMN IDs, TAC etc. in the SIB1 message. After measuring these values, the UE reports the detected ECGI to the serving eNB, and the eNB decides to add this neighbour relation. The eNB can use the PCI and ECGI to lookup a transport layer address to the new eNB, update the Neighbour Relation list, and, if needed, setup a new X2 interface towards this eNB [1].

Figure 1. Automatic Neighbour Relation Function in LTE [1]

- In LTE, the UE reports to the serving eNB measurement reports associated to locally unique identifiers (e.g. PCIs).

- For handover decisions based on UE reporting of local IDs, a Neighbour Relation Table (NRT) mapping local IDs to global IDs (e.g. E-CGI) is necessary to identify the target eNB in order to send a handover preparation and/or a handover request.

- In LTE, a serving eNB can request the UE to acquire a global identifier (e.g. ECGI), TAC, and all available PLMN ID(s) associated to a previously reported local identifier (PCI).

- In LTE, a globally unique identifier (ECGI) is broadcasted in SIB1 and can be associated to a locally unique identifier (PCI).

In LTE, this Global ID, the ECGI, is not necessary as often as the PCI. Therefore, it is transmitted in SIB1 every 80 ms while the PCI is transmitted every 5ms.

- In LTE, a globally unique identifier (ECGI) is broadcasted with a larger periodicity than the locally unique identifier (PCI).

In RAN2#97 NR meeting in Athens, it was noted that an RRC_CONNECTED UE should be able to perform RRM measurements on the so-called IDLE RS, later renamed to an NR-SS Block.

An RRC_CONNECTED UE should be able to perform RRM measurements on always on IDLE RS (e.g. synchronization signal).

The measurement reports based on SS Blocks including cell measurement results will likely be associated to local identifiers (like the LTE PCIs) that the UE has measured. Based on such a measurement report, if the serving gNodeB does not recognize the reported PCI in its neighbour relation list, then the serving gNodeB could reuse the globally unique identifier , TAC and PLMN ID(s) measurement and reporting procedure for the UE, as listed in LTE, for NR neighbour relation establishment also.

- The RRM measurements on SS Blocks are expected to be reported as per the measured PCI values by the UE.

- The neighbour relation establishment procedure based on the UE reporting the globally unique identifier , TAC and all available PLMN ID(s) of the unknown cell as listed in LTE can be re-used for NR also.

It is our understanding that the information required to derive the globally unique identifier, GID, are provided in NR as part of the SIB1 message as well. However, this has not been agreed yet. It can be discussed further as to how is the GID is structured (number of bits, what sub-identifiers does it encompass in what position like PLMN ID, CID etc. ) in NR and if it is of similar nature as ECGI of LTE or not.

- The information elements required to derive the Global Identifier, GID are provided in NR as expected to be part of the SIB1 message.

As in LTE, network can configure the UE to read and report the neighboring cell’s Global identifier (GID), TAC and all available PLMN ID(s) to aid neighbor relation establishment procedure in NR.

    - It is FFS as to whether structure of GID is same as the structure of ECGI in LTE or not.

It should further be straightforward to reuse the same solution for detecting NR neighbours from LTE using IRAT ANR functionality.

LTE IRAT ANR procedure should support for UEs to read and report the neighboring NR cell’s Global identifier (GID), TAC and all available PLMN ID(s) to aid neighbor relation establishment.

# Conclusion

In the previous section, the following has been observed:

- In LTE, the UE reports to the serving eNB measurement reports associated to locally unique identifiers (e.g. PCIs).

- For handover decisions based on UE reporting of local IDs, a Neighbour Relation Table (NRT) mapping local IDs to global IDs (e.g. E-CGI) is necessary to identify the target eNB in order to send a handover preparation and/or a handover request.

- In LTE, a serving eNB can request the UE to acquire a global identifier (e.g. ECGI), TAC, and all available PLMN ID(s) associated to a previously reported local identifier (PCI).

- In LTE, a globally unique identifier (ECGI) is broadcasted in SIB1 and can be associated to a locally unique identifier (PCI).

- In LTE, a globally unique identifier (ECGI) is broadcasted with a larger periodicity than the locally unique identifier (PCI).

- The RRM measurements on IDLE RSs are expected to be reported as per the measured PCI values by the UE.

- The neighbour relation establishment procedure based on the UE reporting the globally unique identifier , TAC and all available PLMN ID(s) of the unknown cell as listed in LTE can be re-used for NR also.

- The information elements required to derive the ECGI (PLMN ID and the Cell Identity) are provided in NR as expected to be part of the SIB1 message.

Based on these observations, we have proposed the following:

- As in LTE for the ECGI reading and reporting procedure, a UE in NR should be able to read and report the neighboring cell’s Global identifier (GID), TAC and all available PLMN ID(s) to aid neighbor relation establishment procedure in NR.

    - It is FFS as to whether structure of GID is same as the structure of ECGI in LTE or not.

- LTE IRAT ANR procedure should support for UEs to read and report the neighboring NR cell’s Global identifier (GID), TAC and all available PLMN ID(s) to aid neighbor relation establishment.

# References

3GPP TS 36.300 V13.4.0 (2016-06), “Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description”

A. Dahlén, A. Johansson, F. Gunnarsson, J. Moe, T. Rimhagen, H. Kallin, ”Evaluations of LTE Automatic Neighbor Relations”, in Proc. IWSON at VTC Spring, Budapest, Hungary, 2011.