3GPP TSG-RAN WG2 #99	R2-1708729

Berlin, Germany, August 21st-25th, 2017	Revision of R2-1706682

Agenda Item:	10.3.1.7

Source:	InterDigital Inc.

Title:	LCP for LCHs with Multiple RRC Configured Mappings

Document for:	Discussion, Decision

# Introduction

RAN2 has agreed that a single NR MAC entity can support one or multiple TTI durations/numerologies. The logical channel prioritization procedure then takes into account a RRC configured mapping between a logical channel and applicable numerologies and/or TTI duration(s) [1]. In RAN2#97bis and RAN2#98, the details of the LCP procedure were discussed further and the following agreements were reached [2][3]:

RAN2#97 Agreements

- For the purpose of LCP, the MAC entity learns the TTI duration/numerology from the PHY layer.  FFS on the details of how it is signalled

- Priority, PBR concept is used in NR as a baseline.

RAN2#98 Agreements

- For LCP and to know which restrictions to use the MAC needs to be aware of more information than just TTI length (e.g. numerology). A transmission based on index or profiles can be supported.   Exact parameters are FFS.

- Logical Channel Priority is configured per MAC entity per logical channel

- PBR is not configured per numerology, it is per “logical channel” as in LTE

- Bj is calculated per logical channel. It is up to UE implementation to ensure that Bj is updated at the right time.

This contribution further discusses the LCP procedure in NR for logical channels mapping to multiple transmission profiles. A text proposal to TS 38.321 [4] is included in the Appendix. Our companion contributions [11, 12] discuss how to perform logical channel selection in LCP for a new transmission.

# Contending for Resources in LCP Steps 1,2,3 vs. Step 3 Only

Mapping a single logical channel to multiple transmission profiles provides the scheduler with more flexibility to achieve higher spectral efficiency at high loads (e.g. using unused resources of short data transmission durations for eMBB when possible). A scheduler implementation may benefit from scheduling eMBB traffic using shorter TTIs once higher priority data is exhausted. For example, this may be used to shorten the slow start phase for small TCP transfers, or simply to maximize resource allocation in a cell that supports multiple numerologies and/or TTI durations.

RAN2 has agreed to support UE-specific LCH priorities based on the assumption that services associated with the transmission profile of the most stringent QoS level should always be given the highest priorities, even for LCHs configured with multiple mappings. This implicitly assumes that the gNB will use separate priority ranges per transmission profile to enforce proper mapping of data for different services that can map to multiple transmission profiles. However, the purpose of the priorities between LCHs is to characterize the relative importance of the data across different services to each other, and not to enforce different grants to be served differently according to their relative transmission properties.

- Observation 1:	Configuring a LCH with multiple transmission profiles does not necessarily imply its data should contend equally on resources of different transmission profiles.

LCP should therefore not rely on LCH priorities to ensure that services that best map to a given transmission profile be always served first for that transmission profile. Further, setting the PBR of a LCH with higher priority to infinity is not a flexible method to enforce such mapping, given that this may lead to flow starvation for other services that can only contend in that same transmission profile.

During RAN2#97bis and RAN2#98, it was mentioned that logical channels that map to multiple transmission profiles may be scheduled in Step 1 on resources that are more optimal for higher priority logical channels with remaining data for transmission. This happens as a result of Step 1 assigning resources to all logical channels that map to the grant’s transmission profile in decreasing order of priority, and according to the Bj value set by the LCH's PBR. This is performed as stated in 38.321 [4]:

-	Step 1: Relevant logical channels for the UL grant with Bj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to “infinity”, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s);

This may be not always yield a desirable UE behaviour when a lower priority logical channels configured with multiple transmission profiles maps to a resource that is more optimally used by higher priority logical channels. These LCHs will occupy some resources already in Step 1, even though data is available from other higher priority/low-latency LCHs which would have been otherwise obtained in Step 3.

- Observation 2:	LCP should support that LCHs configured with multiple transmission profiles may contend differently for resources in Step 1.

One alternative would be to avoid mapping such LCHs to transmission profiles used by higher priority/lower latency data. However, this would completely rule out that the possibility that the LCH uses such resources in Step 3, even if they are vacant and no other LCHs are contending on them in Step 3, thereby defeating the intent of supporting multiple mappings.

One alternative discussed in [5] and [6] is to configure multiple PBR values for a LCH that maps to multiple transmission profiles. For example, a large data rate can be configured for the PBR of a first numerology and a PBR of 0 for a secondary numerology used for higher priority LCHs. In such case, other LCHs of higher priority for a certain numerology are first served up to their PBR, and also get any remaining resources they may use before a lower priority LCH is allocated any resources on that numerology. However, the UE has to maintain a Bj parameter for each transmission profile, and it was agreed to use a single PBR value per LCH in RAN2#98.

Given the current agreements, another alternative is to set PBR to infinity for higher priority LCHs. However, this does not provide any means to distinguish in terms of guaranteed bit rates among those LCHs. This results in splitting the UL resource equally among high priority LCHs if the resource is not sufficient to serve all their respective data available for transmission, regardless of the QoS profiles of their underlying applications.

Consequently, it is proposed to introduce a differentiation between Steps 1,2 and Step 3 per transmission profile for LCHs that maps to multiple transmission profiles, as follows:

- Proposal 1:	A LCH always contend for transmission resources in all steps of the LCP procedure for at least one transmission profile.

- Proposal 2:	A LCH can be configured to contend for transmission resources only in Step 3 of the LCP procedure for a given transmission profile.

- Proposal 3:	RRC configures whether or not a LCH contends for transmission resources only in Step 3 of the LCP procedure for a given transmission profile.

# Conclusion

RAN2 should discuss the aforementioned observations and agree to the following proposals:

- Observation 1:	Configuring a LCH with multiple numerologies/TTIs does not necessarily imply its data should contend equally on resources of different numerologies/TTIs.

- Observation 2:	LCP should support that LCHs configured with multiple mappings may contend differently for resources of different numerologies/TTI durations.

- Proposal 1:	A LCH always contend for transmission resources in all steps of the LCP procedure for at least one transmission profile.

- Proposal 2:	A LCH can be configured to contend for transmission resources only in Step 3 of the LCP procedure for a given transmission profile.

- Proposal 3:	RRC configures whether or not a LCH contends for transmission resources only in Step 3 of the LCP procedure for a given transmission profile.

- A text proposal for these proposals is in Appendix A, using draft TS 38.321 v0.2.0 [4] as a baseline.

# Appendix A – Text Proposal for TS 38.321 based on draft v0.2.0

<Start text proposal, using TS 38.321 v0.0.4 as baseline>

#### 5.4.3.1	Logical channel prioritization

[Unchanged text not included] RRC controls the scheduling of uplink data by signalling for each logical channel per MAC entity: - priority where an increasing priority value indicates a lower priority level; - prioritisedBitRate which sets the Prioritized Bit Rate (PBR); - bucketSizeDuration which sets the Bucket Size Duration (BSD); - stepThreeOnly which determines what LCP steps are applicable. [Editor’s note: stepThreeOnly is configured per transmission profile] [Unchanged text not included] The MAC entity shall, when a new transmission is performed: 1> allocate resources to the logical channels in the following steps: - Step 1: Relevant logical channels for the UL grant with Bj > 0 and stepThreeOnly == false are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to “infinity”, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s); Editor's note: compared to LTE, 'All the logical channels' is replaced with 'Relevant logical channels for the UL grant’. - Step 2: the MAC entity shall decrement Bj by the total size of MAC SDUs served to logical channel j in Step 1; NOTE: The value of Bj can be negative. - Step 3: if any resources remain, all the relevant logical channels are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally. Editor's note: the wording 'relevant' needs to be further clarified after having concrete RAN2 agreements (by considering e.g. numerology/TTI, packet duplication, transmission profile, etc.). [Unchanged text not included]

# References

3GPP TS 38.300 V0.6.0, “NR; NR and NG-RAN Overall Description; Stage 2 (Release 15)”.

“RAN2 Chairman’s Notes,” 3GPP TSG RAN WG2 #97bis, Spokane, USA, April 3rd-7th, 2017.

R2-1704030, “Report from NR/LTE Break-Out Session (UP NR, FeD2D, sTTI)”, RAN2 #98.

R2-1707510, 3GPP TS 38.321 V0.2.0, “NR; Medium Access Control (MAC) protocol specification (Release 15)”.

R2-1704903, “LCP with Multiple Numerologies”, Qualcomm Inc.

R2-1702871, “Logical Channel Prioritization for NR”, InterDigital Inc.

R2-1704397, “Further aspects on LCP”, Ericsson.

R2-1704911, “Details of the Logical Channel Prioritization Procedure for NR”, InterDigital Inc.

R2-1705624, “Remaining issues on LCP with Multiple Numerologies”, Huawei.

R2-1704504, “Visibility of numerology for NR MAC functions”, Samsung.

R2-1708728, “LCH Selection in LCP based on Transmission Profiles”, InterDigital Inc.

R2-1708727, “LCH Selection in LCP based on TTI Duration”, InterDigital Inc.