3GPP TSG-RAN WG2 #99	R2-1708771

Berlin, Germany, 21 - 25 August 2017

Agenda item:	10.3.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Fixed and extension part in RLC PDUs

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, it is proposed that the concept of fixed and extension parts is no more applied for RLC PDUs in NR.

# 2	Discussion

LTE RLC TS 36.322 includes the concept of fixed and extension parts in the UMD and AMD PDUs. Depending on the PDU type, the fixed part may consist of a D/C, a RF, a P, a FI, an E, a SN, a LSF and a SO field whereas the extension part (if exists) consist of E(s) and LI(s). I.e., the extension part’s length varies based on the number of RLC SDUs concatenated in the PDU while the fixed part length is fixed based on the used UMD or AMD PDU (segment) type.

As the NR RLC does not support concatenation, it is no longer meaningful to retain the concept of fixed and extension parts for the UMD and AMD PDUs to be defined.

Proposal 1: Remove the concept of fixed and extension parts of the UMD and AMD PDUs.

Proposal 2: Agree the text proposal provided in Annex for TS 38.322.

# Annex: Text proposal

*** Start of change ***

#### 6.2.1.3	UMD PDU

UMD PDU consists of a Data field and an UMD PDU header.

UMD PDU header consists of:

- a SI for a complete RLC SDU;

- a SI and a SN for a first segment of a RLC SDU;

- a SI, a SN, and a SO for a middle and last segment of a RLC SDU.

The UMD PDU header

is byte aligned.

Figure 6.2.1.3-1: UMD PDU containing a complete RLC SDU

Editor’s note: To be updated based on agreements on supported RLC SN lengths, and relative position of SI and reserve bits

#### 6.2.1.4	AMD PDU

AMD PDU consists of a Data field and an AMD PDU header.

AMD PDU header is byte aligned and consists of:

-	a D/C, a P, a SI, and a SN for a complete RLC SDU and a first segment of RLC SDU;

-	a D/C, a P, a SI, a SN and a SO for a middle and last segment of a RLC SDU.

An AM RLC entity is configured by RRC to use either a 12 bit SN or a 18 bit SN. The default value for SN field length used by an AM RLC entity is 12 bits. The SO field is 16 bits.

Figure 6.2.1.4-1: AMD PDU with 12 bit SN (No SO)

Figure 6.2.1.4-2: AMD PDU with 18 bit SN (No SO)

Figure 6.2.1.4-3: AMD PDU with 12 bit SN with SO

Figure 6.2.1.4-4: AMD PDU with 18 bit SN with SO

Editor’s note: The relative placement of P and SI bits is FFS.

*** End of change ***