3GPP TSG-RAN WG2 Meeting #99			     R2-1708855

Berlin, Germany, August 21 – August 25, 2017

Agenda item:	9.14.2 (LTE_eMTC4-Core)

Source:	LG Electronics Inc.

Title: 	Fast uplink transmission for MTC

Document for:	Discussion and Decision

1.	Introduction

For Rel-15 MTC, one of objectives in work item description is further latency and power consumption reduction. Given that latency reduction, simplified uplink transmission can achieve this goal. In this contribution, we discuss how to reduce latency for uplink transmission and propose a new fast uplink transmission for MTC.

2.	Discussion

A MTC UE in RRC connected state may be put in Long DRX state to save UE battery. In this state, If UL data occurs, the MTC UE has to perform (RA or SR) - BSR - UL TX to transmit UL. On the other hand, the MTC UE in RRC Idle state requires more time before transmitting UL packets. This is because the MTC UE has to first perform RA procedure to make RRC connection, and then establish RB to transmit UL packets. It would take more time than UL latency in RRC Connected.

If the MTC UE wants to transmit a small packet which can be transmitted by only one uplink transmission, one of above explained processes should be performed to complete only one uplink transmission according to UE’s state. This would consume excessive power and take long latency to transmit only one small packet.

In order to minimize UL latency for MTC UEs in both RRC Connected and RRC Idle, it is proposed to introduce a new type of radio channel, called Fast UL Channel (FUCH).

We think the FUCH has following characteristics:

-	FUCH is always available channel in a cell

-	FUCH can be used by UEs in any state

-	FUCH is a contention based channel

-	FUCH is not allowed to retransmit a packet

-	FUCH configuration is broadcast by System Information

-	Multiple FUCHs can be configured in a cell

If such a Fast UL Channel is introduced, the MTC UE can send UL packets anytime it wants regardless of UE’s state. Though the FUCH is subject to contention, it is possible to minimize the contention by proper grouping of MTC UEs that can use a specific FUCH.

Proposal.	In Rel-15 MTC, RAN2 studies always available Fast UL Channel to minimize UL latency and power consumption in any UE state.

3.	Conclusion

In this contribution, we explained the need for Fast UL Channel for MTC in order to minimize UL latency and power consumption regardless of UE state.

Proposal.	In Rel-15 MTC, RAN2 studies always available Fast UL Channel to minimize UL latency and power consumption in any UE state.