3GPP TSG-RAN WG2 Meeting #99	R2-1709293

# Berlin, Germany, 21st – 25th August 2017			resubmission of R2-1706948

Agenda Item:	10.4.1.4.4

Source:	Ericsson

Title:	Measurement events Cx in NR

Document for:	Discussion, Decision

# Introduction

In RAN1#88 NR meeting in Athens, it was agreed [1] that CSI-RS can be used for RRM measurements for mobility in RRC_CONNECTED mode.

For CONNECTED mode RRM measurement for L3 mobility, CSI-RS can be used, in addition to IDLE mode RS

•	Note that RAN1 will consider configuration overhead and possible inter-gNB signaling overhead

•	Detection of neighbor cell for measurement is based on NR-SS

In RAN2#96 the following agreements have been made concerning measurement reports in NR for RRC_CONNECTED UEs:

1	At least events like LTE A1-A6 and periodic will be supported for NR (modification to the events may be considered)

FFS other events may be studied

…

In RAN2#97, further progress was made in the measurement report triggering events related area.

Agreements

1: Measurement events can be configured for xSS and for additional RS for RRM measurement.

2:	At least event A1-A6 can be configured for xSS

FSS Which events that can be configured for additional RS

In RAN2#97bis, it was agreed to have A1-A6 events for CSI-RS as well.

Agreements

1: CSI-RS configured for RRM purpose can be used to derive a cell level quality

2: Events A1-A6 can be configured to use CSI-RS. Events are evaluated on the cell level quality.

In this paper we revisit the discussion on NR measurement report triggering criteria associated to the different mobility events under the agreement that the UE can perform RRM measurements based on CSI-RS.

# Discussion

## Cell based A events

Until now, RAN2 has agreed to adopt events A1-A6 for both SSB and CSI-RS.  As all A events cell quality events, cell quality based on NR-SS and based on CSI-RS needs to be defined. Cell quality is discussed under AI 10.4.1.4.2 and depending on outcome of that discussion, the events A1-A6 may need further discussion on details. Our views for cell quality derivation are given in [1]. For example, as either SSB or CSI-RS can be used to derive the cell quality, it should be clear which cell quality is used in the trigger of A1-A6 event. Options are to define A1-A6 separately for NR-SS and e.g. A1’-A6’ to CSI-RS, or to give the indication of the RS when configuring the event. For Section 5.5.4(36.331 section number) measurement report triggering in 38.331, it is cleaner to have only one set of A event descriptions. As the email discussion [98#33] converged on enabling MO to be linked for several measurement report configurations, it would be enough to indicate in the reportConfig IE which RS is used for the configured A event.

Define only one set of A events for both NR-SS and CSI-RS based cell quality based measurements and configure the RS type in IE reportingConfig.

## Triggering based on TRP quality

In single cell ID scenarios, a cell has remote radio heads aka transmission points, which share the same cell ID and transmit the same NR-SS. Thus a transmission point can be reflected only by a CSI-RS which has different scrambling sequence from the cell ID. One TRP may have several CSI-RSs which represent different beams as shown in Figure 1.

Figure 1: Cell B is single cell ID cell with TRP1 and TRP2 and cell C does not have TRPs.

There is a definite need to be able to identify and report TRP quality in addition to cell quality and the A events are not able to distinguish a TRP.

Define event(s) to identify and trigger measurement reports based on TRP quality.

For this, the UE can be configured to measure the signal quality of these associated CSI-RS, and furthermore UE can be configured with the following related report triggering events when following LTE C1 and C2 events:

- Event C1 (CSI-RS resource becomes better than threshold)

- Event C2 (CSI-RS resource becomes offset better than reference CSI-RS resource)

These events can be configured with individual offsets, hysteresis, and thresholds.

It is natural to consider the existing CSI-RS report events also for NR. By configuring CSI-RS per beam, these events enable absolute and relative per beam report triggering events.

The LTE report triggering events C1 and C2 based on CSI-RS are relevant for CSI-RS’s configured per beam in NR

One identified challenge due to beam based report triggering is the potentially excessive signaling since per-beam report triggering and narrow beams can give numerous beam transitions even over a short time window. The discussed solution is to use set of beams in the event triggering conditions. The natural extension of event C1 is to consider not only one CSI-RS but a set of CSI-RS resources.

Therefore, we introduce an event triggering condition where a set of CSI-RS becomes better than a threshold. Whether this event is a new event, or a generalization of the existing LTE event C1, still needs to be discussed, but it is likely that the overhead to generalize event C1 to consider CSI-RS sets of one or more CSI-RSs is tolerable.

Generalize the C1 event to be based on a configurable set of CSI-RS resources.

In a similar fashion, also the relative comparisons for event triggering can be generalized to group of beams. Again, it seems reasonable to generalize the C2 event to consider CSI-RS groups of one or more CSI-RSs.

Generalize the C2 event to be based on a configurable set of CSI-RS resources.

While the main use case for the generalized C events is TRP quality recognition,  there is no reason to restrict which CSI-RS resources that have beem configured in the MO can or cannot be  listed for the Cx event. The Cx event should give a list of measurement IDs of the CSI-RSs.

The generalized Cx events should give a list of measurement IDs among the CSI-RSs which have been configured for the UE in a MO.

# Conclusion

- In this contribution, the following observations were captured:

Observation 1	The LTE report triggering events C1 and C2 based on CSI-RS are relevant for CSI-RS’s configured per beam in NR

Proposal 1	Define only one set of A events for both NR-SS and CSI-RS based cell quality based measurements and configure the RS type in IE reportingConfig.

Proposal 2	Define event(s) to identify and trigger measurement reports based on TRP quality.

Proposal 3	Generalize the C1 event to be based on a configurable set of CSI-RS resources.

Proposal 4	Generalize the C2 event to be based on a configurable set of CSI-RS resources.

Proposal 5	The generalized Cx events should give a list of measurement IDs among the CSI-RSs which have been configured for the UE in a MO.

# References

R2-1708583, Remaining issues on cell quality derivation, 3GPP TSG-RAN WG2 NR#99, Ericsson, August 2017

R2-1709292, Measurement configuration, 3GPP TSG-RAN WG2 NR#99, Ericsson, August 2017