3GPP TSG-RAN WG2 Meeting#99                                               	R2-1709655

Berlin, Germany, 21th – 25th August 2017

Source: 	KDDI

Title:	Considerations on Establishment cause for NR

Tdoc Type:	Discussion and Decision

Agenda Item:	10.4.1.9

Document for:	Discussion

Release:	Rel-15

Introduction

In this contribution, we discuss the following two aspects for the NR RRC Establishment cause.

- Relationship between “Paging message” and “Establishment cause”

- Clear description about the expected gNB behaviors for establishment cause spare bits

Discussion

Paging message

Regarding LTE specifications for paging message, there is  the information element ”cn-domain(CS or PS)” in TS36.331 which is used for UEs to differentiate the call type of the paging  (voice or data)[1]. If RAN2 standardize that UEs have to prioritize several call types for NR mobile terminating use cases, RAN2 have to define the paging message so that UE can understand which call type is used for the call initiated by paging message. Therefore we make the following proposal 1.

Proposal1: “Paging message” is related to “Establishment cause” tightly (especially for mobile
terminating case), so RAN2 discuss both “Paging message” and “Establishment cause” together.

LTE discussion on spare bits (new RRC Establishment cause)

There were some controversial discussions when introducing new RRC Establishment cause for “mo-VoiceCall” in LTE release14, and RAN2 and RAN Plenary consumed a lot of time to discuss this matter [2][3]. In TS36.331, there were no clear descriptions about the expected eNB behaviors when eNB receive spare bits at that time. And that caused the above mentioned controversial discussions. So, for NR RRC specification, a clear description on expected gNB behaviors for spare bits is necessary for the future forward compatibility.

Proposal2: Expected gNB behaviors for spares for “Establishment cause” should be standardized clearly to avoid the forward compatibility discussion in the future release.

Conclusion and proposals

In this contribution, we make the following two proposals.

Proposal1: “Paging message” is related to “Establishment cause” tightly (especially for mobile terminating cause), so RAN2 discuss both “Paging message” and “Establishment cause” together.

Proposal2: Expected gNB behaviors for spares for “Establishment cause” should be standardized clearly to avoid the forward compatibility discussion in the future release.

Reference

[1]  TS36.331 V14.2.2 (2017-04) V14.2.2 (2017-04) Radio Resource Control (RRC); Protocol specification (Release 14)

[2]  R2-156970 “Way forward with new Establishment Cause for VoLTE calls” Nokia Networks, KDDI, CMCC, ZTE, Samsung, Vodafone, Huawei, Ericsson, Intel, Qualcomm Incorporated, Sony, Fujitsu, Kyocera”, Huawei

[3]  RP-152225 “Way forward on VoLTE call establishment cause”  Nokia Networks