3GPP TSG-RAN WG2 #99	R2-1709872

Berlin, Germany, August 21 – August 25, 2017

Agenda item:		10.3.3.3 (NR_newRAT-Core)

Source:	LG Electronics Inc.

Title: 	Support for out-of-order delivery in PDCP

Document for:	Discussion and Decision

1.	Introduction

RAN2 has discussed the issue of out-of-order delivery in PDCP during RAN2#99.

R2-1707711 TP on out-of-sequence delivery from PDCP OPPO, Nokia, Nokia Shanghai Bell, Sequans, Ericsson discussion Rel-15 NR_newRAT-Core - LG asks why we added a note. Sequans explains it is about COUNT wrap around. LG doesn’t thinks COUNT wrap around occurs. Sequans has seen it already in the draft specs. - Ericsson thinks that that note is there to say that whatever falls out of the window after window moved up, the packets should be considered as received. - Nokia and Oppo explains the behaviour of in-sequence delivery is not changed. - Samsung it is important that we do not change the in-sequence delivery. We changed the state variable name. - LG thinks that the only way to make sure that legacy behaviour is not impacted is to introduce a new variable [CB Thursday] - Determine if there is impact to legacy procedure, whether we should introduce a new variable, and how to capture R2-1709098 Support for out-of-order delivery in PDCP LG Electronics Inc. discussion Rel-15 NR_newRAT-Core R2-1708964 Update of RX_DELIV with disabled reordering Huawei, HiSilicon discussion Rel-15 NR_newRAT-Core R2-1708293 Out-of-order delivery in PDCP receive operation Qualcomm Incorporated discussion Rel-15 NR_newRAT-Core R2-1708749 Alternative PDCP Receive Operation TP with updated RX_DELIV Sequans Communications, OPPO, Nokia, Nokia Shanghai Bell discussion Rel-15 NR_newRAT-Core - Sequans thinks maybe we can think of changing the name. => RX_DELIV definition should be updated such as “new RX_DELIV = old RX_DELIV+1” => Noted R2-1708504 Issues on the PDCP packet reception vivo discussion - QC thinks that the first proposal is useful for EN-DC - Oppo thinks we should also consider: Out-of-order delivery is configured only when the radio bearer is established. LG ask why. OPPO has done the same change as LTE. - Ericsson thinks that there are ROHC that can work with out-of-order delivery. => Noted Agreements 1. ROHC context continue is applied for AM DRB 2. The header compression should not be configured when out-of-order delivery is allowed for PDCP SDU. 3. Out-of-order delivery is configured only when the radio bearer is established (added to the field description of RRC)

Based on the discussion, a text proposal is provided for PDCP receive operation.

2.	Discussion

Option1: TP based on R2-1709098

Basic principle:

-	The in-order delivery PDCP receive operation is not impacted by the out-of-order delivery PDCP receive operation. The out-of-order delivery operation is separated from the in-order delivery operation.

-	Leaving the in-order delivery operation as it is, except for the change of definition of RX_DELIV (i.e. new RX_DELIV = old RX_DELIV + 1).

-	For out-of-order delivery operation, use a new definition for RX_DELIV such that it indicates the COUNT value of the first PDCP SDU not delivered to the upper layers but still waited for.

Option2: TP based on R2-1707711 and Re-1708749

Basic principle:

-	One procedure is applied to both in-order delivery PDCP receive operation and out-of-order delivery PDCP receive operation. Only the different step is separated out for out-of-order delivery operation.

-	IOD operation should be changed so that the same operation is applied to both IOD operation and OOD operation.

-	The definition of RX_DELIV should be changed to be used for both IOD operation and OOD operation.

The example of text proposal to 38.323 is provided below for each option.

Option 1: TP to 38.323v0.2.1 5.2.2 Receive operation In this section, following definitions are used: - HFN(State Variable): the HFN part (i.e. the number of most significant bits equal to HFN length) of the State Variable. - SN(State Variable): the SN part (i.e. the number of least significant bits equal to PDCP SN length) of the State Variable. - RCVD_SN: the PDCP SN of the received PDCP Data PDU, included in the PDU header. - RCVD_HFN: the HFN of the received PDCP Data PDU, calculated by the receiving PDCP entity. - RCVD_COUNT: the COUNT of the received PDCP Data PDU = [RCVD_HFN, RCVD_SN]. 5.2.2.1 Actions when a PDCP Data PDU is received from lower layers At reception of a PDCP Data PDU from lower layers, the receiving PDCP entity shall determine the COUNT value of the received PDCP Data PDU, i.e. RCVD_COUNT, as follows: - if RCVD_SN < SN(RX_DELIV) – Window_Size: - RCVD_HFN = HFN(RX_DELIV) + 1; - else if RCVD_SN >= SN(RX_DELIV) + Window_Size: - RCVD_HFN = HFN(RX_DELIV) – 1; - else: - RCVD_HFN = HFN(RX_DELIV); - RCVD_COUNT = [RCVD_HFN, RCVD_SN]. After determining the COUNT value of the received PDCP Data PDU = RCVD_COUNT, the receiving PDCP entity shall: - if RCVD_COUNT < RX_DELIV; or - if the PDCP Data PDU with COUNT = RCVD_COUNT has been received before: - perform deciphering and integrity verification of the PDCP Data PDU using COUNT = RCVD_COUNT; - if integrity verification fails: - indicate the integrity verification failure to upper layer; - discard the PDCP Data PDU; - else: - perform deciphering and integrity verification of the PDCP Data PDU using COUNT = RCVD_COUNT; - if integrity verification fails: - indicate the integrity verification failure to upper layer; - discard the PDCP Data PDU; If the received PDCP Data PDU with COUNT value = RCVD_COUNT is not discarded above, the receiving PDCP entity shall: - if outOfOrderDelivery is not configured: - store the resulting PDCP SDU in the reception buffer; - if RCVD_COUNT >= RX_NEXT: - update RX_NEXT to RCVD_COUNT + 1; - if RCVD_COUNT = RX_DELIV: - deliver to upper layers in ascending order of the associated COUNT value after performing header decompression; - all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from COUNT = RX_DELIV; - update RX_DELIV to the COUNT value of the last PDCP SDU delivered to upper layers + 1; - if t-Reordering is running, and if the PDCP SDU with COUNT = RX_REORD – 1 has been delivered to upper layers: - stop and reset t-Reordering; - if t-Reordering is not running (includes the case when t-Reordering is stopped due to actions above), and if there is at least one stored PDCP SDU: - update RX_REORD to RX_NEXT; - start t-Reordering; - else (i.e. outOfOrderDelivery is configured): - deliver the resulting PDCP SDU to upper layers; - if RCVD_COUNT >= RX_NEXT: - update RX_NEXT to RCVD_COUNT + 1; - if RCVD_COUNT = RX_DELIV: - update RX_DELIV to the COUNT value of the first PDCP SDU not delivered to upper layers but still waited for, among PDCP SDUs with COUNT value > RX_DELIV; - if t-Reordering is running, and if all PDCP SDUs with COUNT values with RX_DELIV <= COUNT < RX_REORD have been delivered to upper layers: - stop and reset t-Reordering; - if t-Reordering is not running (includes the case when t-Reordering is stopped due to actions above), and if RX_DELIV < RX_NEXT: - update RX_REORD to RX_NEXT; - start t-Reordering. 5.2.2.2 Actions when a t-Reordering expires When t-Reordering expires, the receiving PDCP entity shall: - if outOfOrderDelivery is not configured: - deliver to upper layers in ascending order of the associated COUNT value after performing header decompression: - all stored PDCP SDU(s) with associated COUNT value(s) < RX_REORD; - all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from RX_REORD; - update RX_DELIV to the COUNT value of the last PDCP SDU delivered to upper layers + 1; - if there is at least one stored PDCP SDU: - update RX_REORD to RX_NEXT; - start t-Reordering; - else (i.e. outOfOrderDelivery is configured): - update RX_DELIV to the COUNT value of the first PDCP SDU not delivered to upper layers but still waited for, among PDCP SDUs with COUNT value >= RX_REORD; - if RX_DELIV < RX_NEXT: - update RX_REORD to RX_NEXT; - start t-Reordering. 5.2.2.3 Actions when the value of t-Reordering is reconfigured When the value of the t-Reordering is reconfigured by upper layers while the t-Reordering is running, the UE shall: - update RX_REORD to RX_NEXT; - stop and restart t-Reordering. 7.1 State variables This sub clause describes the state variables used in PDCP entities in order to specify the PDCP protocol. The state variables defined in this subclause are normative. All state variables are non-negative integers, and take values from 0 to [232 – 1]. PDCP Data PDUs are numbered integer sequence numbers (SN) cycling through the field: 0 to [2[pdcp-SN-Size] – 1]. The transmitting PDCP entity shall maintain the following state variables: a) TX_NEXT This state variable indicates the COUNT value of the next PDCP SDU to be transmitted. The initial value is 0. The receiving PDCP entity shall maintain the following state variables: a) RX_NEXT This state variable indicates the COUNT value of the next PDCP SDU expected to be received. The initial value is 0. b) RX_DELIV If outOfOrderDelivery is not configured, this state variable indicates the COUNT value following the COUNT value associated with the last PDCP SDU delivered to the upper layers. If outOfOrderDelivery is configured, this state variable indicates the COUNT value following the COUNT value associated with the first PDCP SDU not delivered to the upper layers but still waited for. The initial value is 0. c) RX_REORD This state variable indicates the COUNT value following the COUNT value associated with the PDCP Data PDU which triggered t-Reordering. 7.2 Constants a) Window_Size This constant indicates the size of the reordering window. The value equals to 2[pdcp-SN-Size] – 1. 7.3 Timers The transmitting PDCP entity shall maintain the following timers: a) discardTimer This timer is configured only for DRBs. The duration of the timer is configured by upper layers [3]. In the transmitter, a new timer is started upon reception of an SDU from upper layer. The receiving PDCP entity shall maintain the following timers: b) t-Reordering The duration of the timer is configured by upper layers [3]. This timer is used to detect loss of PDCP Data PDUs as specified in the subclause 5.2.2. If t-Reordering is running, t-Reordering shall not be started additionally, i.e. only one t-Reordering per receiving PDCP entity is running at a given time.

Option 2: TP to 38.323v0.2.1 5.2.2 Receive operation In this section, following definitions are used: - HFN(State Variable): the HFN part (i.e. the number of most significant bits equal to HFN length) of the State Variable. - SN(State Variable): the SN part (i.e. the number of least significant bits equal to PDCP SN length) of the State Variable. - RCVD_SN: the PDCP SN of the received PDCP Data PDU, included in the PDU header. - RCVD_HFN: the HFN of the received PDCP Data PDU, calculated by the receiving PDCP entity. - RCVD_COUNT: the COUNT of the received PDCP Data PDU = [RCVD_HFN, RCVD_SN]. 5.2.2.1 Actions when a PDCP Data PDU is received from lower layers At reception of a PDCP Data PDU from lower layers, the receiving PDCP entity shall determine the COUNT value of the received PDCP Data PDU, i.e. RCVD_COUNT, as follows: - if RCVD_SN < SN(RX_DELIV) – Window_Size: - RCVD_HFN = HFN(RX_DELIV) + 1; - else if RCVD_SN >= SN(RX_DELIV) + Window_Size: - RCVD_HFN = HFN(RX_DELIV) – 1; - else: - RCVD_HFN = HFN(RX_DELIV); - RCVD_COUNT = [RCVD_HFN, RCVD_SN]. After determining the COUNT value of the received PDCP Data PDU = RCVD_COUNT, the receiving PDCP entity shall: - if RCVD_COUNT < RX_DELIV; or - if the PDCP Data PDU with COUNT = RCVD_COUNT has been received before: - perform deciphering and integrity verification of the PDCP Data PDU using COUNT = RCVD_COUNT; - if integrity verification fails: - indicate the integrity verification failure to upper layer; - discard the PDCP Data PDU; - else: - perform deciphering and integrity verification of the PDCP Data PDU using COUNT = RCVD_COUNT; - if integrity verification fails: - indicate the integrity verification failure to upper layer; - discard the PDCP Data PDU; If the received PDCP Data PDU with COUNT value = RCVD_COUNT is not discarded above, the receiving PDCP entity shall: - store the resulting PDCP SDU in the reception buffer; - if RCVD_COUNT >= RX_NEXT: - update RX_NEXT to RCVD_COUNT + 1; - if outOfOrderDelivery is configured and the resulting PDCP SDU has not been delivered to upper layers: - deliver the resulting PDCP SDU to upper layers; - if RCVD_COUNT = RX_DELIV: - deliver to upper layers in ascending order of the associated COUNT value after performing header decompression; - all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from COUNT = RX_DELIV; - update RX_DELIV to the COUNT value of the first PDCP SDU which has not beend delivered to upper layers, with COUNT value >= RX_DELIV; - if t-Reordering is running, and if RX_DELIV >= RX_REORD: - stop and reset t-Reordering; - if t-Reordering is not running (includes the case when t-Reordering is stopped due to actions above), and RX_DELIV < RX_NEXT: - update RX_REORD to RX_NEXT; - start t-Reordering. 5.2.2.2 Actions when a t-Reordering expires When t-Reordering expires, the receiving PDCP entity shall: - deliver to upper layers in ascending order of the associated COUNT value after performing header decompression: - all stored PDCP SDU(s) with associated COUNT value(s) < RX_REORD; - all stored PDCP SDU(s) with consecutively associated COUNT value(s) starting from RX_REORD; - update RX_DELIV to the COUNT value of the first PDCP SDU which has not been delivered to upper layers, with COUNT value >= RX_REORD; - if RX_DELIV < RX_NEXT: - update RX_REORD to RX_NEXT; - start t-Reordering. 5.2.2.3 Actions when the value of t-Reordering is reconfigured When the value of the t-Reordering is reconfigured by upper layers while the t-Reordering is running, the UE shall: - update RX_REORD to RX_NEXT; - stop and restart t-Reordering. 7.1 State variables This sub clause describes the state variables used in PDCP entities in order to specify the PDCP protocol. The state variables defined in this subclause are normative. All state variables are non-negative integers, and take values from 0 to [232 – 1]. PDCP Data PDUs are numbered integer sequence numbers (SN) cycling through the field: 0 to [2[pdcp-SN-Size] – 1]. The transmitting PDCP entity shall maintain the following state variables: a) TX_NEXT This state variable indicates the COUNT value of the next PDCP SDU to be transmitted. The initial value is 0. The receiving PDCP entity shall maintain the following state variables: a) RX_NEXT This state variable indicates the COUNT value of the next PDCP SDU expected to be received. The initial value is 0. b) RX_DELIV This state variable indicates the COUNT value of the first PDCP SDU not delivered to the upper layers, but still waited for. The initial value is 0. c) RX_REORD This state variable indicates the COUNT value following the COUNT value associated with the PDCP Data PDU which triggered t-Reordering. 7.2 Constants a) Window_Size This constant indicates the size of the reordering window. The value equals to 2[pdcp-SN-Size] – 1. 7.3 Timers The transmitting PDCP entity shall maintain the following timers: a) discardTimer This timer is configured only for DRBs. The duration of the timer is configured by upper layers [3]. In the transmitter, a new timer is started upon reception of an SDU from upper layer. The receiving PDCP entity shall maintain the following timers: b) t-Reordering The duration of the timer is configured by upper layers [3]. This timer is used to detect loss of PDCP Data PDUs as specified in the subclause 5.2.2. If t-Reordering is running, t-Reordering shall not be started additionally, i.e. only one t-Reordering per receiving PDCP entity is running at a given time.

3.	Proposal

It is proposed to discuss two options above, and agree on one of the options. The detailed text proposal can be discussed later once RAN2 agree on one of the options.