3GPP TSG-RAN WG2 Meeting #99bis								R2-1710402

Prague, Czech Republic, 9 - 13 October 2017

Agenda item:	9.2

Source: 	Huawei, HiSilicon

Title: 	HARQ Process ID Calculation to support SPS for sTTI

Document for:	Discussion and decision

1	Introduction

In RAN2 #99 meeting, the agreements on SPS for sTTI were achieved [1].

Agreements

=>	RAN2 sees benefits to support SPS for short TTI

=>	Same LCP restriction as dynamic grant will apply for SPS grant.

=>	sTTI SPS intervals are configured by RRC. New sTTI-based interval values are introduced. The minimum value of 1 sTTI is supported.  Other values are FFS.

=>	In DL/UL SPS with asynchronous HARQ, the HARQ process ID is derived from the sTTI-number (index of sTTI within subframe).  Details are FFS

=>	Send LS to RAN1 to indicate RAN2 desire to support SPS and agreements.  Ask the feasibility of SPS activation/deactivation for sTTI

There is some FFS on how to calculate the HARQ Process ID for sSPS (SPS in sTTI). In this document, the HARQ Process ID calculation in current specification as well as for sTTI is analysed and proposed.

2	Discussion

2.1	HARQ Process ID Calculation for Legacy TTI

The HARQ Process ID is used to identify the HARQ process number and combine the new transmission and retransmission to achieve the combining gain. In SPS, the new transmission is configured periodically on PDSCH/PUSCH and unlike retransmission, and there is no PDCCH order to indicate the HARQ process ID. The HARQ process ID is calculated based on the formulas in the following [2]. It can be seen that the HARQ process ID for SPS is calculated based on the SPS interval, the start subframe and number of the SPS HARQ process.

Downlink

For configured downlink assignments, the HARQ Process ID associated with this TTI is derived from the following equation:

HARQ Process ID = [floor(CURRENT_TTI/semiPersistSchedIntervalDL)] modulo numberOfConfSPS-Processes,

where CURRENT_TTI=[(SFN * 10) + subframe number].

Uplink

For configured uplink grants, the HARQ Process ID associated with this TTI is derived from the following equation for asynchronous UL HARQ operation:

HARQ Process ID = [floor(CURRENT_TTI/semiPersistSchedIntervalUL)] modulo numberOfConfUlSPS-Processes,

where CURRENT_TTI=[(SFN * 10) + subframe number] and it refers to the subframe where the first transmission of a bundle takes place.

2.2	HARQ Process ID Calculation in sTTI

In accordance with the agreements in RAN2 #99 meeting, the HARQ process ID is derived from the sTTI-number. Since the HARQ Process ID in legacy is calculated based on the parameter CURRENT_TTI, CURRENT_TTI is sTTI can be derived using sTTI-number, i.e. the CURRENT_TTI is composed of SFN number, subframe number and sTTI number is sTTI. In order to determine the sTTI index used for current SPS transmission, the CURRENT_TTI in sTTI is mapped into sTTI level which is expressed as below:

CURRENT_TTI = [(SFN * 10 * sTTI_Number_Per_Subframe) + subframe number * sTTI_Number_Per_Subframe + sTTI number].

Here sTTI_Number_Per_Subframe is the sTTI number in one subframe. For example, if the sTTI length is configured as 7OS length, sTTI_Number_Per_Subframe is 2, and if sTTI length is configured as 2OS, sTTI_Number_Per_Subframe is 6. And sTTI number is the index of sTTI in one subframe.

Besides, sTTI level for SPS interval was also agreed in last meeting which means that the SPS resource is allocated using sTTI interval value. In order to avoid HARQ ID confusion in one SPS interval, the interval of sTTI level is used for HARQ process ID calculation.

The HARQ process ID is define below, taking  CURRENT_TTI as well as sSPS interval into consideration:

Downlink

HARQ Process ID = [floor(CURRENT_TTI/semiPersistSchedIntervalDL- sTTI)] modulo numberOfConfSPS-Processes- sTTI,

where CURRENT_TTI = [(SFN * 10 * sTTI_Number_Per_Subframe) + subframe number * sTTI_Number_Per_Subframe + sTTI number].

Uplink

HARQ Process ID = [floor(CURRENT_TTI/semiPersistSchedIntervalUL-sTTI)] modulo numberOfConfUlSPS-Processes-sTTI,

Where CURRENT_TTI = [(SFN * 10 * sTTI_Number_Per_Subframe) + subframe number * sTTI_Number_Per_Subframe + sTTI number] and it refers to the sTTI occasion where the first transmission of a bundle takes place.

Proposal 1: to use the formula for HARQ process ID calculation for downlink in SPS

HARQ Process ID = [floor(CURRENT_TTI/semiPersistSchedIntervalDL- sTTI)] modulo numberOfConfSPS-Processes- sTTI,

where CURRENT_TTI = [(SFN * 10 * sTTI_Number_Per_Subframe) + subframe number * sTTI_Number_Per_Subframe + sTTI number].

Proposal 2: to use the formula for HARQ process ID calculation for uplink in SPS

HARQ Process ID = [floor(CURRENT_TTI/semiPersistSchedIntervalUL-sTTI)] modulo numberOfConfUlSPS-Processes-sTTI,

Where CURRENT_TTI = [(SFN * 10 * sTTI_Number_Per_Subframe) + subframe number * sTTI_Number_Per_Subframe + sTTI number] and it refers to the sTTI occasion where the first transmission of a bundle takes place.

3	Conclusions

In this document, the HARQ process ID for SPS in sTTI is analysed. And, we kindly propose that:

Proposal 1: to use the formula for HARQ process ID calculation for downlink in SPS

HARQ Process ID = [floor(CURRENT_TTI/semiPersistSchedIntervalDL- sTTI)] modulo numberOfConfSPS-Processes- sTTI,

where CURRENT_TTI = [(SFN * 10 * sTTI_Number_Per_Subframe) + subframe number * sTTI_Number_Per_Subframe + sTTI number].

Proposal 2: to use the formula for HARQ process ID calculation for uplink in SPS

HARQ Process ID = [floor(CURRENT_TTI/semiPersistSchedIntervalUL-sTTI)] modulo numberOfConfUlSPS-Processes-sTTI,

Where CURRENT_TTI = [(SFN * 10 * sTTI_Number_Per_Subframe) + subframe number * sTTI_Number_Per_Subframe + sTTI number] and it refers to the sTTI occasion where the first transmission of a bundle takes place.

4	References

- RAN2 #99 meeting chairman notes, R2-170xxxx;

- 3GPP TS 36.321 v14.3.0, Medium Access Control (MAC) protocol, Release 14;