3GPP TSG-RAN WG2#99bis	Tdoc R2-1710473

Prague, Czech Republic, Oct 9th – 13th 2017	(Resubmission of R2-1708172)

Agenda Item:	10.4.1.7.6

Source:	Ericsson

Title:	RAN Sharing and identifier aspects in NR

Document for:	Discussion, Decision

# Introduction

RAN Sharing (e.g, MOCN, Multi-Operator Core Network) has been supported for LTE since Release 8. With NR, shared deployments will also be relevant and thus, RAN sharing solutions are needed also for NR.

This contribution address in particular RAN Sharing in NR and resume ID.

# Discussion

## Cell identity and Tracking Area Index

One challenge with, e.g., MOCN is to limit the amount of necessary coordination between different operators and/or different supported PLMNs in a cell. For LTE, this problem was addressed in the last RAN Plenary (#76) and CRs were approved [1] to allow the possibility to associate a cell identity and a tracking area code per PLMN instead of using the same cell identity and tracking area code for all supported PLMNs. We think this is attractive also for NR and in another contribution, we propose to allow TAC and Cell ID per PLMN also for NR, see [2].

## RAN Areas

It can be assumed that in a Shared RAN, the RAN areas will be separate per PLMN (Irrespective of how they are defined, e.g., with cell lists or other means).

In a shared RAN deployment, RAN Areas will be specific per PLMN.

## Resume ID

For a shared RAN or a Neutral Host Operator, another identifier to handle is the resume ID that is used both for RAN paging of the UE, RAN Area updates and when a UE trigger a transition from RRC_INACTIVE to RRC_CONNECTED, for purposes of transmitting UL data.

In LTE the resume ID is constructed including a combination of the eNB ID (20 bits) and a eNB specific UE identity (20 bits). It is currently not decided exactly how the Resume ID will be constructed in NR, but from a UE perspective though, it is proposed that the resume ID should only be a random number, irrespective of how it is constructed see, e.g., [3].

When the Resume ID is received, e.g., in RRCConnectionResumeRequest and if the context is not stored in the receiving gNB, the context needs to be fetched from another node. If the receiving gNB is a gNB serving multiple PLMNs, there is currently no obvious way of knowing what other gNBs that may hold the context of the UE and with many PLMNs, it could quickly become many other gNBs to inquire about context information from, in several PLMNs.

Receiving a resume ID in a gNB serving several PLMNs, if context is not stored in the same gNB, it may be necessary to attempt to fetch the context from many different gNBs, belonging to different PLMNs.

One way of addressing this is to divide or coordinate the usage of the resume IDs, such that a gNB receiving a resume ID will immediately be able to connect it to a specific PLMN and then implicitly be able to limit the amount of other nodes to inquire about context ID. We believe though that it would be attractive if this coordination could be avoided as it will affect the amount of resume IDs in all PLMNs that share a RAN node.

It would be attractive if coordination of resume IDs between PLMNs can be avoided.

To further address the minimization of coordination between different PLMNs, we propose to make the Resume IDs valid only per Registered PLMN and this Registered PLMN would then need to be included in UE-initiated signaling, e.g., RRCConnectionResumeRequest / msg3 and stored with/in the UE context.

Including the PLMN identity would remove any need for coordination over PLMN borders, it would be more feasible to handle resume requests in gNBs serving several PLMNs and each PLMN would have the complete set of Resume IDs available to use.

Include a Registered PLMN indication in RRCConnectionResumeRequest to support UE Context ID fetch in shared RAN scenarios

Include the Registered PLMN in the UE Context

With a similar reasoning, the same must go for RAN Area Paging and the Registered PLMN must be included when performing (RAN Area) Paging.

Include a Registered PLMN indication in RAN Area Paging

There are different ways to include a PLMN ID. Including the complete PLMN ID will be bit costly and even though it is possible, it may be preferable to indicate the PLMN with an index, for example given from system information where an index for each supported PLMN may be set.

Represent Registered PLMN together with Resume ID with a PLMN Index retrieved from System information and valid in the particular cell

# Conclusion

In section 2 we made the following observations:

Observation 1	In a shared RAN deployment, RAN Areas will be specific per PLMN.

Observation 2	Receiving a resume ID in a gNB serving several PLMNs, if context is not stored in the same gNB, it may be necessary to attempt to fetch the context from many different gNBs, belonging to different PLMNs.

Observation 3	It would be attractive if coordination of resume IDs between PLMNs can be avoided.

Based on the discussion in section 2 we propose the following:

Proposal 1	Include a Registered PLMN indication in RRCConnectionResumeRequest to support UE Context ID fetch in shared RAN scenarios

Proposal 2	Include the Registered PLMN in the UE Context

Proposal 3	Include a Registered PLMN indication in RAN Area Paging

Proposal 4	Represent Registered PLMN together with Resume ID with a PLMN Index retrieved from System information and valid in the particular cell

# References

RP-171246, RAN2 CRs EUTRAN Sharing enhancement, RAN2, 3GPP RAN Plenary #76, West Palm Beach, USA, 5th-8th June, 2017

R2-1710391, System information content at network sharing, Ericsson, 3GPP RAN WG2#99bis, Prague, Czech Republic, Oct 9th – 13th, 2017

R2-1710831, UE Context ID discussion, Ericsson, 3GPP RAN WG2 #99bis, Prague, Czech Republic, Oct 9th – 13th, 2017