3GPP TSG-RAN WG2 Meeting #99bis		R2-1710617

Prague, Czech Republic, 9-13 October, 2017		(Revision of R2-1708792)

Agenda Item:	10.4.1.3.1

Source: 		Intel Corporation

Title: 		UE handling of combined configuration messages

Document for:			Discussion and Decision

# Introduction

In LTE, in order to prevent UE from exceeding its capability for band combination, measurement and DRB support during message processing, it adopts a procedure text approach where UE will perform a release action first before an addition action.

However, for EN-DC, SN NR message may be embedded in the MN LTE message. In such case, using procedure text of ensuring release first before addition is not sufficient, as the release and addition have to be coordinated across the processing of the separate messages. Otherwise, the UE may trigger configuration error due to UE capability exceeded during the processing of the messages, if processing the messages in isolation.

This contribution discusses the issue in detail and makes proposals on how to handle this.

# Discussion

## Background

LTE adopts a procedure text approach where UE will perform according to procedure text such that a release action is performed first before an addition action within a single RRC message. This sequence ensures that UE capability is not exceeded even during the processing of the message.

Examples of such text are as follow:

- In SCell management in CA in MCG during RRC Connection reconfiguration (Section 5.3.5.3):

…..

1>	if the received RRCConnectionReconfiguration includes the sCellToReleaseList:

2>	perform SCell release as specified in 5.3.10.3a;

1>	if the received RRCConnectionReconfiguration includes the sCellToAddModList:

2>	perform SCell addition or modification as specified in 5.3.10.3b;

…..

- In SCell management in CA in SCG during RRC Connection reconfiguration (Section 5.3.10.10):

…..

2>	if the received scg-ConfigPartSCG includes the sCellToReleaseListSCG:

3>	perform SCell release for the SCG as specified in 5.3.10.3a;

2>	if the received scg-ConfigPartSCG includes the pSCellToAddMod:

3>	perform PSCell addition or modification as specified in 5.3.10.3c;

NOTE 0:	This procedure is also used to release the PSCell e.g. PSCell change, SI change for the PSCell.

2>	if the received scg-ConfigPartSCG includes the sCellToAddModListSCG:

3>	perform SCell addition or modification as specified in 5.3.10.3b;

…..

The above approach is not sufficient to prevent (re)configuration error due to capability exceeded in the case of EN-DC operation as 2 separate RRC messages (LTE RRC message and NR RRC message) are to be processed and message handling of the 2 RRC messages are in separate specification.  We use a couple of examples to illustrate the problem for EN-DC.

Assuming that a UE may support the following band combinations for EN-DC:

The eNB may send in one single LTE message (with embedded NR message) to change the band combination for EN-DC from Band Combination A to Band Combination B. If the UE processes the LTE message and NR message independently without coordination, it may result in (re)configuration error due to capability exceeded when NR message is processed first (resulting in UE band combination of X, Y, Z, W) before the LTE message is processed. Furthermore, if the UE applied one (LTE or NR) configuration first before the other, the UE behaviour during the processing time of the combined message could be temporarily different from the intended behaviour with the configuration provided until both messages are processed and executed.

Since it is also agreed to allow MN and SN to independently configure the measurement configuration, the above problem can also apply to measurement capability (e.g. max frequency layers the UE can monitor etc.).

Also there are some restrictions in terms of the configuring simultaneously different bearer types e.g. simultaneous MCG split and SCG DRB etc.

Observation#1: If the UE processes the LTE message and NR message independently without coordination, it may result in perceived (re)configuration error due to capability exceeded when for example, NR message is processed first before the LTE message is processed.

Observation #2: If the UE applied one (LTE or NR) configuration first before other, the UE behaviour during the processing time of the combined message could be temporarily different from the intended behaviour with the configuration provided until both messages are processed and executed.

Other than UE capability, the DRB ID in the messages may also require the coordination between the LTE and NR messages.  In RAN2#98, it is agreed that ‘For MR-DC the DRB ID is uniquely assigned for one UE (independent of whether it is MCG or SCG DRB)’. If LTE has DRB-ID (1,2) for MCG DRBs and NR has the DRB IDs (3 and 4) for SCG DRBs and then a reconfiguration message comes which deletes the MCG DRB in LTE (say voice service with DRB-ID 1) and adds NR with a SCG DRB (which is assigned for a data service as DRB-ID 1). If release is not done before addition then during a transient period it may result that both LTE and NR shall have DRB-ID 1 which is an invalid configuration.

Observation#3: If the UE processes the LTE message and NR message independently without coordination, it may result in perceived (re)configuration error due non-unique DRB ID).

## Solutions

One solution is to avoid such band combination switching occurring by the network. That is, the network ensures that such band combination change does not occur in 1 RRC message. However this may not be acceptable from system performance point of view.

The alternative is to leave it to UE to avoid triggering (re)configuration error.  That is, UE will only generate reconfiguration failure due to capability exceeded if the resultant max capability, after applying the radio bearer configuration, measurement configuration and MCG configuration in the LTE message and the radio bearer configuration, measurement configuration and SCG configuration in the NR message, is exceeded. It should not base it on the application of either the LTE message or the NR message alone.

Proposal#1: UE shall generate reconfiguration failure only if the combined configuration is invalid (e.g., max capability exceeded in terms of band combination support, measurement capability and DRB support), after processing fully both LTE and NR messages.

Proposal #2: It is not considered an error if the UE behaviour during the message processing results in some intermediate configurations different from the combined configuration.

## How to specify it in the LTE and NR specification

In LTE 36.331 specification, if the UE cannot comply with the configuration in the RRC Connection Reconfiguration message, it will apply the Reconfiguration Failure procedure as specified in Section 5.3.5.5. The following is an extraction of the section:

#### 5.3.5.5	Reconfiguration failure

The UE shall:

1>	if the UE is unable to comply with (part of) the configuration included in the RRCConnectionReconfiguration message:

2>	continue using the configuration used prior to the reception of RRCConnectionReconfiguration message;

2>	if security has not been activated:

3>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause other;

2>	else:

3>	initiate the connection re-establishment procedure as specified in 5.3.7, upon which the connection reconfiguration procedure ends;

NOTE 1:	The UE may apply above failure handling also in case the RRCConnectionReconfiguration message causes a protocol error for which the generic error handling as defined in 5.7 specifies that the UE shall ignore the message.

NOTE 2:	If the UE is unable to comply with part of the configuration, it does not apply any part of the configuration, i.e. there is no partial success/ failure.

A new note can be added to the section. An example of such note:

NOTE 3:	For MR-DC UE, the UE applies the above failure handling only if it is unable to comply with the combined configuration in the LTE RRC message and the embedded NR RRC message.

Alternatively, such statement can also be added to the reconfiguration failure section in the NR RRC specification.

Proposal#3: Include the following note in the Reconfiguration failure section in both LTE and NR RRC specifications:

NOTE 3:	For MR-DC UE, the UE applies the above failure handling only if it is unable to comply with the combined configuration in the LTE RRC message and the embedded NR RRC message.

Due to the need to coordinate the processing of the LTE and NR RRC message (and execution of the messages) in the case of LTE message embedded with NR message, the processing time for the LTE message (e.g. the RRCConnectionReconfiguration message) with embedded NR RRC message may increase relative to the process of a LTE RRC message without embedded NR RRC message. The LTE RRC message embedded with NR RRC message may or may not have any MCG configuration included. The delta, say Xms, would need to be added to each of the reconfigurations shown below where the value X is FFS, as shown in Proposal#4.

It is proposed to add EN-DC specific processing time for RRC Connection reconfiguration with embedded NR message as follow:

Proposal#4: EN-DC/NGEN-DC specific processing time for RRC Connection Reconfiguration with embedded NR message should be increased (FFS on the value of time increase X):

| RRC connection re-configuration (radio resource configuration) with NR message | RRCConnectionReconfiguration | RRCConnectionReconfigurationComplete | 15 + X |  |
| --- | --- | --- | --- | --- |
| RRC connection re-configuration (measurement configuration) with NR message | RRCConnectionReconfiguration | RRCConnectionReconfigurationComplete | 15 + X |  |
| RRC connection re-configuration (intra-LTE mobility) with NR message | RRCConnectionReconfiguration | RRCConnectionReconfigurationComplete | 15 + X |  |
| RRC connection reconfiguration (SCell addition/release) with NR message | RRCConnectionReconfiguration | RRCConnectionReconfigurationComplete | 20 + X |  |
| RRC connection reconfiguration (SCG establishment/ release, SCG cell addition/ release) with NR message | RRCConnectionReconfiguration | RRCConnectionReconfigurationComplete | 20 + X |  |

The UE may receive NR message directly from the SN via SRB3 (SCG SRB) as well as indirectly via LTE message via SRB1. It is possible that UE is processing a NR RRCReconfiguration message from SCG SRB3 while another NR RRCReconfiguration message is received from the LTE side via SRB1. As RAN 2 has agreed that “UE processes [NR] messages received on SCG SRB [or over MCG SRB] one message at a time in the order received at the RRC. (i.e. same rules as in LTE)”, it should be clear that the processing delay requirement only consider the processing delay when the UE is only receiving 1 message at a time.  Hence it is proposed to make this clear in the processing delay requirement section:

With EN-DC, it is also possible for the UE to receive a LTE message (with no NR message) in SRB1/2 from the MN while processing NR message in SRB3 from the SN and vice versa. The processing delay requirement may need to be studied further for this case.

Proposal#5: For EN-DC, the processing delay requirement in Proposal 4 is on the assumption that the UE only receives 1 NR RRC message at a time.

FFS on the processing delay requirement where UE receives a LTE message (containing no NR message) and NR message from SRB3 directly.

# Conclusion and proposals

It is requested that RAN 2 discuss and agree on the following proposal:

Observation#1: If the UE processes the LTE message and NR message independently without coordination, it may result in perceived (re)configuration error due to capability exceeded when for example, NR message is processed first before the LTE message is processed.

Observation #2: If the UE applied one (LTE or NR) configuration first before other, the UE behaviour during the processing time of the combined message could be temporarily different from the intended behaviour with the configuration provided until both messages are processed and executed.

Observation#3: If the UE processes the LTE message and NR message independently without coordination, it may result in perceived (re)configuration error due non-unique DRB ID).

Proposal#1: UE shall generate reconfiguration failure only if the combined configuration is invalid (e.g., max capability exceeded in terms of band combination support, measurement capability and DRB support), after processing fully both LTE and NR messages.

Proposal #2: It is not considered an error if the UE behaviour during the message processing results in some intermediate configurations different from the combined configuration.

Proposal#3: Include the following note in the Reconfiguration failure section in both LTE and NR RRC specifications:

NOTE 3:	For MR-DC UE, the UE applies the above failure handling only if it is unable to comply with the combined configuration in the LTE RRC message and the embedded NR RRC message.

Proposal#4: EN-DC/NGEN-DC specific processing time for RRC Connection Reconfiguration with embedded NR message should be increased (FFS on the value of time increase X):

| RRC connection re-configuration (radio resource configuration) with NR message | RRCConnectionReconfiguration | RRCConnectionReconfigurationComplete | 15 + X |  |
| --- | --- | --- | --- | --- |
| RRC connection re-configuration (measurement configuration) with NR message | RRCConnectionReconfiguration | RRCConnectionReconfigurationComplete | 15 + X |  |
| RRC connection re-configuration (intra-LTE mobility) with NR message | RRCConnectionReconfiguration | RRCConnectionReconfigurationComplete | 15 + X |  |
| RRC connection reconfiguration (SCell addition/release) with NR message | RRCConnectionReconfiguration | RRCConnectionReconfigurationComplete | 20 + X |  |
| RRC connection reconfiguration (SCG establishment/ release, SCG cell addition/ release) with NR message | RRCConnectionReconfiguration | RRCConnectionReconfigurationComplete | 20 + X |  |

Proposal#5: For EN-DC, the processing delay requirement in Proposal 4 is on the assumption that the UE only receives 1 NR RRC message at a time.

FFS on the processing delay requirement where UE receives a LTE message (containing no NR message) and NR message from SRB3 directly.