3GPP TSG-RAN WG2 #99bis	R2-1710853

Prague, Czech Republic, 9 – 13 October 2017

Agenda Item:	10.4.1.9

Source:	Ericsson

Title:	Text proposal for mobility related inter-node messages

Document for:	Discussion, Decision

# Introduction

This contribution contains a text proposal to 38.331 for mobility related inter-node messages.

The text is based on LTE RRC specs. In order to highlight the changes compared to LTE change marks towards 36.331 are used, except for the ASN.1, which is based on the structure agreed in NR. In particular, we are trying to capture the agreements reached in the email discussion [NR-AH2#11]:

Agreements relating to information carried between source and target node:

1: NR RRC specifications define a transparent RRC container (like the HandoverPreparationInformation message in LTE) to be transmitted from the source gNB to the target gNB as part of the Xn Handover Request message.

2.1	As in LTE, the HandoverPreparationInformation to be transmitted from the source gNB to the target gNB includes the UE capabilities for different RATs

2.2	As in LTE, the HandoverPreparationInformation to be transmitted from the source gNB to the target gNB can include the AS configuration, the RRM configuration and the AS context (including information necessary to handle handover failures). The details of the content of each IE are FFS.

3.1: The AS configuration includes the measurement configuration and radio resource configurations, UE identifier in the source, at least Antenna Info and DL Carrier Frequency.

FFS Whether the AS configuration includes the QoS flow to DRB mapping that was not configured to the UE via RRC signalling.

FFS Need to provide the system information from source equivalent to LTE’s MIB, SIB-1 and SIB-2 (some checking of use of this in LTE to be done)

4.1: The RRM configuration can include at least the inactive time.

4.2: As in LTE, to support CA case, the RRM configuration can include the list of best cells on each frequency for which measurement information is available.

5	Available beam measurement information can be part of the RRM configuration of the HandoverPreparationInformation message if beam measurement information (i.e. beam indexes and optionally measurement results) have been configured by the source gNodeB to be reported by a UE. That information is not a mandatory part of the HandoverPreparationInformation message

FFS For which cell(s) beam measurement information can be included e.g. only candidate target cell.

6	The RRM configuration can include both beam measurement information (for layer 3 mobility) associated to SS Block(s) and CSI-RS(s) for the reported cell (or cells depending on outcome of FFS above) if both types of measurements are available.

# Discussion

## Criticality information

One of the aspects differing between NW to UE and NW node to NW node configuration is the error handling. Typically RAN2 specifications have limited opportunities for recovering the communication if the UE does not understand the configuration. This is understandable, as many of the RAN2 configuration messages are actually configuring the properties needed to establish and maintain a succesfull communication link between the UE and the NW.

For NW node to NW node communication, RAN3 has adopted a more flexible mechanism, which identifies whether certain part of configuration was critical or not. This is feasible as the network nodes can often maintan communication even if they do not fully understand all information exchanged. However, most of the RRC containers have criticality “reject” (see e.g. UE Context Information contained below).

| UE Context Information |  | 1 |  |  | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |
| >MME UE S1AP ID | M |  | INTEGER (0..232 -1) | MME UE S1AP ID allocated at the MME | – | – |
| >UE Security Capabilities | M |  | 9.2.29 |  | – | – |
| >AS Security Information | M |  | 9.2.30 |  | – | – |
| >UE Aggregate Maximum Bit Rate | M |  | 9.2.12 |  | – | – |
| >Subscriber Profile ID for RAT/Frequency priority | O |  | 9.2.25 |  | – | – |
| >E-RABs To Be Setup List |  | 1 |  |  | – | – |
| >>E-RABs To Be Setup Item |  | 1 .. <maxnoof Bearers> |  |  | EACH | ignore |
| >>>E-RAB ID | M |  | 9.2.23 |  | – | – |
| >>>E-RAB Level QoS Parameters | M |  | 9.2.9 | Includes necessary QoS parameters | – | – |
| >>>DL Forwarding | O |  | 9.2.5 |  | – | – |
| >>>UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.2.1 | SGW endpoint of the S1 transport bearer. For delivery of UL PDUs. | – | – |
| >RRC Context | M |  | OCTET STRING | Includes the RRC Handover Preparation Information message as defined in subclause 10.2.2 of TS 36.331 [9] | – | – |

For NR messages, we would like to discuss a possibility to provide a finer granularity of criticality, preferably on IE level. This is something RAN2 should do together with RAN3.

Proposal 1: RAN2 should, together with RAN3, identify the criticality of the RRC containers on an IE level.

## Common radio configuration

The common antenna information is used in LTE to inform the UE about the antenna configuration for CRS. This should not be relevant for NR, and can in our understanding be omitted.

Proposal 2: antennaInfoCommon is not provided in the HandoverPreparationInformation

## QoS flow to DRB mapping

FFS Whether the AS configuration includes the QoS flow to DRB mapping that was not configured to the UE via RRC signalling.

## Providing minimum system information

Our understanding is that the full system information is only provided from the source to be further forwarded to the UE so that the UE can do interference cancellation of the system information transmission. For NR, it is not clear that the system information transmission is creating similar interference as for LTE (especially due to lack of CRS), and we would propose to not provide full minimum system information in NR.

Proposal 3: Minimum system information is not provided in the HandoverPreparationInformation

## Beam level measurement information

It has been agreed that the available beam measurement information can be part of the RRM configuration of the HandoverPreparationInformation message if beam measurement information (i.e. beam indexes and optionally measurement results) have been configured by the source gNodeB to be reported by a UE. That information is not a mandatory part of the HandoverPreparationInformation message. However, it was not possible to decide for which cell(s) beam measurement information can be included.

Our preference would be to allow the source gNB to include the beam measuremen information for all cells it is available.

Proposal 4: The RRM configuration can include both beam measurement information  associated to SS Block(s) and CSI-RS(s) for all available cells.

# Conclusion

Based on the discussion in Section 2, we propose following

Proposal 1: RAN2 should, together with RAN3, identify the criticality of the RRC containers on an IE level.

Proposal 2: antennaInfoCommon is not provided in the HandoverPreparationInformation

Proposal 3: Minimum system information is not provided in the HandoverPreparationInformation

Proposal 4: The RRM configuration can include both beam measurement information  associated to SS Block(s) and CSI-RS(s) for all available cells.

In addition, we propose to agree to text proposal in section 4.

# Text proposal 38.331

# 10 Radio information related interactions between network nodes

## 10.1	General

This section specifies RRC messages that are transferred between network nodes. These RRC messages may be transferred to or from the UE via another Radio Access Technology. Consequently, these messages have similar characteristics as the RRC messages that are transferred across the NR radio interface, i.e. the same transfer syntax and protocol extension mechanisms apply.

## 10.2	Inter-node RRC messages

### 10.2.1	General

This section specifies RRC messages that are sent either across the X2, Xn- or the NG-C -interface, either to or from the eNB, i.e. a single 'logical channel' is used for all RRC messages transferred across network nodes. The information could originate from or be destined for another RAT.

### –	EUTRA-InterNodeDefinitions

This ASN.1 segment is the start of the -network inter-node PDU definitions.

-- ASN1START

-- TBD

-- ASN1STOP

### 10.2.2	Message definitions

#### –	HandoverCommand

This message is used to transfer the handover command generated by the NR RRC protocol entity in the target NG-RAN node.

Direction: target NG-RAN node to source source RAN

HandoverCommand message

-- ASN1START

HandoverCommand ::=					SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE{

handoverCommand-r15					HandoverCommand-r15-IEs,

spare7 NULL,

spare6 NULL, spare5 NULL, spare4 NULL,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

HandoverCommand-r15-IEs ::=			SEQUENCE {

handoverCommandMessage				OCTET STRING (CONTAINING DL-DCCH-Message),

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

-- ASN1STOP

HandoverCommand field descriptions

handoverCommandMessage Contains the entire DL-DCCH-Message including the RRCConnectionReconfiguration message used to perform handover within NG-RAN or handover to NG-RAN, generated (entirely) by the target gNB.

#### –	HandoverPreparationInformation

This message is used to transfer the NG-RAN RRC information used by the target NR RRC protocol entity in the target NG-RAN node during handover preparation, including UE capability information.

Direction: source  RAN to target NG-RAN node.

HandoverPreparationInformation message

-- ASN1START

HandoverPreparationInformation ::=	SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE{

handoverPreparationInformation-r15	HandoverPreparationInformation-r15-IEs,

spare7 NULL,

spare6 NULL, spare5 NULL, spare4 NULL,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

HandoverPreparationInformation-r15-IEs ::= SEQUENCE {

ue-RadioAccessCapabilityInfo		UE-CapabilityRAT-ContainerList,

as-Config							AS-Config					OPTIONAL, 		-- Cond HO

rrm-Config							RRM-Config					OPTIONAL,

as-Context							AS-Context					OPTIONAL, 		-- Cond HO

ue-ConfigRelease-r9					ENUMERATED {

rel15, rel16, rel17, rel18, rel19, rel20,

rel21, rel22, ...} 		OPTIONAL,

nonCriticalExtension				SEQUENCE {}					OPTIONAL

}

-- ASN1STOP

HandoverPreparationInformation field descriptions

as-Config The radio resource configuration. Applicable in case of intra-NR handover. If the target receives an incomplete MeasConfig and RadioResourceConfigDedicated in the as-Config, the target gNB may decide to apply the full configuration option based on the ue-ConfigRelease.

as-Context Local NG-RAN context required by the target eNB.

rrm-Config Local NG-RAN context used depending on the target node’s implementation, which is mainly used for the RRM purpose.

ue-ConfigRelease Indicates the RRC protocol release or version applicable for the current UE configuration. This could be used by target gNB to decide if the full configuration approach should be used. If this field is not present, the target assumes that the current UE configuration is based on the release 15 version of RRC protocol. NOTE 1.

ue-RadioAccessCapabilityInfo For NR radio access capabilities, it is up to NR how the backward compatibility among supportedBandCombinationReduced, supportedBandCombination and supportedBandCombinationAdd is ensured. If supportedBandCombinationReduced and supportedBandCombination/supportedBandCombinationAdd are included into ueCapabilityRAT-Container, it can be assumed that the value of fields, requestedBands, reducedIntNonContCombRequested and requestedCCsXL are consistend with all supported band combination fields.

NOTE 1:	The source typically sets the ue-ConfigRelease to the release corresponding with the current dedicated radio configuration. The source may however also consider the common radio resource configuration e.g. in case interoperability problems would appear if the UE temporary continues extensions of this part of the configuration in a target PCell not supporting them.

| Conditional presence | Explanation |
| --- | --- |
| HO | The field is mandatory present in case of handover within NR; otherwise the field is not present. |

#### –	SCG-Config

[To be based on SCG-ConfigNR in the 36.331]

#### –	SCG-ConfigInfo

[To be based on copied from SCG-ConfigInfoNR in the 36.331]

## 10.3	Inter-node RRC information element definitions

#### –	AS-Config

The AS-Config IE contains information about RRC configuration information in the source RAN which can be utilized by target NG-RAN node to determine the need to change the RRC configuration during the handover preparation phase. The information can also be used after the handover is successfully performed or during the RRC connection re-establishment or resume.

AS-Config information element

-- ASN1START

AS-Config ::=				SEQUENCE {

sourceRadioBearerConfig					RadioBearerConfig,

sourceMasterCellGroupConfig				CellGroupConfig,

sourceSecondaryCellGroupConfig			CellGroupConfig					OPTIONAL,

sourceMeasConfig						MeasConfig,

sourceOtherConfig						OtherConfig,

sourceSecurityAlgorithmConfig			SecurityAlgorithmConfig,

sourceUE-Identity						C-RNTI,

sourceDl-CarrierFreq					ARFCN-ValueEUTRA,

...,

}

-- ASN1STOP

NOTE:	The AS-Config re-uses information elements primarily created to cover the radio interface signalling requirements. Consequently, the information elements may include some parameters that are not relevant for the target NG-RAN node e.g. the SFN as included in the MasterInformationBlock.

#### –	AS-Context

The IE AS-Context is used to transfer local E-UTRAN context required by the target eNB.

AS-Context information element

-- ASN1START

AS-Context ::=							SEQUENCE {

reestablishmentInfo						ReestablishmentInfo			OPTIONAL,

...

}

-- ASN1STOP

#### –	RRM-Config

The RRM-Config IE contains information about UE specific RRM information before the handover which can be utilized by target  NG-RAN node.

RRM-Config information element

-- ASN1START

RRM-Config ::=				SEQUENCE {

ue-InactiveTime				ENUMERATED {

s1, s2, s3, s5, s7, s10, s15, s20,

s25, s30, s40, s50, min1, min1s20c, min1s40,

min2, min2s30, min3, min3s30, min4, min5, min6,

min7, min8, min9, min10, min12, min14, min17, min20,

min24, min28, min33, min38, min44, min50, hr1,

hr1min30, hr2, hr2min30, hr3, hr3min30, hr4, hr5, hr6,

hr8, hr10, hr13, hr16, hr20, day1, day1hr12, day2,

day2hr12, day3, day4, day5, day7, day10, day14, day19,

day24, day30, dayMoreThan30}		OPTIONAL,

candidateCellInfoList		CandidateCellInfoList

...

}

CandidateCellInfoList ::=	SEQUENCE (SIZE (1..maxFreq)) OF CandidateCellInfo

CandidateCellInfo-r10 ::=		SEQUENCE {

-- contents TBD

...

}

-- ASN1STOP

## 10.4	Inter-node RRC multiplicity and type constraint values

### –	Multiplicity and type constraints definitions

-- ASN1START

-- TBD

-- ASN1STOP

### –	End of EUTRA-InterNodeDefinitions

-- ASN1START

END

-- ASN1STOP

## 10.5	Mandatory information in AS-Config

The AS-Config transferred between source RAN and target-NG-RAN node shall include all IEs necessary to describe the AS context. The conditional presence in section 6 is only applicable for RAN to UE communication.

The "need" or "cond" statements are not applied in case of sending the IEs from source RAN to target NG-RAN node. Some fields shall be included regardless of the "need" or "cond" e.g. discardTimer. The AS-Config re-uses information elements primarily created to cover the radio interface signalling requirements. The information elements may include some parameters that are not relevant for the target NG-RAN node e.g. the SFN as included in the MasterInformationBlock.

Within the sourceRadioResourceConfig, sourceMeasConfig and sourceOtherConfig, the source RAN shall include fields that are optional for RAN to UE communication, if the functionality is configured unless explicitly specified otherwise in the following:

-	in accordance with a condition that is explicitly stated to be applicable; or

-	a default value is defined for the concerned field; and the configured value is the same as the default value that is defined; or

-	the need of the field is OP and the current UE configuration corresponds with the behaviour defined for absence of the field;

The following fields, if the functionality is configured, are not mandatory for the source eNB to include in the AS-Config since delta signalling by the target eNB for these fields is not supported:

- semiPersistSchedC-RNTI

- measGapConfig

For the measurement configuration, a corresponding operation as 5.5.6.1 and 5.5.2.2a is executed by target eNB.