3GPP TSG-RAN WG2 Meeting #99bis 	R2-1710890

# Prague, Czechia, 9-13 Oct 2017

Agenda item:	9.4.4

Source: 	Qualcomm Incorporated

Title: 	Handover performance results for aerial vehicles

Document for:     Discussion/Decision

# Introduction

Cellular connectivity will be key for coordinated operation and control of Unmanned Aerial Vehicles (UAVs), more commonly referred to as “Drones”, enabling a growing set of use cases within and beyond the drone operator’s visual line of sight. RAN#75 approved a study item on Enhanced Support for Aerial Vehicles [1].

One of the objectives of the study is [1]:

- Handover: Identify if enhancements in terms of cell selection and handover efficiency as well as robustness in handover signalling can be achieved. [RAN2, RAN1]

In this contribution, we present some results on mobility performance of aerial vehicles taking into account the latest RAN2 assumptions.

# Mobility Analysis

Mobility simulations are performed to analyze handover performance and to compare handover and link reliability between ground/terrestrial UEs and airborne UAVs.

## Simulation Setup

For the mobility study, each modeled UE starts at a randomly selected location in the network, and a bearing angle is selected randomly and uniformly. The UE then moves at the assigned speed in a straight line for the duration of the simulation with wrap around upon hitting the simulation boundary.

The following table shows the parameters used to model the handover for these simulations.

| Parameter | Description/Note | - Value |
| --- | --- | --- |
| Cell layout and scenario | Same as RAN1 | Hexagonal grid, 19 macro sites, 3 sectors per site ISD = 500m for UMa ISD = 1732m for RMa |
| Band/bandwidth | Same as RAN1 | - 2000 MHz band for UMa - 700 MHz band for RMa 10 MHz bandwidth |
| TimeToTrigger | Time to trigger a measurement report | 160 ms |
| A3Offset | Offset between signal strength of serving and neighboring cells | 2 dB |
| MeasurementInterval | Physical layer measurement interval | 10 ms |
| TMeasurement_Period, Intra | L1 filtering time in TS 36.133 | 200 ms |
| L3RRMCoefficient | Filtering coefficient for layer 3 measurements | 1 |
| Qin | Qin Threshold | -6 dB |
| Qout | Qout Threshold | -8 dB |
| TEvaluate_Qout | Qout evaluation period | L1 samples filtered linearly over a sliding window of 200 ms |
| TEvaluate_Qin | Qin evaluation period | L1 samples filtered linearly over a sliding window of 100 ms |
| T310 | Timer to trigger radio link failure | 1 s |
| N310 | Maximum number of consecutive "out-of-sync" indications from lower layers | 1 |
| T311 | Not used (RLF recovery not simulated) | Not used (RLF recover not simulated) |
| N311 | Maximum number of consecutive "in-sync" indications from lower layers | 1 |
| HOPreparationDelay | Handover preparation delay | 50 ms |
| HOExecutionDelay | Handover execution delay | 40 ms |
| RSRPError | Standard deviation of RSRP measurement error | 1.22 dB |
| MTS | Minimum time to stay for ping-pong metric | 1 s |
| UE height | Height in meter above ground level NOTE: 0 m AGL corresponds to ground UEs, in which case the UEs are modelled as being at 1.5m height. | 0 m, 50 m, 100 m, 300 m (AGL) |
| UE speed | Horizontal speed for terrestrial and aerial UTs | 3 km/h, 30 km/h, 60 km/h, 160 km/h |
| Total number of UEs | Same as RAN1 | 15 |
| Outdoor terrestrial UE ratio |  | 100% |
| Aerial UT ratio | Ratio of number of Aerial UTs to total UTs per sector | 0% (i.e., all terrestrial UTs) 100% (i.e., all Aerial UTs) |
| Traffic model | Traffic model for terrestrial and aerial UTs | Full buffer |

## Analysis of results

In this section, we evaluate the results of the handover simulations in different scenarios using performance metrics tabulated in Table 2. All values are averages.

Table 2 KPIs

| KPI | Unit | Description |
| --- | --- | --- |
| Handover rate | HO/UE/sec | Number of HO attempts over time (including HOF) |
| HOF rate | % | Number of HO failures/Total number of HO attempts (including HOF) |
| Radio Link Failure (RLF) rate | RLF/UE/sec | Number of RLFs over time |
| Time in handoff | % | Fraction of time a UE is in HO procedure including time for successful HO (HO execution delay) and HOF (reestablishment delay) |
| Time in Qout | % | Fraction of time a UE is in Qout state |
| Ping pong rate | % | Number of ping-pongs/Total number of successful handovers (excluding handover failures) |

Figure 1 shows the plots for all the KPIs for UMa scenario.

Figure 1. UMa

Observations from Figure 1 (UMa Scenario):

- HO rate (attempts) increases with increase in speed as expected.

- HO rates are lower for UAVs at lower speed compared to terrestrial UEs, but higher for UAVs compared to terrestrial UEs at the maximum considered speed.

- HOF rates are much lower for UAVs at lower speed compared to terrestrial UEs. HOF rates increase with speed and are within comparable range at medium speed and slightly higher for UAVs at high speed.

- RLF increases significantly with speed. UAVs in general suffer higher RLF than terrestrial UEs.

- Ping-pong rates go down with increase in speed. Terrestrial UEs suffer higher ping-pong compared to UAVs in general (except for very high speed). The ping-pong rates for UAVs are almost half compared to grand UEs for low to moderate speed. This is expected to be because of direct LOS for UAVs and more NLOS for terrestrial UEs.

- Time in Handoff and Time in Qout are lower for UAVs at low speed, comparable to terrestrial UEs for medium speeds and higher for UAVs at high speed.

- In terms of comparison of UEs at different heights for the same speed, in general the mobility performance is worse for UAVs when the height is in the mid range. This is possibly due to the nulls in-between different vertical lobes of antenna as previously illustrated by different companies (e.g., [2]) where a UE may be served by a side lobe of a far cell instead of the main lobe of a near cell.

Figure 2 shows the plots for all the KPIs for RMa scenario.

Figure 2. RMa

Observations from Figure 2 (RMa Scenario):

- Similar to UMa, HO attempt rate increases with increase in speed as expected. The HO rates for RMa are lower than UMa due to different ISD.

- Unlike UMa, HO rates are generally higher for UAVs for all considered speeds in RMa scenario.

- Unlike UMa, HOF rates in RMa highly depend on height. E.g., the difference between the HOF rate for 50m and 100m are higher in RMa than in UMa.

- Similar to UMa, RLF increases significantly with the UE speed in RMa. However, unlike UMa, at the highest considered speed, the RLF rate for UAVs with medium height (100m AGL) is significantly higher than that for UAVs at lower or much higher altitudes. This could also be due to the vertical antenna pattern (UE being served by sidelobe of far cell).

- Similar to UMa, Ping-pong rates go down with increase in speed in RMa. Terrestrial UEs suffer higher ping-pong (about 1.5x) compared to UAVs in general.

- Time in Handoff and Time in Qout for UAVs at medium height is more prominent compared to other heights for RMas, especially at medium and high speeds.

- When comparing UEs with different heights at the same speed, in general, the mobility performance is worse for UAVs when the height is in the mid-range. This is more clearly seen in RMa compared to UMa scenario.

Overall, these observations from the above results can be summarized as:

For low to moderate speeds and heights, UAVs show on par or better mobility performance compared to terrestrial UEs due to more stable radio conditions (LOS and free space propagation).

For high speed and for certain heights (depending on the considered scenario), UAV mobility performance suffers compared to terrestrial UEs which is likely due to higher level of interference and because of the nulls in the vertical antenna pattern.

Based on the above observations and discussion, we propose:

Take the above results into account in the discussions of potential mobility enhancement solutions for UAVs.

Capture the results in the TR.

# Conclusion

In this contribution, based on the simulation results, we made the following observations on mobility performance of UAVs:

Observation 1.	For low to moderate speeds and heights, UAVs show on par or better mobility performance compared to terrestrial UEs due to more stable radio conditions (LOS and free space propagation).

Observation 2.	For high speed and for certain heights (depending on the considered scenario), UAV mobility performance suffers compared to terrestrial UEs which is likely due to higher level of interference and because of the nulls in the vertical antenna pattern.

Based on the discussion and observations, we propose:

Proposal 1.	Take the above results into account in the discussions of potential mobility enhancement solutions for UAVs.

Proposal 2.	Capture the results in the TR.

# References

[1] RP-170779, SID on Enhanced LTE Support for Aerial Vehicles, NTT DOCOMO INC, Ericsson

[2] R2-1708662, Potential enhancements for UAV interference problem, Ericsson