3GPP TSG-RAN WG2 #99bis	Tdoc R2-1711237

Revision of R2-1708324

Prague, Czech Republic, 9th – 13th October 2017

Agenda Item:	10.3.4.2

Source:	Ericsson

Title:	SDAP header format

Document for:	Discussion, Decision

# Introduction

RAN2-99 in Berlin agreed:

Agreements

1.	RAN2 aims at designing a 1 byte SDAP header.  Whether the QFI is 6 bit or 7 bits is FFS.

2.	If configured, SDAP header size for a DRB is static (assuming 1 byte header).  The QFI will always be present.

3. 	No SN will be introduced in SDAP

In RAN2 June Ad Hoc following was agreed:

Agreements:

- There is a need to tell the UE that it has to update the mapping rule.   For the AS reflective QoS it is up to the RAN to decide when to update the mapping rules.  FFS on the details of the header format.

- It is up to the RAN to decide when and which mechanism, explicit RRC re-configuration and/or AS reflective QoS, should be used to provide mapping information to the UE.

- A UE follows the latest QoS flow to DRB mapping information regardless of the fact whether it was explicit RRC or AS reflective QoS.

- Whether a SDAP header is present or not is configured by RRC per DRB

- The gNB indicates to UE using RRC signaling the default DRB for a PDU session.

- RAN decides and configures the default DRB for a PDU session.

This contribution discusses about remaining open issues with the SDAP header after the RAN2#99 meeting.

# Discussion

## QFI alignment with SA2

In order to allow efficient implementation from gNB towards 5CN, SDAP header format should be aligned with the N3 header format. QFI is always attached to N3 header when there are multiple flows in PDU session. Having QFI aligned between SA2 and RAN allows implementations to copy the QFI to SDAP header and vice versa.

SDAP header format is aligned with SA2 to allow QFI and RQI copying behaviour from and to N3 header.

## Header format options

RAN2-99 agreed to have SDAP header design, where SDAP header length is 1-byte. This leaves two options for QFI size, either 6 or 7-bit QFI.

Figure 1: SDAP 1-byte header format options

In RAN2 #99, decision was made to include QFI always in DL and UL SDAP header. The question of QFI length remains still open. Possible sizes of QFI are 6 and 7 bits. SDAP header is assumed to be one byte and Maximum defined QFI value in 5QI table defined by SA is 79 [1]. Having value range, which is able to indicate full value range of 5QI (from 0 to 79) is beneficial as it results that the standardized 5QI values can be directly signalled to the gNB and UE without need for intermediate mapping from 5QI value to RAN QFI value.

Having 7-bit QFI allows full value range of 5QI table to be signalled.

Alternative option for the 7-bit QFI is to have 6-bit QFI. This results that there are two bits in both, DL and UL that can be used for other purposes. In DL, there is need to convey RQI bit from the N3 header and therefore, there would be only one spare bit (R) with 6-bit QFI. However, having 6-bit QFI would require either RAN or 5CN to make mapping from 5QI value to QFI value in order to convey large 5QI values. For non-standardized 5QI values this is not a problem, as there is always QoS profile associated with the QFI and the mapping could be carried there, but for standardized QFIs, coordination, or mapping function would be needed when value of the standardized QFI would exceed the 6-bit value range. Therefore, the most convenient solution would be to carry 7-bit QFI from CN to UE.

- Confirm that the DL and UL SDAP header contains a 7-bit Flow ID

For UL, QFI provides information to the gNB from which gNB is able to observe the QoS marking to be carried in NG3 UL header. There is no need to have RQI information in the UL header. Hence the resulting UL header has one spare bit (R) for later use as shown in Figure 1.

- The UL SDAP header has one spare bit (R) for later use.

The SA2 specifies in TS 23.501: “when an RQI is received by RAN in a DL packet on N3 reference point, the RAN shall indicate to the UE the QFI and that this DL packet is subject to Reflective QoS.”  This results that the SDAP header is required to have capability to pass the RQI information per packet basis and current working assumption is that there is RQI indication in SDAP header. However, it has not yet been agreed whether the same mechanism can be used to inform UE about AS reflective QoS QFI to DRB mapping update.

The mechanism would be simple from specification and implementation perspective, when RQI is set to ‘1’, the UE would check if NAS or AS mapping rules require update. Furthermore, as both reflective mechanisms are independently configurable, the UE knows from the configuration whether it needs to update one or both mappings. Therefore, we propose that the working assumption is confirmed.

- Working assumption is confirmed that the DL SDAP header includes 1-bit RQI indication which indicates whether NAS and/or AS mapping rules need to be updated.

## Control Headers and SDAP header placement

In previous meeting, companies proposed to have control PDU for SDAP layer. Information carried by the control PDU would be, for instance control of AS reflective QoS activation / deactivation. Having control PDU is not necessary as the controlling of the AS reflective QoS can be left for the gNB implementation. Since SDAP layer is used currently only with QoS and it is currently tightly coupled with the PDCP entity, control PDU would add complexity that may not be justified with the benefits. Additionally, dynamic SDAP header (control PDUs) introduces complexity to the ROHC implementation as the position of the IP packet needs to be known by ROCH. Alternatively having SDAP header at the end would avoid the ROCH problems but introduce coupling of SDAP header with PDCP SDU length information. Parsing from the end with dynamic headers would create complexity to the receiver parser as the receiver would need to predict the length of the SDAP header or have it indicated otherwise.

Control headers are not introduced in SDAP layer.

SDAP header is placed at the beginning of the PDU.

# Conclusion

In section 2 we made the following observations:

Observation 1	Having 7-bit QFI allows full value range of 5QI table to be signalled.

Based on the discussion in section 2 we propose the following:

Proposal 1	SDAP header format is aligned with SA2 to allow QFI and RQI copying behaviour from and to N3 header.

Proposal 1	Confirm that the DL and UL SDAP header contains a 7-bit Flow ID

Proposal 2	The UL SDAP header has one spare bit (R) for later use.

Proposal 3	Working assumption is confirmed that the DL SDAP header includes 1-bit RQI indication which indicates whether NAS and/or AS mapping rules need to be updated.

Proposal 4	Control headers are not introduced in SDAP layer.

Proposal 5	SDAP header is placed at the beginning of the PDU.

# References

TS 23.501, System Architecture for the 5G System, V1.3.0 (2017-09).