3GPP TSG-RAN WG2 Meeting #99bis 	R2-1711284

Prague, Czech Republic, 9 - 13 October 2017	R2-1709203

Agenda item:	10.2.15

Source:	Nokia, Nokia Shanghai Bell

Title:	Slice assistance Information over RRC

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

Network Slicing technical realization with UE assistance has been recognized by SA2, RAN3 and RAN2. In this context, RAN2 #98 meeting agreed the following:

Agreements:

1	RAN2 assumption is that MSG3 does not to deliver assistance information for AMF selection due to RRC size constraints as in LTE.

2	RAN2 assumption is that MSG5 is the earliest message that can be used to deliver assistance information for AMF selection.

Besides, SA2 has discussed the topic of the size of the NSSAI and agreed that a UE may at most register with 8 S-NSSAI instances simultaneously [3]. This establishes requirements for detailed RRC signalling, while RAN2 did not agreed more detailed signalling content yet.

This contribution provides an overview of latest status and conclusions related to the UE assistance in Network Slicing and proposes to progress the RAN2 work.

# 2	Discussion

## 2.1	Slice assistance information requirements

Network Slicing requires that slice-enabled UE provides its pre-configured NSSAI (Network Slice Selection Assistance Information) to facilitate appropriate CN instance and slice selection [1]. The slice identification is concerned over the radio interface as this is used by RAN for different purposes:

- Select appropriate RAN part and resources of slice,

- Enable selection of the appropriate AMF node.

The provisioned assistance information (NSSAI) consist of multiple S-NSSAIs. SA2 agreed that a UE may at most register with 8 S-NSSAI instances simultaneously. This means that in the worst case the UE may request a NSSAI with 8 S-NSSAI instances in it [3], where S-NSSAI format is comprised of the following:

-	A Slice/Service type (SST), which refers to the expected Network Slice behavior in terms of features and services;

-	(optional) a Slice Differentiator (SD). which information that complements the Slice/Service type(s) to allow further differentiation for selecting an Network Slice instance from the potentially multiple Network Slice instances that all comply with the indicated Slice/Service type. This information is referred to as SD.

Observation 1: NSSAI is indicating one or a list of S-NSSAIs, which identify expected characteristics and behaviour in terms of features and services.

Observation 2: NSSAI can at most consist of 8 S-NSSAI instances simultaneously.

Figure 1: AMF instance selection (as per TS38.300)

2.2	NSSAI format over the RRC

## There has been two ways considered for providing the Assistance Info (as per Figure 1) by the UE over RRC:

- Implicit or/and reduced form of S-NSSAI(s) requiring mapping at the UE

- Explicit information according to the S-NSSAI(s) format received by upper layer

The implicit format has been considered in RAN3 due to RRC messages size constrains. Given the RAN2 agreements [4], the assumed RRC message for slicing assistance information delivery, i.e. RRC Connection Setup Complete (MSG5) has not itself be considered as size critical. In addition, addressing potential impact to RRC messages size, SA2 agreed that [3]:

- only the SST field is mandatory, and the SD field may be omitted when it is not needed

- the SST should consists of 8 bits (with range is 0-255)

- the SD should consist of 24 bits.

The most frequent slices types will be standardized ones that are just using the SST field of the S-NSSAI only. Hence, focusing on the use cases where the full NSSAI is to be sent (as RRC does not distinguish the use cases) it would imply the worst case of 64 bits if we allow a S-NSSAI to be sent as just 8 bits when the S-NSSAI misses the SD field.

Observation 3: Neither standard requirements nor practical deployment scenarios indicate optimization of S-NSSAI size is needed.

Use of S-NSSAI list across all the other interfaces is aligned, hence we believe, the alternative and optimization of S-NSSAI could actually increase the complexity. The use of a new short indicator in other format representing the combination of S-NSSAI supported by the UE would require additional standardization and implementation efforts in terms of algorithms determining the actual format.

Therefore, we propose to follow SA2 requirements explicitly:

Proposal 1: The UE conveys slice assistance information over RRC in the explicitly indicated format by the upper layer.

Proposal 2: The slice assistance information consists of one or a list of S-NSSAIs, where an S-NSSAI is a combination of:

- mandatory SST (Slice/Service Type) field, which identifies the slice type and consists of 8 bits (with range is 0-255)

- optinal SD (Slice Differentiator) field, which differentiates among Slices with same SST field and consist of 24 bits.

Proposal 3: The UE conveys up to 8 S-NSSAIs over the RRC.

2.3	RRC message for slice assistance information delivery

RAN3 agreed that RAN uses the assistance information provided by the UE at:

- RRC connection establishment to select the appropriate AMF instance [6].

RAN2 assumption that

- MSG5 is the earliest message that can be used to deliver assistance information for AMF selection is corresponding.

Therefore, we believe for the sake of concept consistency MSG5 is apparent selection for the slice assistance information delivery.

Proposal 4: RRC Connection Setup Complete conveys the slice assistance information once provided by the upper layer.

# 3	Conclusion

We conclude the contribution with the following observations and proposals to be agreed:

Observation 1: NSSAI is indicating one or a list of S-NSSAIs, which identify expected characteristics and behaviour in terms of features and services.

Observation 2: NSSAI can at most consist of 8 S-NSSAI instances simultaneously.

Observation 3: Neither standard requirements nor practical deployment scenarios indicate optimization of S-NSSAI size is needed.

Proposal 1: The UE conveys slice assistance information over RRC in the explicitly indicated format by the upper layer.

Proposal 2: The slice assistance information consists of one or a list of S-NSSAIs, where an S-NSSAI is a combination of:

- mandatory SST (Slice/Service Type) field, which identifies the slice type and consists of 8 bits (with range is 0-255)

- optinal SD (Slice Differentiator) field, which differentiates among Slices with same SST field and consist of 24 bits.

Proposal 3: The UE conveys up to 8 S-NSSAIs over the RRC.

Proposal 4: RRC Connection Setup Complete conveys the slice assistance information once provided by the upper layer.

Proposal 5: Agree Text Proposal in the Annex below.

# Annex:Text Proposal to 38.300

Modified Subclause

# 3	Abbreviations and Definitions

## 3.1	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1], in 3GPP TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1] and 3GPP TS 36.300 [2].

5GC	5G Core Network

AMF	Access and Mobility Management Function

gNB	NR NodeB

NG-RAN	NG Radio Access Network

NCGI	NR Cell Global Identifier

NR	NR Radio Access

QFI	QoS Flow ID

RLAU	RAN-based Location Area Update

RNA	RAN Notification Area

SD	Slice Differentiator

SDAP	Service Data Adaptation Protocol

S-NSSAI	Single Network Slice Selection Assistance Information

SST	Slice/Service Type

SMF	Session Management Function

UPF	User Plane Function

Xn-C	Xn-Control plane

Xn-U	Xn-User plane

Next Modified Subclause

## 17.3	Network Slicing

### 17.3.1	General Principles and Requirements

In this sub clause, the general principles and requirements related to the realization of network slicing in the NG-RAN for NR connected to 5GC are given.

A network slice always consists of a RAN part and a CN part. The support of network slicing relies on the principle that traffic for different slices is handled by different PDU sessions. Network can realise the different network slices by scheduling and also by providing different L1/L2 configurations. UE should be able to provide assistance information for network slice selection in RRC Connection Setup Complete message, if it has been provided by NAS. While the network can support large number of slices (hundreds), the UE need not support more than 8 slices in parallel.

Network Slicing is a concept to allow differentiated treatment depending on each customer requirements. With slicing, it is possible for Mobile Network Operators (MNO) to consider customers as belonging to different tenant types with each having different service requirements that govern in terms of what slice types each tenant is eligible to use based on Service Level Agreement (SLA) and subscriptions.

NSSAI (Network Slice Selection Assistance Information) includes one or more S-NSSAIs (Single NSSAI). Each network slice is uniquely identified by a S-NSSAI, as defined in TS 23.501 [3].

The following key principles apply for support of Network Slicing in NG-RAN:

# RAN awareness of slices

-	NG-RAN supports a differentiated handling of traffic for different network slices which have been pre-configured. How NG-RAN supports the slice enabling in terms of NG-RAN functions (i.e. the set of network functions that comprise each slice) is implementation dependent.

# Selection of RAN part of the network slice

-	NG-RAN supports the selection of the RAN part of the network slice, by assistance information provided by the UE or the 5GC which unambiguously identifies one or more of the pre-configured network slices in the PLMN.

# Resource management between slices

-	NG-RAN supports policy enforcement between slices as per service level agreements. It should be possible for a single NG-RAN node to support multiple slices. The NG-RAN should be free to apply the best RRM policy for the SLA in place to each supported slice.

# Support of QoS

-	NG-RAN supports QoS differentiation within a slice.

# RAN selection of CN entity

-	For initial attach, the UE may provide assistance information to support the selection of an AMF. If available, NG-RAN uses this information for routing the initial NAS to an AMF. If the NG-RAN is unable to select an AMF using this information or the UE does not provide any such information the NG-RAN sends the NAS signalling to a default AMF.

-	For subsequent accesses, the UE provides a Temp ID, which is assigned to the UE by the 5GC, to enable the NG-RAN to route the NAS message to the appropriate AMF as long as the Temp ID is valid (NG-RAN is aware of and can reach the AMF which is associated with the Temp ID). Otherwise, the methods for initial attach applies.

# Resource isolation between slices

-	the NG-RAN supports resource isolation between slices. NG-RAN resource isolation may be achieved by means of RRM policies and protection mechanisms that should avoid that shortage of shared resources in one slice breaks the service level agreement for another slice. It should be possible to fully dedicate NG-RAN resources to a certain slice. How NG-RAN supports resource isolation is implementation dependent.

# Slice Availability

-	Some slices may be available only in part of the network. Awareness in the NG-RAN of the slices supported in the cells of its neighbours may be beneficial for inter-frequency mobility in connected mode. It is assumed that the slice configuration does not change within the UE’s registration area.

-	The NG-RAN and the 5GC are responsible to handle a service request for a slice that may or may not be available in a given area. Admission or rejection of access to a slice may depend by factors such as support for the slice, availability of resources, support of the requested service by other slices.

# Support for UE associating with multiple network slices simultaneously

-	In case a UE is associated with multiple slices simultaneously, only one signalling connection is maintained and for intra-frequency cell reselection, the UE always tries to camp on the best cell. For inter-frequency cell reselection, dedicated priorities can be used to control the frequency on which the UE camps.

# Granularity of slice awareness

-	Slice awareness in NG-RAN is introduced at PDU session level, by indicating the S-NSSAI corresponding to the PDU Session, in all signalling containing PDU session resource information.

# Validation of the UE rights to access a network slice

-	It is the responsibility of the 5GC to validate that the UE has the rights to access a network slice.  Prior to receiving the Initial Context Setup Request message, the NG-RAN may be allowed to apply some provisional/local policies, based on awareness of which slice the UE is requesting access to. During the initial context setup, the NG-RAN is informed for all network slices for which resources are being requested.

### 17.3.2	CN Instance and NW Slice Selection

#### 17.3.2.1	   CN-RAN interaction and internal RAN aspects

NG-RAN selects AMF based on a Temp ID or assistance information provided by the UE over RRC. The mechanisms used in the RRC protocol are described in the next sub clause.

Table 17.3.2.1-1 AMF selection based on Temp ID and assistance information

| Temp ID | Assistance Info | AMF Selection by NG-RAN |
| --- | --- | --- |
| not available or invalid | not available | Default AMF is selected |
| not available or invalid | present | Selects AMF which supports UE requested slices |
| valid | not available, or present | Selects AMF per CN identity information in Temp ID |

#### 17.3.2.2	Radio Interface Aspects

The UE conveys the slice assistance information over RRC in the explicitly indicated format by the upper layer. The slice assistance information consists of one or a list of S-NSSAIs, where an S-NSSAI is a combination of:

- mandatory SST (Slice/Service Type) field, which identifies the slice type and consists of 8 bits (with range is 0-255)

- optional SD (Slice Differentiator) field, which differentiates among Slices with same SST field and consist of 24 bits.

#### The list shall convey at most 8 S-NSSAI instances simultaneously.

### 17.3.3    Resource Isolation and Management

Resource isolation enables specialized customization and avoids one slice affecting another slice.

Hardware/software resource isolation is up to implementation. Each slice may be assigned with either shared or dedicated radio resource up to RRM implementation and SLA.

To enable differentiated handling of traffic for network slices with different SLA:

-	NG-RAN is configured with a set of different configurations for different network slices;

-	To select the appropriate configuration for the traffic for each network slice, NG-RAN receives relevant information indicating which of the configurations applies for this specific network slice.

### 17.3.4    Signalling Aspects

It is FFS whether signalling procedures will be kept in this section or moved to other sections describing procedures.

#### 17.3.4.1	   General

In this sub clause, signalling flows related to the realization of network slicing in the NG-RAN are given.

#### 17.3.4.2	   CN Instance and NW Slice Selection

RAN selects AMF based on a Temp ID or assistance information provided by the UE.

Figure 17.3.4.1-1: AMF instance selection

In case a Temp ID is not available, the RAN uses the assistance information provided by the UE at RRC connection establishment to select the appropriate AMF instance (the information is provided after MSG3 of the random access procedure). If such information is also not available, the RAN routes the UE to a default AMF instance.

End of Modified Subclause

# References

- 3GPP TS 23.501, System Architecture for the 5G System (Release 15), v.0.4.0, SA2

- 3GPP TR 23.799, Study on Architecture for Next Generation System (Release 14), v.2.0.0, SA2

- R2-1706318,  Response LS regarding RAN support for NW slicing, SA2

- R2-1706142, Reply LS regarding RAN support for NW slicing, RAN2

- RP-170823, New WID on New Radio Access Technology, NTT DOCOMO, INC.

- R3-171329, TP to 38.300, RAN3