3GPP TSG-RAN WG2 #99bis	Tdoc R2-1711369

# Prague, Czech Republic, 9th – 13th October 2017

Agenda Item:	10.4.4.6

Source:	Ericsson

Title:	Configuration of paging transmissions in multi-beam operation

Document for:	Discussion, Decision

# Introduction

Following agreements related to paging have been taken in RAN2 so far:

Agreements

1: UE in RRC_IDLE and RRC_INACTVE state monitors paging/notification every DRX cycle.

2: UE monitors one paging occasion per DRX cycle. Paging occasion is the time interval over which a paging message is transmitted by gNB.

3: The length of DRX cycle is configurable. A default DRX cycle length is provided in system information. Additionally, a UE specific DRX cycle length can also be provided to UE in dedicated signalling.

4: The number of paging occasions in the DRX cycle is configurable and provided in system information.

5: If multiple paging occasions are configured by network in the DRX cycle then UEs can be distributed to these paging occasions based on UE ID.

6: RAN2 understanding is that paging can be transmitted at least using beam sweeping (content of paging may be a paging indicator or the paging message, FFS)

7: Paging occasion can consists of multiple time slots (e.g. subframe or OFDM symbol). (Multiple time slots enables transmission of paging using a different set of DL TX beam(s) in each time slot, or could enable repetition - RAN1 decision).

8: The number of time slots in a paging occasion is provided in system information.

In lower carrier frequencies (e.g. below 6 GHz), paging (as well as SS Block transmission) is expected to be transmitted by means of a single omnidirectional or sector beam transmission. In these conditions, the NR paging conditions are likely to be similar to those of LTE, thus the LTE paging mechanism can be used as a baseline for NR.

On higher carrier frequencies though, the efficiency of omnidirectional or sector beam transmission may not be good enough. Instead the concept of beam sweeping of paging has been widely discussed, along with the side-effects from this type of transmission.

In this contribution we discuss some issues related to paging in multi-beam operation.

# Discussion

In low frequency bands, omnidirectional or wide-beam transmission will typically be used for paging delivery.

In higher frequencies, sweeping of narrow beams may also be used for paging in the same way as is already agreed for transmission of SS Blocks (RAN1).

In multi-beam operation, the gNB does not know the exact location of the UE within the cell, thus the paging message need to be transmitted in each beam in the beam sweep. As according to one of the RAN2 agreements, each Paging Occasion (PO) may consist of multiple time slots (number of time slots is provided in the system information) where each beam containing the paging message is transmitted within one time slot.

As to the RAN1 agreement, a beam sweep transmission will consist of an SS Block containing PSS/SSS/NR-PBCH information. Assuming the PSS/SSS/NR-PBCH information provided in each beam sweep transmission will only occupy a fraction of the carrier bandwidth, one can also assume that each beam will have capacity for transmission of other type of information, such as paging. Furthermore, if analog beamforming is used for the beam sweep, meaning that the gNB can only transmit in one beam direction at a time in the same cell, this capacity is likely to be wasted, unless it is utilized for transmission of other signals/data suitable for beam sweeping. Otherwise, this capacity may only be opportunistically utilized to transmit data to a UE which happens to have DL data pending and which happens to be located in the beam direction.

A paging solution in which the paging transmissions within a PO are placed in direct conjunction with the SS Block transmissions, i.e. by means of frequency multiplexing paging in the same OFDM symbols as the SS Block, is an efficient solution in terms of sweep duration and resource utilization. It is also favourable from other aspects, e.g. the UE would then be able to perform PSS/SSS synchronization, channel quality measurements and page monitoring in a single wake up period.

However, such a solution comes with the disadvantage that a narrow band capable UE cannot perform simultaneous SS Block and paging reception (assuming minimum bandwidth for a narrow band capable UE is the same as the bandwidth of the SS Block). For a narrow band capable UE, adjacent location of the SS Block and the paging transmission in time is therefore the preferred choice, whereas for a wide band capable UE, adjacent location of the SS Block and the paging transmission in frequency is more efficient, see Figure 1.

SSB1

SSB2

Paging 1

Paging 2

SSB1

SSB2

Paging 1

Paging 2

≈

≈

t

t

f

f

Figure 1: SS Blocks and paging transmission in adjacently in frequency and time.

The two paging methods of time- and frequency multiplexed SSB/paging has been discussed in RAN1 and it will also be further studied according to following RAN1 agreement.

- For paging in multi-beam operation, support beam sweeping for paging, and study the following methods: - Alt-1: Multiplexing paging with SS blocks - FFS: Details of paging - Alt-2: Adding another round of beam sweeping for paging - Note: Another round of beam sweeping is different from the beam sweeping of SS burst set

In multi-beam operation, narrow band capable UEs and wide band capable UEs have different prerequisites for reception of paging messages.

A paging solution in which the paging transmissions within a PO are placed in direct conjunction with SS Block transmissions is favourable from several aspects, e.g. the UE would then be able to perform PSS/SSS synchronization, channel quality measurements and page monitoring in a single wake up period.

If paging information is chosen to be transmitted in conjunction with SS Blocks, a potential solution could be to configure different paging transmissions within a PO for wide band and narrow band capable UEs.
For a wide band capable UE it would be a benefit to let the paging transmission within a PO coincide (i.e. frequency multiplexed) with SS Block transmissions as it would increase the radio resource efficiency. But for a narrow band capable UE, due to the bandwidth limitation, it will not be able to receive a paging message frequency multiplexed with an SS Block transmission. Hence, for narrow band capable UEs the paging transmissions must not coincide with SS Block transmissions.

Different solutions can be anticipated, as for example:

- Specify different paging transmissions within a PO for narrow band and wide band capable UEs and ensure that the paging transmissions for narrow band capable UEs never coincides or overlaps with an SS Block transmission.

- Use the same paging transmission as the basis for both narrow band and wide band capable UEs. If the paging transmission within a PO coincides or overlaps with an SS Block transmission, then a rule is added so that the paging transmission for the narrow band capable UE is moved to the next of the subsequent symbols or slots in which no SS Block is transmitted.

Yet another alternative could be to only transmit SS Blocks and paging transmission adjacently in time - which of course will be a waste of radio resource efficiency in the network.

Discuss whether different paging transmissions (within a PO) need to be configured for wide band and narrow band capable UEs when multi-beam operation is deployed.

# Conclusion

In this paper we made the following observation:

Observation 1	In multi-beam operation, narrow band capable UEs and wide band capable UEs have different prerequisites for reception of paging messages.

Based on the observation we propose the following:

Proposal 1	Discuss whether different paging transmissions (within a PO) need to be configured for wide band and narrow band capable UEs when multi-beam operation is deployed.

# References

- -