3GPP TSG-RAN WG2 #98	R2-1711576

Hangzhou, China, May 15 – May 19, 2017

Agenda item:		10.3.3.1 (NR_newRAT-Core)

Source:	LG Electronics Inc.

Title: 	TP on PDCP data volume calculation

Document for:	Discussion and Decision

1.	Introduction

The PDCP data volume calculation in TS38.323v1.0.0 is still FFS. This document provides a text proposal to PDCP data volume calculation.

2.	Text proposal to 38.323v1.0.0

RAN2 is asked to review and adopt the text proposal provided below.

5.6 Data volume calculation For the purpose of MAC buffer status reporting, the transmitting PDCP entity shall consider the following as PDCP data volume: - the PDCP SDU for which no PDCP Data PDU has been submitted to lower layers; - the PDCP Data PDU that has not been submitted to lower layers; - the PDCP Control PDU; - for AM DRBs, the PDCP SDU to be retransmitted according to subclause 5.1.2; - for AM DRBs, the PDCP Data PDU to be retransmitted according to subclause 5.5. For split bearers, when indicating the PDCP data volume to a MAC entity for BSR triggering and Buffer Size calculation, the UE shall: - if the PDCP data volume is less than ul-DataSplitThreshold: - indicate the PDCP data volume to the primary MAC entity; - indicate the PDCP data volume as 0 to the secondary MAC entity; - else: - indicate the PDCP data volume to both primary and secondary MAC entities.