3GPP TSG-RAN WG2 #99bis	R2-1711746

Prague, Czech Republic, 9 - 13 October 2017	Revision of R2-1708775

Agenda item:	10.3.2.6

Source:	Nokia, Nokia Shanghai Bell

Title:	RLC pre-processing

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, the UE pre-processing aspect at RLC layer is discussed and proposed to be left up to UE implementation. The following was agreed in the last RAN2#99 meeting in RLC and PDCP sessions, respectively:

=>	RAN2 can clarify UE requirement on PDCP discard and SN utilization

=>	The UE is allowed to pre-process data for split bearer before a request from lower layers is received and is allowed to submit to lower layers before a request is received.  A restriction on bad UE behaviour or a requirement on proper behaviour will be added.  FFS how to capture it (e.g.  capture how avoid bad UE behaviours related to which PDCP SN are sent to the RLC and not transmitted at the end and whether and how to capture a pre-processing limit)

Update of the contribution considers the issues if RLC PDUs would be allowed to be submitted to lower layer before grant reception. Also, it is proposed to add a note into RLC specification that UE may create RLC PDUs before receiving an UL grant.

# 2	Discussion

The most recently endorsed TS 38.322 includes references that imply pre-processing would be enforced by default in the UE, e.g., generating and modifying the headers. Given the agreement of UE being allowed to submit PDCP PDUs to lower layers before a grant reception, requiring to generate AMD/UMD PDU and associate the headers upon SDU reception from higher layers could be problematic. For instance, when the SDU needs to be discarded esp. for ARQ operation for RLC AM, for instance; this may lead to possible SN gaps in the transmissions which should be solved by a newly defined solution. Besides, in this case such generation of UMD/AMD PDUs upon SDU reception from higher layers should be fully an implementation choice, rather than procedural requirement by specification. For RLC UM mode, the type of header which to associate RLC SDU with is not even known in advance but is dependent on the possible segmentation – in this case it is hard to justify requiring to create the UMD PDU in advance by the procedure.

In general, the specification should not restrict implementation, hence, we propose such references should be taken out and leave the RLC pre-processing up to UE implementation as already agreed in general. We note that UE may do header/data pre-processing etc. according to implementation as long as it is able to obey the procedures how they’re specified when tested. However, we also note the UE may be allowed to form RLC PDUs before a transmission opportunity is notified by lower layers but as such it shall not be visible in the protocol operation.

Furthermore, as the RLC is performing segmentation, STATUS PDU creation, and polling which are all done in real-time, pre-submitting RLC PDUs to MAC should not be allowed at least in the procedural text as it comes with too much complexity. Besides, discarding such a RLC PDU may bring again the issue of SN gaps as discussed above or not being able to discard anymore may bring issues when employed with split/duplicated bearer, like HFN desynchronization. Hence, we think keeping the principle that RLC PDUs shall only be submitted to lower layers upon a transmission opportunity is justified.

# 3	Conclusions

Based on the above discussion, we make the following proposals.

Proposal 1: RLC PDUs are submitted to lower layers only when a transmission opportunity has been notified by lower layer – TX_Next (if incremented) is only updated in procedural text when the RLC PDU is delivered to lower layer.

Proposal 2: Add a NOTE in the RLC specification the UE may form RLC PDUs before notified by lower layer of a transmission opportunity.

Proposal 3: Pre-processing is left up to UE implementation (as already agreed).

Proposal 4: Agree the text proposal provided in Annex for TS 38.322.

# Annex: Text proposal to TS 38.322

*** Start of change ***

## 4.2	RLC architecture

### 4.2.1	RLC entities

The description in this sub clause is a model and does not specify or restrict implementations.

RRC is generally in control of the RLC configuration.

Functions of the RLC sub layer are performed by RLC entities. For a RLC entity configured at the gNB, there is a peer RLC entity configured at the UE and vice versa.

An RLC entity receives/delivers RLC SDUs from/to upper layer and sends/receives RLC PDUs to/from its peer RLC entity via lower layers.

An RLC PDU can either be a RLC data PDU or a RLC control PDU. If an RLC entity receives RLC SDUs from upper layer, it receives them through a single RLC channel between RLC and upper layer, and after forming RLC data PDUs from the received RLC SDUs, the RLC entity delivers the RLC data PDUs to lower layer through a single logical channel. If an RLC entity receives RLC data PDUs from lower layer, it receives them through a single logical channel, and after forming RLC SDUs from the received RLC data PDUs, the RLC entity delivers the RLC SDUs to upper layer through a single RLC channel between RLC and upper layer. If an RLC entity delivers/receives RLC control PDUs to/from lower layer, it delivers/receives them through the same logical channel it delivers/receives the RLC data PDUs through.

An RLC entity can be configured to perform data transfer in one of the following three modes: Transparent Mode (TM), Unacknowledged Mode (UM) or Acknowledged Mode (AM). Consequently, an RLC entity is categorized as a TM RLC entity, an UM RLC entity or an AM RLC entity depending on the mode of data transfer that the RLC entity is configured to provide.

A TM RLC entity is configured either as a transmitting TM RLC entity or a receiving TM RLC entity. The transmitting TM RLC entity receives RLC SDUs from upper layer and sends RLC PDUs to its peer receiving TM RLC entity via lower layers. The receiving TM RLC entity delivers RLC SDUs to upper layer and receives RLC PDUs from its peer transmitting TM RLC entity via lower layers.

An UM RLC entity is configured either as a transmitting UM RLC entity or a receiving UM RLC entity. The transmitting UM RLC entity receives RLC SDUs from upper layer and sends RLC PDUs to its peer receiving UM RLC entity via lower layers. The receiving UM RLC entity delivers RLC SDUs to upper layer and receives RLC PDUs from its peer transmitting UM RLC entity via lower layers.

An AM RLC entity consists of a transmitting side and a receiving side. The transmitting side of an AM RLC entity receives RLC SDUs from upper layer and sends RLC PDUs to its peer AM RLC entity via lower layers. The receiving side of an AM RLC entity delivers RLC SDUs to upper layer and receives RLC PDUs from its peer AM RLC entity via lower layers.

Figure 4.2.1-1 illustrates the overview model of the RLC sub layer.

Figure 4.2.1-1: Overview model of the RLC sub layer

The following applies to all RLC entity types (i.e. TM, UM and AM RLC entity):

-	RLC SDUs of variable sizes which are byte aligned (i.e. multiple of 8 bits) are supported;

-	When RLC entity forms RLC PDUs, the formed RLC PDUs are only delivered to lower layer when a transmission opportunity has been notified by lower layer (i.e. by MAC).

NOTE: UE may form RLC PDUs before notified by lower layer of a transmission opportunity.

Description of different RLC entity types are provided below.

*** End of change ***

*** Start of change ***

#### 4.2.1.2	UM RLC entity

##### 4.2.1.2.1	General

An UM RLC entity can be configured to deliver/receive RLC PDUs through the following logical channels:

- DL/UL DTCH

Figure 4.2.1.2.1-1: Model of two unacknowledged mode peer entities

An UM RLC entity delivers/receives the following RLC data PDU:

-	UMD PDU.

An UMD PDU contains either one complete RLC SDU or one RLC SDU segment.

Editor’s note: FFS if a different type of RLC data PDU is needed for transport of RLC SDU segments.

##### 4.2.1.2.2	Transmitting UM RLC entity

When a transmitting UM RLC entity forms UMD PDUs from RLC SDUs, it shall:

-	when needed, segment the RLC SDUs so that the UMD PDUs fit within the total size of RLC PDU(s) indicated by lower layer at the particular transmission opportunity notified by lower layer;

- 	include relevant RLC headers in the UMD PDU.

*** End of change ***

*** Start of change ***

#### 4.2.1.3	AM RLC entity

##### 4.2.1.3.1	General

An AM RLC entity can be configured to deliver/receive RLC PDUs through the following logical channels:

- DL/UL DCCH or DL/UL DTCH.

Figure 4.2.1.3.1-1: Model of an acknowledged mode entity

An AM RLC entity delivers/receives the following RLC data PDUs:

- AMD PDU.

An AMD PDU contains either one complete RLC SDU or one RLC SDU segment.

Editor’s note: FFS if another type of AM RLC data PDU needs to be defined for segmented case, For now, it is assumed here that the same PDU format can be used for both unsegmented and segmented AM RLC data PDUs. This section will be updated if the assumption turns out to be incorrect.

An AM RLC entity delivers/receives the following RLC control PDU:

-	STATUS PDU.

##### 4.2.1.3.2	Transmitting side

When the transmitting side of an AM RLC entity forms AMD PDUs from RLC SDUs, it shall:

-	when needed, segment the RLC SDUs so that the AMD PDUs fit within the total size of RLC PDU(s) indicated by lower layer at the particular transmission opportunity notified by lower layer.

The transmitting side of an AM RLC entity supports retransmission of RLC SDUs or RLC SDU segments (ARQ):

-	if the RLC SDU or RLC SDU segment to be retransmitted does not fit within the total size of RLC PDU(s) indicated by lower layer at the particular transmission opportunity notified by lower layer, the AM RLC entity can segment the RLC SDU or re-segment the RLC SDU segments into RLC SDU segments.

-	the number of re-segmentation is not limited.

When the transmitting side of an AM RLC entity forms AMD PDUs from RLC SDUs or RLC SDU segments, it shall:

-	include relevant RLC headers in the AMD PDU.

*** End of change ***

*** Start of change ***

### 5.1.3	AM data transfer

#### 5.1.3.1	Transmit operations

##### 5.1.3.1.1	General

The transmitting side of an AM RLC entity shall prioritize transmission of RLC control PDUs over AMD PDUs. The transmitting side of an AM RLC entity shall prioritize retransmission of AMD PDUs containing previously transmitted RLC SDUs or RLC SDU segments over transmission of new AMD PDUs.

The transmitting side of an AM RLC entity shall maintain a transmitting window according to the state variable TX_Next_Ack as follows:

-	a SN falls within the transmitting window if TX_Next_Ack <= SN < TX_Next_Ack + AM_Window_Size;

-	a SN falls outside of the transmitting window otherwise.

The transmitting side of an AM RLC entity shall not deliver to lower layer any AMD PDU whose SN falls outside of the transmitting window.

When delivering an AMD PDU to lower layer, the transmitting side of an AM RLC entity shall:

-	set the SN of the AMD PDU to TX_Next;

-	if the AMD PDU contains a RLC SDU or a segment of RLC SDU that maps to the last byte of a RLC SDU:

-	increment TX_Next by one.

The transmitting side of an AM RLC entity can receive a positive acknowledgement (confirmation of successful reception by its peer AM RLC entity) for an AMD PDU by the following:

-	STATUS PDU from its peer AM RLC entity.

When receiving a positive acknowledgement for an AMD PDU with SN = x, the transmitting side of an AM RLC entity shall:

-	if positive acknowledgements have been received for all AMD PDU(s) associated with the transmitted RLC SDU:

-	send an indication to the upper layers of successful delivery of the RLC SDU;

-	set TX_Next_Ack equal to the SN of the RLC SDU with the smallest SN, whose SN falls within the range TX_Next_Ack <= SN <= TX_Next and for which all positive acknowledgments of associated AMD PDU(s) have not been received yet.

*** End of change ***

*** Start of change ***

## 5.3	SDU discard procedures

When indicated from upper layer (i.e. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity or the transmitting UM RLC entity shall discard the indicated RLC SDU if the RLC SDU or RLC SDU segment has not been mapped to a RLC data PDU and delivered to the lower layer yet.

*** End of change ***