3GPP TSG-RAN WG2 NR AH#3	Tdoc R2-1801631

Vancouver, Canada, 22nd – 26th January 2018

Title:	LS on Baseband Processing Capabilities

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN2

To:	RAN4, RAN1

Contact Person:

#### Name:	Naveen Palle

Tel. Number:	+1 224 715 6778

E-mail Address:	naveen [DOT] palle [AT] intel [DOT] com

# 1	Overall description

RAN2 would like to thank RAN1/RAN4 on their reply LS concerning the provided information regarding baseband processing capability (BPC). RAN2 has discussed the capability structure for BPC for NR and has reached agreements on certain aspects, while agreeing on getting RAN4/RAN1 views on certain aspects of BPC parameters. This LS aims to inform RAN4 and RAN1 about the reached agreements and request RAN4 and RAN1 input on the concerning BPC and other RF parameters.

## 1.1	Agreements on the BPC

RAN2 has agreed on the following in the BPC:

- Keep supportedBW-PerCC as per CC capability. (Value range to wait for RAN4)

- Keep modulationOrder as per CC capability

- Separate supported modulation for DL and UL.

## 1.2	Questions to RAN1/RAN4

RAN2 would like to ask RAN1/4 inputs for the following questions related to the remaining parameters in BPC:

- RAN2 understands that SCS support is separate for UL and DL and is dependent on the operating RF band, so does RAN4/1 view that the baseband processing capabilities in the BPC table depend on SCS used? If so, RAN2 has further questions as below:

Q1.1: Does RAN4 see a need to signal the SCS capability per CC in the BPC?

Q1.2: Does RAN4 see a need to signal the SCS capability separately for UL and DL in the BPC?

- What should be value ranges for supportedBW-PerCC which is reported per CC in BPC?

According to RP-172832, RAN2 is also tasked to introduce signalling support for channel bandwidths as the following:

  - Maximum channel bandwidth supported in each band for DL and UL separately and for each SCS that UE supports

  - UE shall support any Rel-15 channel bandwidth as defined in 38.101-1 v15.0.0 that is smaller than its UE supported maximum channel bandwidth

RAN2 plan to implement according to the RAN plenary agreement unless RAN4 indicates otherwise.

With regards to the above, RAN2 wold like to ask RAN4:

- How does RAN4 intend to define the carrier bandwidth and the number of aggregated intra-frequency carriers? Is that supposed to be done by the bandwidth class (as in LTE) or by other means?

- What is the relationship between the carrier bandwidth of aggregated carriers and the maximum channel bandwidth supported by the UE?

# 2	Actions

ACTION to RAN1/RAN4:

RAN2 respectfully requests that the RAN2 agreements be taken into consideration.

RAN2 respectfully asks inputs on Q1

ACTION to RAN4:

RAN2 respectfully asks inputs on Q2, Q3, Q4, Q5 and Q6.

# 3	Date of Next RAN2 Meetings:

RAN2#101	26 February - 2 March 2018	Athens, Greece