# 3GPP TSG-RAN WG2 Meeting NR Adhoc 1807	                                           R2-1809464

# Montreal, Canada, 2nd - 6th July 2018

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0187 | rev | 1 | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | To align the RAR window start timing with TS 38.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-06-19 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | The RAR window start timing is unclear in MAC spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In TS 38.213, the RAR monitoring for BFR procedure is started at slot n+4 for PRACH transmission n. “For PRACH transmission in slot n and according to antenna port quasi co-location parameters associated with periodic CSI-RS configuration or SS/PBCH block with index provided by higher layers [11, TS 38.321], the UE monitors PDCCH in a search space provided by higher layer parameter recoverySearchSpaceId for detection of a DCI format with CRC scrambled by C-RNTI starting from slot n+4 within a window configured by higher layer parameter BeamFailureRecoveryConfig. ” While in MAC TS 38.321, the RAR window start from first PDCCH occasion after the MSG 1 transmission. “start the ra-ResponseWindow configured in BeamFailureRecoveryConfig at the first PDCCH occasion as specified in TS 38.213 [6] from the end of the Random Access Preamble transmission” We think the RAR start timing should follow the RAN1 spec to avoid any misunderstanding between RAN1 and RAN2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Align the MAC spec with physical payer protcol. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.1.4	Random Access Response reception

Once the Random Access Preamble is transmitted and regardless of the possible occurrence of a measurement gap, the MAC entity shall:

1>	if the contention-free Random Access Preamble for beam failure recovery request was transmitted by the MAC entity:

2>	start the ra-ResponseWindow configured in BeamFailureRecoveryConfig at the first PDCCH occasion of the slot as specified in TS 38.213 [6] from the end of the Random Access Preamble transmission;

2>	monitor the PDCCH of the SpCell for response to beam failure recovery request identified by the C-RNTI while ra-ResponseWindow is running.

1>	else:

2>	start the ra-ResponseWindow configured in RACH-ConfigCommon at the first PDCCH occasion as specified in TS 38.213 [6] from the end of the Random Access Preamble transmission;

2>	monitor the PDCCH of the SpCell for Random Access Response(s) identified by the RA-RNTI while the ra-ResponseWindow is running.

1>	if notification of a reception of a PDCCH transmission is received from lower layers; and

1>	if PDCCH transmission is addressed to the C-RNTI; and

1>	if the contention-free Random Access Preamble for beam failure recovery request was transmitted by the MAC entity:

2>	consider the Random Access procedure successfully completed.