3GPP TSG-RAN WG2 AH-1807 	R2-1809522

# Montreal, Canada, July 2nd - July 6th 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.321 | CR | 1307 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Ci bitmap length determination in the Activation/Deactivation MAC CE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-06-15 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to the current MAC specification, Ci bitmap length is determined based on whether there is a Serving Cell configured with a ServCellIndex larger than 7, in which case four octets are used; otherwise, one octet is used. But for EN-DC and NR-NR DC, MN cannot know which servCellIndex is actually configured to UE by SN since both nodes configure their cells independently of each other. Hence it is not always possible for the MN or SN to determine a suitable Ci bitmap length in the SCell Activation/Deactivation MAC CE, being the same as UE expects according to the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that in case of EN-DC, the Ci bitmap length in the SCell Activation/Deactivation MAC CE is determined based on the ServCellIndex configuration in the MAC entity receiving the MAC CE. Impact analysis Impacted functionality: SCell Activation/Deactivation. Inter-operability: - If the network is implemented according to the CR and the UE is not, the Ci bitmap length in the SCell Activation/Deactivation MAC CE sent by network node may be different from what UE expected. - If the UE is implemented according to the CR and the network is not, the network is unable to determine which Ci bitmap length should be used in the SCell Activation/Deactivation MAC CE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The network is unable to detemine whether one octet or four octets should be used in the SCell Activation/Deactivation MAC CE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of change

6.1.3.8	Activation/Deactivation MAC Control Elements

The Activation/Deactivation MAC control element of one octet is identified by a MAC PDU subheader with LCID as specified in table 6.2.1-1. It has a fixed size and consists of a single octet containing seven C-fields and one R-field. The Activation/Deactivation MAC control element with one octet is defined as follows (figure 6.1.3.8-1).

The Activation/Deactivation MAC control element of four octets is identified by a MAC PDU subheader with LCID as specified in table 6.2.1-1. It has a fixed size and consists of a four octets containing 31 C-fields and one R-field. The Activation/Deactivation MAC control element of four octets is defined as follows (figure 6.1.3.8-2).

For the case with no serving cell with a ServCellIndex [8] larger than 7, Activation/Deactivation MAC control element of one octet is applied, otherwise Activation/Deactivation MAC control element of four octets is applied. In case EN-DC is configured, the above rule for selecting the MAC control element size only considers the ServCellIndex among serving cells configured in this MAC entity.

-	Ci: if there is an SCell configured with SCellIndex i as specified in [8], this field indicates the activation/deactivation status of the SCell with SCellIndex i, else the MAC entity shall ignore the Ci field. The Ci field is set to "1" to indicate that the SCell with SCellIndex i shall be activated. The Ci field is set to "0" to indicate that the SCell with SCellIndex i shall be deactivated;

-	R: Reserved bit, set to "0".

Figure 6.1.3.8-1: Activation/Deactivation MAC control element of one octet

Figure 6.1.3.8-2: Activation/Deactivation MAC control element of four octets

End of change