3GPP TSG-RAN WG2 AH-1807	R2-1809540

Montreal, Canada, 2nd – 6th July 2018	Resubmission of R2-1806729

Source:	CATT

Title:	Discuss on Access Category for RNAU

Agenda Item:	10.4.1.8.3

Document for:	Discussion and Decision

Introduction

Currently there is an FFS on whether a new access category shall be defined for RNAU:

FFS Whether to use access category 3 for MO-signalling or a standardised RAN specific access category.

In last CT WG1 meeting, a CR is agreed on the general principle to select the access category and the RRC establishment cause [1]. In this contribution, we give our proposal on RRC resume procedure accordingly.

Discussion

Currently there is an FFS on whether a new access category shall be defined for RNAU. Different options are raised in recent meetings as:

Option 1: reuse current access category “MO_sig”;

Option 2: introduce a new access category.

In [1], following description is added to TS 24.501:

When 5GMM requests the establishment of a NAS-signalling connection, the RRC establishment cause used by the UE shall be selected according to one or more determined access identities and the access category as specified in table 4.5.X.1. If the determined access category is an operator-specific access category and there is a standardized access category associated with the operator-specific access category as specified in subclause 4.5.3, the RRC establishment cause used by the UE is selected according to one or more determined access identities and the standardized access category associated with the operator-specific access category.

Table 4.5.X.1: Mapping table for access identities/access categories and RRC establishment cause

| Access identities | Access categories | RRC establishment cause is set to |
| --- | --- | --- |
| 0 | 0 (= MT_acc) | MT access |
|  | 1 (= delay tolerant) | FFS |
|  | 2 (= emergency) | Emergency call |
|  | 3 (= MO_sig) | MO signalling |
|  | 4 (= MO MMTel voice) | MO voice call |
|  | 5 (= MO MMTel video) | FFS |
|  | 6 (= MO SMS and SMSoIP) | FSS |
|  | 7 (= MO_data) | MO data |
| 1 | Any category | "High priority access" |
| 2 | Any category | "High priority access" |
| 11, 15 | Any category | "High priority access" |
| 12,13,14, | Any category | "High priority access" |
| NOTE: See subclause 4.5.2, table 4.5.2.1 for use of the access identities of 0, 1, 2, and 11-15. |  |  |

Editor's note:	It is FFS how to determine RRC establishment causes for the access category 1, 5, 6 and operator-defined access categories.

The table shown above is used for the RRC layer to select the RRC establishment cause. It is reasonable that the RRC layer shall also select the RRC resume cause according to a similar mechanism.

RAN WG2 has agreed that a new cause value shall be used for RNAU procedure. Therefore, if Option 1 is adopted for RNAU (i.e. “MO_sig” is used for RNAU), there will be such a case that two access attempts (e.g. one for RNAU and one for TAU) shall be mapped to different resume causes (the former to “RNAU” and the latter to “MO signalling”) although they share the same access identity (both are 0) and access category (both are “MO_sig”).

Proposal 1: If the access category “MO_sig” is used for RNAU, RAN2 should send an LS to CT1 to confirm that two access attempts can be mapped to different resume causes even though they share the same access identity and access category.

In addition, it is also specified in TS 24.501 that:

In order to determine the access category applicable for the access attempt, the NAS shall check the rules in table 4.5.2.2, and use the access category for which there is a match for barring check. If the access attempt matches more than one rule, the access category of the lowest rule number shall be selected.

Table 4.5.2.2: Mapping table for access categories

| Rule # | Type of access attempt | Requirements to be met | Access Category |
| --- | --- | --- | --- |
| 1 | Response to paging | Access attempt is for MT access | 0 (= MT_acc) |
| 2 | Emergency | UE is attempting access for an emergency session (NOTE 1, NOTE 2) | 2 (= emergency) |
| 3 | Access attempt for operator-defined access category | UE was provided with operator-defined access categories for the current PLMN, and access attempt is matching criteria of an operator-defined access category | 32-63 (= based on operator classification) |
| 4 | Access attempt for delay tolerant service | UE is configured for delay tolerant service, the PLMN is broadcasting one of the categories a, b or c, and the UE is a member of the broadcasted category in the selected PLMN or RPLMN/equivalent PLMN (NOTE 3) | 1 (= delay tolerant) |
| 5 | MO MMTel voice call | Access attempt is for MO MMTel voice call or for NAS signalling connection recovery during ongoing MO MMTel voice call (NOTE 2) | 4 (= MO MMTel voice) |
| 6 | MO MMTel video call | Access attempt is for MO MMTel video call or for NAS signalling connection recovery during ongoing MO MMTel video call (NOTE 2) | 5 (= MO MMTel video) |
| 7 | MO SMS over NAS or MO SMSoIP | Access attempt is for MO SMS or SMSoIP transfer or for NAS signalling connection recovery during ongoing MO SMS or SMSoIP transfer (NOTE 2) | 6 (= MO SMS and SMSoIP) |
| 8 | UE NAS initiated 5GMM specific procedures | Access attempt is for MO signalling | 3 (= MO_sig) |
| 9 | UE NAS initiated 5GMM connection management procedures or 5GMM NAS transport procedure | Access attempt is for MO data | 7 (= MO_data) |
| NOTE 1: This includes 5GMM specific procedures while the service is ongoing and 5GMM connection management procedures required to establish a PDU session with request type = "emergency" or to re-establish radio bearers for such a PDU session. NOTE 2: Access for the purpose of NAS signalling connection recovery during an ongoing service is mapped to the access category of the ongoing service in order to derive an RRC establishment cause, but barring checks will be skipped for this access attempt. NOTE 3: If the UE selects a new PLMN, then the selected PLMN is used to check the membership; otherwise the UE uses the RLPMN or a PLMN equivalent to the RPLMN. |  |  |  |

The table shown above is used for the NAS layer to select the access category. In this table, the access category “MO_sig” has a rule number of “8”, while “MO_data” has a rule number of “9”.

RAN WG2 has agreed that RNAU procedure may not take place immediately when triggered, so that NAS can check whether TAU procedure shall take place. This makes it possible that a NAS message categorised as “MO_data” arrives at the RRC layer when it is waiting for TAU checking. (It is also possible that UL user data of an existing session arrives at the AS layer, and the problem is similar if the access category “MO_data” is reused.)

If Option 1 is adopted for RNAU (i.e. “MO_sig” is used for RNAU), such a case may occur that one single access attempt matches two different access categories: one is “MO_data” caused by the NAS message, and the other is “MO_sig” caused by RNAU.

For this specific case, two different types of action are possible:

Option 1-1: to follow the rule given in TS 24.501, and thus to use the access category “MO_sig” for this access attempt (the resume cause shall be set to “RNAU” for consistency);

Option 1-2: to use the access category “MO_data” for this access attempt regardless of the rule given in TS 24.501.

Option 1-1 is compatible with the method that NAS shall also be responsible to select the access category for AS-triggered access attempts. However, it is very likely that the network will suspend the UE directly by MSG4, and thus the UE has to perform an extra resume procedure in order to transfer the NAS message.

Figure 1. Signalling flow if “MO_sig” is adopted for RNAU and the access category selection rule is followed.

Proposal 2: If the access category “MO_sig” is used for RNAU and the access category for any resume attempts are selected according to the same rule in TS 24.501, RAN2 should confirm the signalling flow shown in Figure 1 of this discussion.

In comparison, Option 1-2 provides a simpler solution:

Figure 2. AS selects the access category with the higher rule number as “MO_sig” is adopted for RNAU.

Proposal 3: If the access category “MO_sig” is used for RNAU and RAN2 wishes that NAS message cannot be blocked by a RNAU procedure to be performed, RAN2 should send an LS to CT1 to confirm that the RRC layer may select an access category regardless of the selection rule specified in TS 24.501.

If Option 2 is adopted for RNAU (i.e. a new access category is used for RNAU), the abovementioned issue can be resolved if the rule number of this new category is higher than the one of “MO_data”. However, other work groups such as CT WG1 should be informed of the new access category.

Proposal 4: If a new access category is used for RNAU, the rule number of this access category should be higher than the one of “MO_data”.

Proposal 5: If a new access category is used for RNAU, RAN2 should send an LS to CT1 to inform this decision.

In conclusion, Option 2 is much simpler than Option 1, and provides more flexibility to control different UE access attempts. Therefore, Option 1 should be adopted.

Proposal 6: A new access category shall be introduced for RNAU.

Proposal 7: RAN WG2 is kindly asked to agree with the draft LS provided in R2-1809541.

Conclusion

Proposal 1: If the access category “MO_sig” is used for RNAU, RAN2 should send an LS to CT1 to confirm that two access attempts can be mapped to different resume causes even though they share the same access identity and access category.

Proposal 2: If the access category “MO_sig” is used for RNAU and the access category for any resume attempts are selected according to the same rule in TS 24.501, RAN2 should confirm the signalling flow shown in Figure 1 of this discussion.

Proposal 3: If the access category “MO_sig” is used for RNAU and RAN2 wishes that NAS message cannot be blocked by a RNAU procedure to be performed, RAN2 should send an LS to CT1 to confirm that the RRC layer may select an access category regardless of the selection rule specified in TS 24.501.

Proposal 4: If a new access category is used for RNAU, the rule number of this access category should be higher than the one of “MO_data”.

Proposal 5: If a new access category is used for RNAU, RAN2 should send an LS to CT1 to inform this decision.

Proposal 6: A new access category shall be introduced for RNAU.

Proposal 7: RAN WG2 is kindly asked to agree with the draft LS provided in R2-1809541.

Reference

[1] C1-182847, RRC establishment cause, vivo, LG Electronics, Huawei, HiSilicon, Ericsson

[2] R2-1809541, LS on new access category for RNAU, CATT