3GPP TSG-RAN2 NR AH#0218	R2-1809552

Montreal, Canada, July 2-6, 2018

Agenda item:         10.4.1.4.1

Source:		Mediatek Inc.

Title:  	   Remove the concept of ‘intra-/inter-frequency’ from RRM measurement in CONNECTED

Document for:	Discussion

Introduction

In RAN2#102 meeting, it was discussed whether to remove the concept of intra-frequency/inter-frequency from RRC specification.  No agreement was achieved during the on-line discussion due to the impact in IDLE/INACTIVE mode for cell selection/reselection. This contribution continues to discuss whether it would make sense to remove the term inter and intra frequency from the connected mode mobility parts of RRC specification.

Discussion

Intra-/Inter-frequency in CONNECTED

In LTE, the concept of intra-frequency and inter-frequency is simple and uniform for both mobility and measurement. Generally, the frequencies shares the same ARFCN with equal or less bandwidth than serving frequency, they are considered as intra-frequency; otherwise, they are considered as inter-frequency. When perform intra-frequency measurement, no measurement gap is needed; while when perform inter-frequency measurement, measurement gap needs to be configured. The purpose to differentiate intra-/inter-frequency measurement is for MG configuration.

However, in NR, the concept is largely change. The purpose of intra-/inter-frequency to determine the necessity of measurement gap no longer exists, since MG is always required even for intra-frequency measurement. From RAN2 aspect, the meaning to define intra-/inter-frequency is quite obscure. If we keep the concept of intra-/inter-frequency measurement, lots of confusion needs to be solved.

Taking Figure 1 as example, if MO1 is associated to serving cell1 and considered as serving frequency; MO2 and MO3 are considered as non-serving frequencies. For SSB-based RRM measurement, MO2 and MO3 are considered as inter-frequency measurement. However, for CSI-RS based RRM, it’s not clear whether MO2 and MO3 are considered as intra-/inter-frequency even if they have the same numerology ad MO1. In our understanding, the purpose to differentiate intra-/inter-frequency measurement is used by RAN4 to define different measurement requirement. From RAN2 aspect, what we need to differentiate is serving cell/non-serving cell for event trigger. RAN2 doesn’t need differentiate intra-/inter-frequency.

Figure 1 MOs vs. serving cells

Observation 1: The purpose to differentiate intra-/inter-frequency is used by RAN4 to define different measurement requirement.

Observation 2: RAN2 only need to differentiate serving cell and neighboring cell for event triggering for RRM measurement.

In RAN2#102 meeting, RAN2 agreed to clarify the spec that we will use the approach to specific fields within the signalling (e.g. MO) instead of referring to the frequency, since it’s unclear what ‘a frequency’ is. So any reference to NR "carrier frequency" (or "frequency") is removed from the measurement-related procedures.

Observation 3: The term of NR ‘carrier frequency’ or ‘frequency’ has already been removed from the RRM measurement procedure.

Considering the term of ‘carrier frequency’ or ‘frequency’ has gone from current NR RRC specification, the concept of intra-/inter-frequency has no meaning for RRM measurement from RAN2 aspect, but inspires much confusions. It is suggested that the term ‘intra-/inter- frequency’ is removed from the connected mode mobility parts of RRC specification.

Proposal 1: Remove the term ’intra-/inter- frequency’ from the connected mode mobility parts of RRC specification.

Intra-/Inter-frequency in IDLE/INACTIVE

In RAN2#102 meeting, the relationship between SSB and cell is clarified. RAN2 confirmed that cell (re)selection only searches for cells on the sync raster.  For UE in IDLE/INACTIVE mode, cell (re)selection is always based on the SSB on sync raster. The concept of intra-/inter-frequency in IDLE/INACTIVE is actually referring to the SSB and has nothing to do with CSI-RS. However, the concept of intra-/inter-frequency in CONNECTED considers both SSB and CSI-RS, which is not exactly aligned with the definition in IDLE/INACTIVE.

Observation 4: Intra-/inter-frequency in IDLE/INACTIVE for cell (re)selection only considers SSB.

The definition of intra-/inter-frequency based on SSB is clearer than the definition for CSI-RS. Similar as LTE, the frequency is defined based on the center frequency of SSB. There is no problem to keep the term of intra-/inter-frequency for cell (re)selection in IDLE/INACTIVE, although the term ‘intra-frequency’ and ‘inter-frequency’ are equivalent to serving frequency and neighboring frequency respectively.

Proposal 2:  Keep the terms ‘intra-frequency’ and ‘inter-frequency’ in mobility related procedure in IDLE/INACTIVE and clarify that ‘intra-/intra-frequency’ refers to SSB only.

Conclusion

Based on the following observations:

Observation 1: The purpose to differentiate intra-/inter-frequency is used by RAN4 to define different measurement requirement.

Observation 2: RAN2 only need to differentiate serving cell and neighboring cell for event triggering for RRM measurement.

Observation 3: The term of NR ‘carrier frequency’ or ‘frequency’ has already been removed from the RRM measurement procedure.

Observation 4: Intra-/inter-frequency in IDLE/INACTIVE for cell (re)selection only considers SSB.

We propose:

Proposal 1: Remove the term ’intra-/inter- frequency’ from the connected mode mobility parts of RRC specification.

Proposal 2:  Keep the terms ‘intra-frequency’ and ‘inter-frequency’ in mobility related procedure in IDLE/INACTIVE and clarify that ‘intra-/intra-frequency’ refers to SSB only.

The CR is provided in the appendix.

Appendix

| CHANGE REQUEST |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38.331 | CR | CRNum | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remove ‘intra-frequency’ and ‘inter-frequency’ from RRM measurement procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-06-22 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Since it’s unclear what ‘a frequency’ is, any reference to NR "carrier frequency" (or "frequency") has been removed from the measurement-related procedures. The purpose to differentiate intra-/inter-frequency is used by RAN4 to define different measurement requirement. From RAN2 aspect, the term ‘intra-frequency’ and ‘inter-frequency’ for mobility related procedure in CONNECTED is meaningless. RAN2 only need to differentiate serving frequency and neighboring frequency for event triggering for RRM measurement. The term of ‘intra-frequency’ and ‘inter-frequency’ should be removed from the RRM measurement procedure. Furthermore, for UE in IDLE/INACTIVE, Intra-/inter-frequency for cell (re)selection only refers to SSB. There is no problem to keep the terms if it is clarified that intra-/inter-frequency refers to the centre frequency of SSBs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove ‘intra-frequency and inter-frequency’ from the MO description in 5.5.1; - Clarify that intra-frequency and inter-frequency refers to the centre frequency of SS/PBCH blocks for cell re-selection information in SIB2 and SIB3 in 6.3.1; - Change ‘intra-frequency’ and ‘inter-frequency’ to ‘serving frequency’ and ‘neighbouring frequency’ in the IE description for MeasConfig, MeasObjectNR and MeasResults in 6.3.2; - Change ‘intra-frequency’ and ‘inter-frequency’ to ‘serving frequency’ and ‘neighbouring frequency’ in the variable description for VarMeasConfig; |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear definition may result in unpredictable UE behaviour. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.1, 6.3.1, 6.3.2, 7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 5.5	Measurements

### 5.5.1	Introduction

The network may configure an RRC_CONNECTED UE to perform measurements and report them in accordance with the measurement configuration. The measurement configuration is provided by means of dedicated signalling i.e. using the RRCReconfiguration.

The network may configure the UE to perform the following types of measurements:

-	NR measurements;

-	Inter-RAT measurements of E-UTRA frequencies.

The network may configure the UE to report the following measurement information based on SS/PBCH block(s):

-	Measurement results per SS/PBCH block;

-	Measurement results per cell based on SS/PBCH block(s);

-	SS/PBCH block(s) indexes.

The network may configure the UE to report the following measurement information based on CSI-RS resources:

-	Measurement results per CSI-RS resource;

-	Measurement results per cell based on CSI-RS resource(s);

-	CSI-RS resource measurement identifiers.

The measurement configuration includes the following parameters:

1.	Measurement objects: A list of objects on which the UE shall perform the measurements.

-	A measurement object indicates the frequency/time location and subcarrier spacing of reference signals to be measured. Associated with this measurement object, the network may configure a list of cell specific offsets, a list of 'blacklisted' cells and a list of 'whitelisted' cells. Blacklisted cells are not applicable in event evaluation or measurement reporting. Whitelisted cells are the only ones applicable in event evaluation or measurement reporting.

- 	The measObjectId  of the MO which corresponds to each serving cell is indicated by servingCellMO within the serving cell configuration.

-	For inter-RAT E-UTRA measurements a measurement object is a single EUTRA carrier frequency. Associated with this E-UTRA carrier frequency, the network can configure a list of cell specific offsets, a list of 'blacklisted' cells and a list of 'whitelisted' cells. Blacklisted cells are not applicable in event evaluation or measurement reporting. Whitelisted cells are the only ones applicable in event evaluation or measurement reporting.

2.	Reporting configurations: A list of reporting configurations where there can be one or multiple reporting configurations per measurement object. Each reporting configuration consists of the following:

-	Reporting criterion: The criterion that triggers the UE to send a measurement report. This can either be periodical or a single event description;.

-	RS type: The RS that the UE uses for beam and cell measurement results (SS/PBCH block or CSI-RS).

-	Reporting format: The quantities per cell and per beam that the UE includes in the measurement report (e.g. RSRP) and other associated information such as the maximum number of cells and the maximum number beams per cell to report.

3.	Measurement identities: A list of measurement identities where each measurement identity links one measurement object with one reporting configuration. By configuring multiple measurement identities, it is possible to link more than one measurement object to the same reporting configuration, as well as to link more than one reporting configuration to the same measurement object. The measurement identity is also included in the measurement report that triggered the reporting, serving as a reference to the network.

4.	Quantity configurations: The quantity configuration defines the measurement filtering configuration used for all event evaluation and related reporting of that measurement type. For NR measurements, the network may configure up to 2 quantity configurations with a reference in the NR measurement object to the configuration that is to be used. In each configuration, different filter coefficients can be configured for different measurement quantities, for different RS types, and for measurements per cell and per beam.

5.	Measurement gaps: Periods that the UE may use to perform measurements, i.e. no (UL, DL) transmissions are scheduled.

A UE in RRC_CONNECTED maintains a measurement object list, a reporting configuration list, and a measurement identities list according to signalling and procedures in this specification. The measurement object list possibly includes NR intra-frequency object(s), NR inter-frequency object(s) and inter-RAT objects. Similarly, the reporting configuration list includes NR and inter-RAT reporting configurations. Any measurement object can be linked to any reporting configuration of the same RAT type. Some reporting configurations may not be linked to a measurement object. Likewise, some measurement objects may not be linked to a reporting configuration.

The measurement procedures distinguish the following types of cells:

1.	The NR serving cell(s) - these are the SpCell and one or more SCells.

2.	Listed cells - these are cells listed within the measurement object(s).

3.	Detected cells - these are cells that are not listed within the measurement object(s) but are detected by the UE on the SSB frequency(ies) and subcarrier spacing(s) indicated by the measurement object(s).

For NR measurement object(s), the UE measures and reports on the serving cell(s), listed cells and/or detected cells.

Whenever the procedural specification, other than contained in sub-clause 5.5.2, refers to a field it concerns a field included in the VarMeasConfig unless explicitly stated otherwise i.e. only the measurement configuration procedure covers the direct UE action related to the received measConfig.

### 6.3.1	System information blocks

#### –	SIB2

SIB2 contains cell re-selection information common for intra-frequency, inter-frequency and/ or inter-RAT cell re-selection (i.e. applicable for more than one type of cell re-selection but not necessarily all) as well as intra-frequency cell re-selection information other than neighbouring cell related. Intra-frequency and inter-frequency refers to the centre frequency of SS/PBCH block(s).

#### –	SIB3

SIB3 contains neighbouring cell related information relevant only for intra-frequency cell re-selection. The IE includes cells with specific re-selection parameters as well as blacklisted cells. Intra-frequency refers to the centre frequency of SS/PBCH block(s).

- 6.3.2	Radio resource control information elements

#### –	MeasConfig

The IE MeasConfig specifies measurements to be performed by the UE, and covers serving frequency, neighbouring frequency and inter-RAT mobility as well as configuration of measurement gaps.

#### –	MeasObjectNR

The IE MeasObjectNR specifies information applicable for SS/PBCH block(s) based measurements or CSI-RS based measurements.

#### –	MeasResults

The IE MeasResults covers measured results for serving frequency, neighbouring frequency, and inter-RAT mobility.

- 7.4	UE variables

#### –	VarMeasConfig

The UE variable VarMeasConfig includes the accumulated configuration of the measurements to be performed by the UE, covering serving frequency, neighbouring frequency and inter-RAT mobility related measurements.