3GPP TSG-RAN WG2 NR AH1807 Meeting	R2-1809603

# Montreal, Canada, 2nd - 6th July 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | CRNum | rev | - | Current version: | 15.1.0 (ASN.1 SA review baseline) |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.331 for cell reselection while T319/T302 are running |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Interdigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | n.a. (draft CR to be merged into rapporteur's CR when agreed) |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT_core |  |  |  |  |  |  | Date: |  | 2018-07-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | At RAN2#102, the following agreement was taken: => If Resume is triggered by AS and NAS triggered event, and cell reselection happens whilst either of the wait timer or T300x are running rely on NAS recovery In the current version of the specification, if cell reselection occurs during T302, T302 is stopped and pending RNAU’s are sent in the new cell. If cell reselection occurs while T319 is running, the UE remains in RRC_INACTIVE. In both cases, the UE should rely on NAS recovery as per agreement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In RRC_IDLE, when T302 expires, UE informs upper layers of the failure. - In RRC_INACTIVE, if cell reselection occurs when either T319 or T302 are running, the UE moves to RRC_IDLE and informs the upper layers of the failure to resume the connection so NAS recovery can be triggered. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Handling of cell reselection while T319 or T302 is running are not as per agreements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.13.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

START OF FIRST CHANGE

#### 5.3.13.6	Cell re-selection while T319 or T302 is running

Editor’s Note: FFS  Whether cell reselection actions need to be defined for other timers e.g. access control timers equivalent to T303, T305, T306 and T308 in LTE).

The UE shall:

1>	if cell reselection occurs while T319 or T302 is running:

2>	stop timer T319 or T302, if running;

2> if the UE is in RRC_INACTIVE:

3>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause RRC Resume failure;

2> else:

3> perform the actions upon T302 expiry or stop as specified in 5.3.13.8a;

END OF FIRST CHANGE