3GPP TSG-RAN WG2 NR AH1807 Meeting	R2-1809883

Montreal, Canada, 2nd - 6th July 2018

Source: 	vivo

Title:  	Text proposals for the evaluation results of the RLM for NR-U

Agenda Item:	11.2

Document for:	Discussion and Decision

# Introduction

This contribution is to prvide the text proposals for TR 38.889, capturing the evaluation results from “R2-1809882 Evaluation of the RLM for NR-U”.

# Text Proposal

---------------------start of change--------------------------------------------------------------------------

# 2	References

---omitted---

[xx] R2-1809882 Evaluation of the RLM for NR-U, vivo, RAN2#NR AH1807, Jul 2018

## 8.3	Evaluation results

### 8.1.4	RLM evaluation [xx]

This simulation focuses on the dense-urban (macro-layer) deployment scenario, in which all macro cells are on the same frequency layer. The number of unique RLM-RS(s) using different set of resources per BWP is set to X. For below 3 GHz, X≤2. For above 3 GHz and below 6 GHz, X≤6. The results of RLF probability are shown in the following figure.

Figure x. LBT failure rate vs. RLF rate

According to the figure illustrated above, the LBT failure leads to a significant increase in the RLF rate, compared with the frequency without LBT failure. For example, for the cell edge users, the RLF rate of 1 RLM RS with 10% LBT failure rate is almost three times of the RLF rate of 1 RLM RSs without LBT failure. Furthermore the LBT failure could cause more impacts on the RLF for the cell center uses. For example, for the cell center users, the RLF rate of 1 RLM RS with 10% LBT failure rate is about four times of the RLF rate of 1 RLM RSs without LBT failure. By increasing the number of reference signals used for RLM, the RLF rate is reduced. However when the LBT falure rate increases, increasing the number of RLM reference signals may not bring sufficient reduction on the RLF rate. For example, for the cell edge users, the RLF rate of 6 RLM RSs with 30% RLF rate is about 30% more than the the RLF rate of 1 RLM RSs without LBT failure.

---------------------next change------------------------------------------------------------------------------

# Annex A:
Evaluation methodology

---omitted---

## A.x	Evaluation assumptions for RLM

The following assumptions are used for the evaluation of radio link monitoring:

- : The probability of indicating OOS to higher layers per indication period, which can calculated according to the following equation,

: The probability of not indicating any indication to higher layers per indication period, which can calculated according to the following equation,

- : The probability of indicating OOS to higher layers per indication period, which can calculated according to the following equation,

- : The probability of detecting RLF by UE, which can calculated according to the following equation,

- : LBT failure probability, which can also be considered as the probability that the RLM-RS(s) can not be successfully transmitted by the gNB.

Table 1. System parameters of system-level simultiaon

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | 4 GHz |
| Mode | DL only |
| Bandwidth | 20 MHz |
| Subcarrier Spacing | 15 kHz |
| Channel Model | UMa in TR 38.900 |
| TXRU mapping to antenna elements | One TXRU per panel per polarization |
| TXRU mapping weights | 2D TXRU virtualization weights for each panel is the Kronecker product between vertical and horizontal weight vectors taken from DFT. |
| Beam selection | Based on the criteria of maximizing receive power after beamforming. |
| ISD | 200 m |
| BS Tx power | 33 dBm |
| BS Antenna Configuration | (Mg, Ng, M, N, P) = (1,1,2,1,2) (dH, dV) = (0.5, 0.5)λ |
| BS array orientation | azimuth 0 degree; mechanic downtilt: 0 degree |
| UE Configuration | (Mg, Ng, M, N, P,) = (1, 1, 1, 1, 2); (dV,dH) = (0, 0)λ. The polarization angles are 0 and 90 |
| UE array orientation | ΩUT,uniformly distributed on [0,360] degree, ΩUT,= 0 degree, ΩUT, = 0 degree |
| BS antenna pattern | See Table A.2.1-6 in TR 38.802 |
| UE antenna pattern | Omnidirectional |
| BS antenna height | 25m |
| UE antenna height | Same as 3D-UMa in TR36.873 |
| UE receiver noise figure | 10dB |
| UE distribution | 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h 10 users per TRP |

Table 2. System parameters of link-level simultiaon

| Attributes | Values or assumptions |
| --- | --- |
| Channel mode | TDL-A |
| Channel coding type | Agreed polar coding |
| Modulation | QPSK |
| Code rate | 1/3 |
| The number of RBs | 50 |
| Target BLER | 10-1, 2*10-2 |
| TB-CRC type | CRC-16, CRC-24 |
| Simulation frames | 10000 |
| N310 | 4 |
| N311 | 4 |
| T310 | 200ms |
| RLM indication period | 10ms |

----------------------end of change--------------------------------------------------------------------------