3GPP TSG-RAN WG2 Meeting NR Adhoc 1807	R2-1809913

Montreal, Canada, 2nd - 6th July 2018

Agenda item:	10.4.1.6.8

Source:	Nokia, Nokia Shanghai Bell

Title:	SIB1 transmission requirement

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

In this paper we discuss how to correct in 38.331 SIB1 is not always transmitted and corresponding UE behaviour.

# 2	SIB1 transmission

Currenlty in system information introduction (5.2.1) it is written that

-	the SIB1 is transmitted on the DL-SCH with variable periodicity as specified in [TS 38.213, Section 13] and repetitions are made within 160ms. The default periodicity of SIB1 is 20ms but the actual periodicity is up to network implementation. For pattern 1, SIB1 transmission period is 20ms. For pattern 2/3, SIB1 transmission period is the same as the SSB period [13]. SIB1 includes information regarding the availability and scheduling (e.g. mapping of SIBs to SI message, periodicity, SI-window size) of other SIBs with an indication whether one or more SIBs are only provided on-demand and, in that case, the configuration needed by the UE to perform the SI request. SIB1 is cell-specific SIB;

But SIB1 is not always transmitted though. Thus we propose to correct this as well as the earlier bullet about SIB1 scheduling information:

-	the MIB is always transmitted on the BCH with a periodicity of 80 ms and repetitions made within 80 ms [17, Section 7.1] and it may also include parameters that are needed to acquire SIB1 from the cell;

-	the SIB1 may be transmitted on the DL-SCH with variable periodicity as specified in [TS 38.213, Section 13] and repetitions are made within 160ms. The default periodicity of SIB1 is 20ms but the actual periodicity is up to network implementation. For pattern 1, SIB1 transmission period is 20ms. For pattern 2/3, SIB1 transmission period is the same as the SSB period [13]. SIB1 includes information regarding the availability and scheduling (e.g. mapping of SIBs to SI message, periodicity, SI-window size) of other SIBs with an indication whether one or more SIBs are only provided on-demand and, in that case, the configuration needed by the UE to perform the SI request. SIB1 is cell-specific SIB;

Additionally in the MIB reception procedural text seems to assume SIB1 is always transmitted

##### 5.2.2.4.1	Actions upon reception of the MIB

Upon receiving the MIB the UE shall:

1>	store the acquired MIB;

1>	if the UE is in RRC_IDLE or in RRC_INACTIVE or if the UE is in RRC_CONNECTED while T311 is running: [FFS]

2> if the cellBarred in the acquired MIB is set to barred:

3>	consider the cell as barred in accordance with TS 38.304 [FFS];

3>	if intraFreqReselection is set to notAllowed:

4> consider cell re-selection to other cells on the same frequency as the barred cell as not allowed, as specified in TS 38.304 [X].

3>	else:

4> consider cell re-selection to other cells on the same frequency as the barred cell as allowed, as specified in TS 38.304 [X].

2>	else;

3>	apply the received pdcch-ConfigSIB1, subCarrierSpacingCommon, ssb-SubcarrierOffset and dmrs-TypeA-Position. And acquire SIB1, if ssb-SubcarrierOffset indicates SIB1  to be transmitted in the cell.

# 5	Conclusion

In this paper we proposed how to capture details of SIB1 transmission and the fact that SIB1 is not always transmitted but optional (NSA).

Proposal: Agree on the changes as provided in the Annex Text Proposal

# Annex TP

### 5.2.1	Introduction

System Information (SI) is divided into the MIB and a number of SIBs where:

-	the MIB is always transmitted on the BCH with a periodicity of 80 ms and repetitions made within 80 ms [17, Section 7.1] and it may include parameters that are needed to acquire SIB1 from the cell;

-	the SIB1 may be transmitted on the DL-SCH with variable periodicity as specified in [TS 38.213, Section 13] and repetitions are made within 160ms. The default periodicity of SIB1 is 20ms but the actual periodicity is up to network implementation. For pattern 1, SIB1 transmission period is 20ms. For pattern 2/3, SIB1 transmission period is the same as the SSB period [13]. SIB1 includes information regarding the availability and scheduling (e.g. mapping of SIBs to SI message, periodicity, SI-window size) of other SIBs with an indication whether one or more SIBs are only provided on-demand and, in that case, the configuration needed by the UE to perform the SI request. SIB1 is cell-specific SIB;

-	SIBs other than SIB1 are carried in SystemInformation (SI) messages, which are transmitted on the DL-SCH. Only SIBs having the same periodicity can be mapped to the same SI message. Each SI message is transmitted within periodically occurring time domain windows (referred to as SI-windows with same length for all SI messages). Any SIB except SIB1 can be configured to be cell specific or area specific. The cell specific SIB is applicable only within a cell that provides the SIB while the area specific SIB is applicable within an area referred to as SI area and identified by systemInformationAreaID;

-	For PCell, NR-RAN may provide the required SI by dedicated signalling if the SI is not broadcast in the UE’s active BWP.

-	For PSCell and SCells, NR-RAN provides the required SI by dedicated signalling. Nevertheless, the UE shall acquire MIB of the PSCell to get SFN timing of the SCG (which may be different from MCG). Upon change of relevant SI for SCell, RAN releases and adds the concerned SCell. For PSCell, SI can only be changed with Reconfiguration with Sync.

##### --------------------------------------------NEXT CHANGES--------------------------------------------

##### 5.2.2.4.1	Actions upon reception of the MIB

Upon receiving the MIB the UE shall:

1>	store the acquired MIB;

1>	if the UE is in RRC_IDLE or in RRC_INACTIVE or if the UE is in RRC_CONNECTED while T311 is running: [FFS]

2> if the cellBarred in the acquired MIB is set to barred:

3>	consider the cell as barred in accordance with TS 38.304 [FFS];

3>	if intraFreqReselection is set to notAllowed:

4> consider cell re-selection to other cells on the same frequency as the barred cell as not allowed, as specified in TS 38.304 [X].

3>	else:

4> consider cell re-selection to other cells on the same frequency as the barred cell as allowed, as specified in TS 38.304 [X].

2>	else;

3>	apply the received pdcch-ConfigSIB1, subCarrierSpacingCommon, ssb-SubcarrierOffset and dmrs-TypeA-Position. And acquire SIB1, if indicated to be transmitted by ssb-SubcarrierOffset.

##### --------------------------------------------NEXT CHANGES--------------------------------------------