3GPP TSG-RAN WG2 AH 1807	Tdoc R2-1809952

Montreal, Canada, 2nd – 6th July 2018

Agenda Item:	10.2.6

Source:	Ericsson

Title:	Inter-node RRC messages for NE-DC

Document for:	Discussion, Decision

# Introduction

In this contribution, we discuss the introduction of inter-node RRC messages for NE-DC by providing also in the Appendix section an example on how they should be specified in NR RRC (i.e., 3GPP TS 38.331).

# Discussion

One of the remaining issues regarding NE-DC and NGEN-DC is how inter-node RRC messages (i.e., sent either across the X2-, Xn- or the NG-interface) will be specified and handled in order to perform procedures such as SN addition/modification/release/change or inter-MN handover. In the RAN2 AH1801 meeting, the outcome from the email discussion [100#31][NR] [1] has been that inter-node RRC messages for EN-DC will be specified in the NR RRC (i.e., TS 38.331).

Agreements:

SCGConfigInfo (specified in NR RRC)

Candidate cell information

1:	SCGConfigInfo will include 2 fields, one for the SN configured measurements and one for the MN configured measurements.

FFS Whether these are containers and the RRC encoding used

2:	Support transfer of CSI RS based results by the field carrying results of SN configured measurements.

3:	Introduce support for reporting of SINR results in both fields field

UL duplication for SRB1/ 2

4:	Do not introduce inter-node signalling regarding UL duplication for SRB 1/ 2 (at least for now)

SCGConfig (specified in NR RRC)

Full configuration

5:	In case target SN triggers fullConfig, MN generates the drb-toReleaseList towards the UE (based upon a fullConfig indication provided by SN)

6:	 X2 should be used for the indication by SN that fullConfig is used

Coordination of band combinations

7	Clarify that all allowedBCs indicated by MN include the same LTE BC

FFS whether SN should always indicate selected NR BC to MN (no progress)

8:	When SN request a change of the allowed EN-BCs (re-negotiation), it signals a single EN-BC that indicates the requested NR BC that SN would like to configure.

HandoverPreparationInfo (specified in LTE RRC)

Re-establishment information

9:	Keep the re-establishment information in the RRC INM

10:	Include the same information structure as used on Uu, at least for now. Can discuss if we should facilitate migration to transferring complete SIBs (i.e. making field optional)

According to this, in the RAN2 #101 meeting [2] a preliminary TP on the inter-node RRN messages in TS 38.331 was agreed and merged into the NR RRC specification.

Inter-node RRC messages for EN-DC are specified in NR RRC (i.e., TS 38.331).

For NGEN-DC, it is straightforward to have EN-DC as a baseline and reuse the internode messages already defined for EN-DC. When it comes to NE-DC, preferred solution would be to specify the related inter-node RRC messages in the NR RRC specification and enhance/extend the messages that are already in place. In this way, we can reduce the standardization effort and have a part of the specification that is general for all the MR-DC options.

Inter-node messages for NE-DC and NGEN-DC are specified in the NR RRC specification (i.e., TS 38.331)

Now, having a look at the current 3GPP TS 38.331, it is possible to notice that the CG-Config and CG-ConfigInfo are mainly related to EN-DC. As an example, the scg-CellGroupConfig field includes lower layers related NR (since it is linked to RRCReconfiguration message) whereas for the NE-DC case those should come from LTE (and thus from RRCConnectionReconfiguration message). Another example could refer to the measurements configuration for NE-DC [3][4]. In this case, the fields candidateCellInfoListMN and candidateCellInforListSN refer to an information element (i.e., MeasResultList2NR) that contain measurements results encoded only in NR format. For this reason, it is evident that the CG-Config, CG-ConfigInfo, and HandoverPreparationInformation should be enhanced/extended for handling the NE-DC procedures (e.g., SN addition/modification/release/change or inter-MN handover).

NGEN-DC inter-node RRC messages take EN-DC as a baseline.

Existing inter-node RRC messages in NR RRC (i.e., TS 38.331) should be extended for the NE-DC option.

RAN2 should consider to include the related TP in TS 34.370.

# Text Proposal to 37.340

START OF CHANGES

## 7.9	Inter-node Resource Coordination

For EN-DC operation, MN and SN may coordinate their UL and DL radio resources in semi-static manner via UE associated signalling.

In MR-DC and NN-DC, the MN and SN may exchange RRC messages that are sent either across the X2-, Xn- or the NG-interface, either to or from the gNB/eNB, i.e. a single 'logical channel' is used for all RRC messages transferred across network nodes. The inter-node RRC message for MR-DC and NN-DC are defined in 38.331 [4].

END OF CHANGES

# Conclusion

In section 2 we made the following observations:

Observation 1	Inter-node RRC messages for EN-DC are specified in NR RRC (i.e., TS 38.331).

Based on the discussion in section 2 we propose the following:

- Inter-node messages for NE-DC and NGEN-DC are specified in the NR RRC specification (i.e., TS 38.331)

NGEN-DC inter-node RRC messages take EN-DC as a baseline.

Existing inter-node RRC messages in NR RRC (i.e., TS 38.331) should be extended for the NE-DC option.

RAN 2 should consider to include the related TP in TS 34.370.

# References

Chairman notes, RAN2 #AH1801, January 2018

Chairman notes, RAN2 #101, February 2018

R2-1809951, NE-DC measurement principles, Ericsson, RAN2 #AH1807, July 2018.

R2-1807051, Open issues on SCG measurement results for NE-DC and NGEN-DC, Ericsson, RAN2 #102, May 2018.

# Appendix (Example TP for 38.331)

### 11.2.2	Message definitions

#### –	HandoverPreparationInformation

Editor’s Note: Targeted for completion in Sept 2018.

This message is used to transfer the NR RRC information used by the target gNB/eNB during handover preparation, including UE capability information.

Direction: source gNB/source RAN to target gNB/eNB.

HandoverPreparationInformation message

-- ASN1START

-- TAG-HANDOVER-PREPARATION-INFORMATION-START

HandoverPreparationInformation ::=	SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE{

handoverPreparationInformation		HandoverPreparationInformation-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

HandoverPreparationInformation-IEs ::= SEQUENCE {

ue-CapabilityRAT-List				UE-CapabilityRAT-ContainerList,

sourceConfig						OCTET STRING (CONTAINING RRCReconfiguration),

rrm-Config							RRM-Config				OPTIONAL,

as-Context							AS-Context				OPTIONAL,

nonCriticalExtension				HandoverPreparationInformation-vxx-IEs				OPTIONAL

}

HandoverPreparationInformation-vxx-IEs ::= SEQUENCE {

sourceConfigEUTRA					OCTET STRING (CONTAINING RRCConnectionReconfiguration-r1250-IEs)	OPTIONAL,	-- Need N

nonCriticalExtension				SEQUENCE {}				OPTIONAL

}

AS-Context ::=							SEQUENCE {

reestablishmentInfo						SEQUENCE {

sourcePhysCellId					PhysCellId,

targetCellShortMAC-I				ShortMAC-I,

additionalReestabInfoList			ReestabNCellInfoList					OPTIONAL

}																				OPTIONAL,

-- FFS Whether to change e.g. move all re-establishment info to Xx

configRestrictInfo					ConfigRestrictInfoSCG						OPTIONAL,

...

[[ reestablishmentInfoEUTRA					OCTET STRING						OPTIONAL,	-- Need N

]]

}

ReestabNCellInfoList ::=		SEQUENCE ( SIZE (1..maxCellPrep) ) OF ReestabNCellInfo

ReestabNCellInfo::=	SEQUENCE{

cellIdentity							CellIdentity,

key-gNodeB-Star							BIT STRING (SIZE (256)),

shortMAC-I								ShortMAC-I

}

RRM-Config ::=				SEQUENCE {

ue-InactiveTime				ENUMERATED {

s1, s2, s3, s5, s7, s10, s15, s20,

s25, s30, s40, s50, min1, min1s20c, min1s40,

min2, min2s30, min3, min3s30, min4, min5, min6,

min7, min8, min9, min10, min12, min14, min17, min20,

min24, min28, min33, min38, min44, min50, hr1,

hr1min30, hr2, hr2min30, hr3, hr3min30, hr4, hr5, hr6,

hr8, hr10, hr13, hr16, hr20, day1, day1hr12, day2,

day2hr12, day3, day4, day5, day7, day10, day14, day19,

day24, day30, dayMoreThan30}		OPTIONAL ,

candidateCellInfoList		MeasResultList2NR		OPTIONAL,

...

[[ candidateCellInfoList-EUTRA		OCTET STRING	OPTIONAL,	-- Need N

]]

}

-- TAG-HANDOVER-PREPARATION-INFORMATION-STOP

-- ASN1STOP

HandoverPreparationInformation field descriptions

as-Context Local RAN context required by the target gNB.

sourceConfig The radio resource configuration as used in the source cell.

rrm-Config Local RAN context used mainly for RRM purposes.

ue-CapabilityRAT-List The UE radio access related capabilities concerning RATs supported by the UE. FFS whether certain capabilities are mandatory to provide by source e.g. of target and/or source RAT.

candidateCellInfoList-EUTRA Includes the CandidateCellInfoList-r10 IE as specified in TS 36.331 [X]. Contains a list of the best cells on each frequency for which measurement information was available, in order of decreasing RSRP.

reestablishmentInfoEUTRA Includes the ReestablishmentInfo IE as specified in TS 36.331 [X].

#### –	CG-Config

This message is used to transfer the SCG radio configuration as generated by the SgNB or SeNB.

Direction: Secondary gNB or eNB to master gNB or eNB.

CG-Config message

-- ASN1START

-- TAG-CG-CONFIG-START

CG-Config ::=					SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE{

cg-Config					CG-Config-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

CG-Config-IEs ::=			SEQUENCE {

scg-CellGroupConfig					OCTET STRING (CONTAINING RRCReconfiguration)	OPTIONAL,

scg-RB-Config						OCTET STRING (CONTAINING RadioBearerConfig)		OPTIONAL,

configRestrictModReq				ConfigRestrictModReqSCG							OPTIONAL,

drx-InfoSCG							DRX-Info										OPTIONAL,

candidateCellInfoListSN				OCTET STRING (CONTAINING MeasResultList2NR)	OPTIONAL,

measConfigSN						MeasConfigSN									OPTIONAL,

selectedBandCombinationNR			BandCombinationIndex							OPTIONAL,

fr-InfoListSCG						FR-InfoList										OPTIONAL,

nonCriticalExtension				CG-ConfigNE-DC-vxx-IEs										OPTIONAL

}

CG-ConfigNE-DC-vxx-IEs ::=		SEQUENCE {

scg-CellGroupConfigEUTRA			OCTET STRING (CONTAINING RRCConnectionReconfiguration-r1250-IEs)	OPTIONAL,

candidateCellInfoListSN-EUTRA		OCTET STRING (CONTAINING MeasResultList2EUTRA-r9)	OPTIONAL,

nonCriticalExtension				SEQUENCE {}											OPTIONAL

}

MeasConfigSN ::= SEQUENCE {

measuredFrequenciesSN				SEQUENCE (SIZE (1.. maxMeasFreqsSN))	OF NR-FreqInfo	OPTIONAL,

...

}

NR-FreqInfo ::= SEQUENCE {

measuredFrequency 					ARFCN-ValueNR									OPTIONAL,

...

[[ measuredFrequencyEUTRA				ARFCN-ValueEUTRA								OPTIONAL,

]]

}

ConfigRestrictModReqSCG ::=			SEQUENCE {

requestedBC-MRDC					BandCombinationIndex							OPTIONAL,

requestedP-MaxFR1				P-Max												OPTIONAL,

...

}

BandCombinationIndex ::= INTEGER (1..maxBandComb)

FR-InfoList ::=	SEQUENCE (SIZE (1..maxNrofServingCells-1)) OF FR-Info

FR-Info ::=	SEQUENCE {

servCellIndex		ServCellIndex,

fr-Type				ENUMERATED {fr1, fr2}

}

-- TAG-CG-CONFIG-STOP

-- ASN1STOP

CG-Config field descriptions

candidateCellInfoListSN Contains information regarding cells that the source secondary node suggests the target secondary gNB to consider configuring.

candidateCellInfoListSN-EUTRA Includes the MeasResultList2EUTRA-r9 as specified in TS 36.331 [X]. Contains information regarding cells that the source secondary node suggests the target secondary eNB to consider configuring.

fr-InfoListSCG Contains information of FR information of serving cells.

measuredFrequenciesSN Used by SN to indicate a list of frequencies measured by the UE.

requestedP-MaxFR1 IRequested value for the maximum power for FR1 (see TS 38.104 [12]) the UE can use in NR SCG.

requestedBC-MRDC Used to request configuring an NR band combination which is forbidden to use by MN. Each entry refers to a band combination numbered according to supportedBandCombination in the UE-MRDC-Capability.

scg-CellGroupConfig Contains the RRCReconfiguration message, used to (re-)configure the SCG configuration upon SCG establishment or modification, as generated (entirely) by the (target) SgNB

scg-CellGroupConfigEUTRA Includes the RRCConnectionReconfiguration-v1250-IEs as specified in TS 36.331 [X]. Used to (re-)configure the SCG configuration upon SCG establishment or modification, as generated (entirely) by the (target) SeNB

scg-RB-Config Contains the IE RadioBearerConfig, used to establish or reconfigure the SCG configuration, used to (re-)configure the SCG RB configuration upon SCG establishment or modification, as generated (entirely) by the (target) SgNB

selectedBandCombinationNR Indicates the band combination selected by SN for the EN-DC.

configRestrictModReq Used by SN to request changes to SCG configuration restrictions previously set by MN to ensure UE capabilities are respected. E.g. can used to request configuring an NR band combination whose use MN has previously forbidden.

#### –	CG-ConfigInfo

This message is used by master eNB or gNB to request the SgNB or SeNB to perform certain actions e.g. to establish, modify or release an SCG. The message may include additional information e.g. to assist the SgNB or SeNB to set the SCG configuration.It can also be used by a CU to request a DU to perform certain actions, e.g. to establish, modify or release an MCG or SCG.

Direction: Master eNB or gNB to secondary gNB or eNB, alternatively CU to DU.

CG-ConfigInfo message

-- ASN1START

-- TAG-CG-CONFIG-INFO-START

CG-ConfigInfo ::=				SEQUENCE {

criticalExtensions				CHOICE {

c1								CHOICE{

cg-ConfigInfo				CG-ConfigInfo-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture		SEQUENCE {}

}

}

CG-ConfigInfo-IEs ::=		SEQUENCE {

ue-CapabilityInfo			OCTET STRING (CONTAINING UE-CapabilityRAT-ContainerList)		OPTIONAL,-- Cond SN-Addition

candidateCellInfoListMN			MeasResultList2NR									OPTIONAL,

candidateCellInfoListSN			OCTET STRING (CONTAINING MeasResultList2NR)			OPTIONAL,

measResultCellListSFTD			MeasResultCellListSFTD									OPTIONAL,

scgFailureInfo					SEQUENCE {

failureType						ENUMERATED { t310-Expiry, randomAccessProblem,

rlc-MaxNumRetx, scg-ChangeFailure,

scg-reconfigFailure,

srb3-IntegrityFailure},

measResultSCG					OCTET STRING (CONTAINING MeasResultSCG-Failure)

}																						OPTIONAL,

configRestrictInfo			ConfigRestrictInfoSCG										OPTIONAL,

drx-InfoMCG					DRX-Info													OPTIONAL,

measConfigMN				MeasConfigMN												OPTIONAL,

sourceConfigSCG				OCTET STRING (CONTAINING RRCReconfiguration)				OPTIONAL,

scg-RB-Config             	OCTET STRING (CONTAINING RadioBearerConfig)        			OPTIONAL,

mcg-RB-Config				OCTET STRING (CONTAINING RadioBearerConfig)					OPTIONAL,

nonCriticalExtension		CG-ConfigInfoNE-DC-vxx-IEs													OPTIONAL

}

CG-ConfigInfoNE-DC-vxx-IEs ::=	SEQUENCE {

candidateCellInfoListMN-EUTRA		OCTET STRING										OPTIONAL,

candidateCellInfoListSN-EUTRA		OCTET STRING (CONTAINING MeasResultList2EUTRA-r9)	OPTIONAL,

measResultCellListSFTD		OCTET STRING												OPTIONAL,

scgFailureInfoEUTRA			OCTET STRING												OPTIONAL,

sourceConfigSCGEUTRA		OCTET STRING (CONTAINING RRCConnectionReconfiguration-r1250-IEs)	OPTIONAL,

nonCriticalExtension		SEQUENCE {}

}

ConfigRestrictInfoSCG ::=		SEQUENCE {

allowedBC-ListMRDC				BandCombinationIndexList								OPTIONAL,

powerCoordination-FR1				SEQUENCE {

p-maxNR							P-Max												OPTIONAL,

p-maxEUTRA						P-Max												OPTIONAL

}																						OPTIONAL,

servCellIndexRangeSCG			SEQUENCE {

lowBound						ServCellIndex,

upBound							ServCellIndex

}																						OPTIONAL,   -- Cond SN-Addition

maxMeasFreqsSCG-NR					INTEGER(1..maxMeasFreqsMN)							OPTIONAL,

maxMeasIdentitiesSCG-NR				INTEGER(1..maxMeasIdentitiesMN)						OPTIONAL,

...

}

BandCombinationIndexList ::= SEQUENCE (SIZE (1..maxBandComb)) OF BandCombinationIndex

DRX-Info ::=					SEQUENCE {

drx-LongCycleStartOffset		CHOICE {

ms10							INTEGER(0..9),

ms20							INTEGER(0..19),

ms32							INTEGER(0..31),

ms40							INTEGER(0..39),

ms60							INTEGER(0..59),

ms64							INTEGER(0..63),

ms70							INTEGER(0..69),

ms80							INTEGER(0..79),

ms128							INTEGER(0..127),

ms160							INTEGER(0..159),

ms256							INTEGER(0..255),

ms320							INTEGER(0..319),

ms512							INTEGER(0..511),

ms640							INTEGER(0..639),

ms1024							INTEGER(0..1023),

ms1280							INTEGER(0..1279),

ms2048							INTEGER(0..2047),

ms2560							INTEGER(0..2559),

ms5120							INTEGER(0..5119),

ms10240							INTEGER(0..10239)

},

shortDRX							SEQUENCE {

drx-ShortCycle						ENUMERATED	{

ms2, ms3, ms4, ms5, ms6, ms7, ms8, ms10, ms14, ms16, ms20, ms30, ms32,

ms35, ms40, ms64, ms80, ms128, ms160, ms256, ms320, ms512, ms640, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 },

drx-ShortCycleTimer					INTEGER (1..16)

}																					OPTIONAL

}

MeasConfigMN ::= SEQUENCE {

measuredFrequenciesMN				SEQUENCE (SIZE (1..maxMeasFreqsMN))	OF NR-FreqInfo	OPTIONAL,

measGapConfig					SetupRelease { GapConfig }							OPTIONAL,

gapPurpose							ENUMERATED {perUE, perFR1}							OPTIONAL,

...

}

-- TAG-CG-CONFIG-INFO-STOP

-- ASN1STOP

CG-ConfigInfo field descriptions

allowedBandCombinationListMRDC A list of indices referring to band combinations in MR-DC capabilities from which SN is allowed to select an NR band combination. Each entry refers to a band combination numbered according to supportedBandCombination in the UE-MRDC-Capability. All MR-DC band combinations indicated by this field comprise the same LTE band combination.

candidateCellInfoListMN, candidateCellInfoListSN Contains information regarding cells that the master node or the source node suggests the target gNB to consider configuring. Including CSI-RS measurement results in candidateCellInfoListMN is not supported in this version of the specification.

candidateCellInfoListMN-EUTRA, candidateCellInfoListSN-EUTRA Includes the MeasResultList2EUTRA-r9 as specified in TS 36.331 [X]. Contains information regarding cells that the master node or the source node suggests the target secondary eNB to consider configuring.

maxMeasFreqsSCG-NR Indicates the maximum number of NR inter-frequency carriers the SN is allowed to configure with PSCell for measurements.

maxMeasIdentitiesSCG-NR Indicates the maximum number of allowed measurement identities that the SCG is allowed to configure.

measuredFrequenciesMN Used by MN to indicate a list of frequencies measured by the UE.

measGapConfig Indicates the measurement gap configuration configured by MN.

mcg-RB-Config Contains the IE RadioBearerConfig of the MN, used to support delta configuration for bearer type change between MN terminated to SN terminated bearer and SN change.

p-maxEUTRA Indicates the maximum power for EUTRA (see TS 36.104 [XX]) the UE can use in LTE MCG.

p-maxNR Indicates the maximum power for NR (see TS 38.104 [12]) the UE can use in NR SCG.

powerCoordination-FR1 Indicates the maximum power that the UE can use in FR1.

scg-RB-Config Contains the IE RadioBearerConfig of the SN, used to support delta configuration e.g. during SN change. This field is absent when master eNB uses full configuration option.

sourceConfigSCG Includes the current dedicated SCG configuration in the same format as the RRCReconfiguration message, i.e. not only CellGroupConfig but also e.g. measConfig. This field is absent when master eNB uses full configuration option.

sourceConfigSCG Includes the RRCConnectionReconfiguration-v1250-IEs as specified in TS 36.331 [X]. This field is absent when master gNB uses full configuration option.

measResultCellListSFTD Includes the MeasResultSSTD-r13 IE as specified in TS 36.331 [X].

scgFailureInfoEUTRA Includes the FailureReportSCG-r12 IE as specified in TS 36.331 [X].

ConfigRestrictInfo Includes fields for which SgNB is explictly indicated to observe a configuration restriction.

servCellIndexRangeSCG Range of serving cell indices that SN is allowed to configure for SCG serving cells.

| Conditional Presence | Explanation |
| --- | --- |
| SN-Addition | The field is mandatory present upon SN addition. |