3GPP TSG-RAN WG2 NR Ad hoc 0718	R2-1809977

# Montreal, Canada, 2nd – 6th Jul 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR |  | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to paging parameter |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-07-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Paging formula is now defined in TS 38.304. In the specification, Parameters nB, PF_offset, and the length of default DRX Cycle should be signaled in SystemInformationBlock1 but the parameters are not yet defined in TS 38.331. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Section 6.3.2, paging parameters are added to “PCCH-Config” according to the paging session of TS 38.304. This CR is based on R2-1809239. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The newtork cannot configure paging parameter |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR 38.331 CR 0103 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of modification >

–	DownlinkConfigCommonSIB

The IE DownlinConfigCommon provides common downlink parameters of a cell.

DownlinkConfigCommonSIB information element

-- ASN1START

-- TAG-DOWNLINK-CONFIG-COMMON-SIB-START

DownlinkConfigCommonSIB ::=		SEQUENCE {

frequencyInfoDL					FrequencyInfoDLSIB,

initialDownlinkBWP				BWP-DownlinkCommon,

bcch-Config 						BCCH-Config,

pcch-Config 						PCCH-Config,

...

}

BCCH-Config ::=					SEQUENCE {

modificationPeriodCoeff			ENUMERATED {n2, n4, n8, n16}

}

PCCH-Config ::=					SEQUENCE {

nB						ENUMERATED { fourT, twoT, oneT, halfT, quarterT, oneEighthT,

oneSixteenthT, spare1 }

defaultPagingCycle                  ENUMERATED { ms320, ms640, ms1280, ms2560 }

PF-offset                           INTEGER (0..15)

}

-- TAG-DOWNLINK-CONFIG-COMMON-SIB-STOP

-- ASN1STOP

DownlinkConfigCommon field descriptions

frequencyInfoDLSIB Basic parameters of a downlink carrier and transmission thereon

initialUplinkBWP The initial downlink BWP configuration for a SpCell (PCell of MCG or SCG).

bcch-Config The modification period related configuration.

pcch-Config The paging related configuration.

nB Number of total paging occasions in T. nB is used as one of parameters to derive the Paging Frame and Paging Occasion according to TS 38.304 [4]. Value in multiples of 'T' as defined in TS 38.304 [4]. A value of fourT corresponds to 4 * T, a value of twoT corresponds to 2 * T and so on. If paging-SearchSpace is not configured, PO can be located at every SSB periodicity i.e. 5ms, 10ms, 20ms, 40ms, 80ms, and 160ms. So nB can be set to 2T, T, T/2, T/4, T/8, and T/16 respectively.

defaultPagingCycle Default paging cycle, used to derive ‘T’ in TS 38.304 [4]. Value ms320 corresponds to 320 ms, ms640 corresponds to 640ms and so on.

PF-Offset For pattern 1 of multiplexing RMSI with SSB, offset is 0 as PO always starts in radio frame for which SFN mod 2 = 0. For pattern 2 or 3, offset is equal to smallest SFN in which SSBs are transmitted. (0 for nB >=T, 0...1 for nB =T/2; 0...3 for nB=T/4; 0...7 for nB = T/8; and 0…15 for nB = T/16)

<End of modification >