3GPP TSG-RAN WG2 Meeting #AH 1807 	R2-1810016

# Montreal, Canada, 2nd July 2018 – 6th July 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 0126 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for beam failure recovery configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-04-05 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When BFR RACH is initiated, and if the active UL BWP is configured with PRACH occasions (i.e., BeamFailureRecoveryConfig), the UE will switch the active DL BWP to the DL BWP with the same BWP ID as the active UL BWP. However, the BeamFailureRecoveryConfig has already included the search space ID, and the UE will monitor the PDCCH according to the configured search space ID. It should be noted that the search space ID is unique among the BWPs of a serving cell according the TS 38.331, which means the search space ID is associated with a specific DL BWP. So, if the UE switches to the DL BWP with the same index as active UL BWP but there is no such search space ID configured on the DL BWP, the UE can not monitor the PDCCH according to the beam failure recovery configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The index of DL BWP on which the recoverySearchSpaceId is configured should be the same as the UL BWP on which the BeamFailureRecoveryConfig is configured Impact analysis Impacted functionality: BeamFailureRecoveryConfig. Inter-operability: - If the network is implemented according to the CR and the UE is not there are no inter-operability problems. - If the UE is implemented according to the CR and the network is not, there are no inter-operability problems. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Network has no means to configured LCP restrictions for certain logical channel which carries the data requiring high reliability. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 38.321 CR xxxx |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# 6	Protocol data units, formats and parameters (ASN.1)

## 6.3	RRC information elements

### 6.3.2	Radio resource control information elements

#### –	BeamFailureRecoveryConfig

The BeamFailureRecoveryConfig IE is used to configure the UE with RACH resources and candidate beams for beam failure recovery in case of beam failure detection. See also 38.321, section 5.1.1.

BeamFailureRecoveryConfig information element

-- ASN1START

-- TAG-BEAM-FAILURE-RECOVERY-CONFIG-START

BeamFailureRecoveryConfig ::= 		SEQUENCE {

rootSequenceIndex-BFR				INTEGER (0..137)															OPTIONAL,	-- Need M

rach-ConfigBFR						RACH-ConfigGeneric															OPTIONAL,	-- Need M

rsrp-ThresholdSSB				RSRP-Range																	OPTIONAL,	-- Need M

candidateBeamRSList					SEQUENCE (SIZE(1..maxNrofCandidateBeams)) OF PRACH-ResourceDedicatedBFR		OPTIONAL,	-- Need M

ssb-perRACH-Occasion				ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen} 	OPTIONAL,	-- Need M

ra-ssb-OccasionMaskIndex			INTEGER (0..15)																OPTIONAL, 	-- Need M

recoverySearchSpaceId				SearchSpaceId																OPTIONAL,	-- Cond CF-BFR

ra-Prioritization					RA-Prioritization															OPTIONAL,	-- Need R

beamFailureRecoveryTimer			ENUMERATED {ms10, ms20, ms40, ms60, ms80, ms100, ms150, ms200}			OPTIONAL,	-- Need M

...

}

PRACH-ResourceDedicatedBFR ::= 		CHOICE {

ssb									BFR-SSB-Resource,

csi-RS								BFR-CSIRS-Resource

}

BFR-SSB-Resource ::= 			SEQUENCE {

ssb								SSB-Index,

ra-PreambleIndex				INTEGER (0..63),

...

}

BFR-CSIRS-Resource ::=			SEQUENCE {

csi-RS							NZP-CSI-RS-ResourceId,

ra-OccasionList					SEQUENCE (SIZE(1..maxRA-OccasionsPerCSIRS)) OF INTEGER (0..maxRA-Occasions-1)	OPTIONAL,	-- Need R

ra-PreambleIndex				INTEGER (0..63)																	OPTIONAL,	-- Need R

...

}

-- TAG-BEAM-FAILURE-RECOVERY-CONFIG-STOP

-- ASN1STOP

BeamFailureRecoveryConfig field descriptions

beamFailureRecoveryTimer Timer for beam failure recovery timer. Upon expiration of the timer the UE does not use CFRA for BFR. Value in ms. ms10 corresponds to 10ms, ms20 to 20ms, and so on.

candidateBeamRSList A list of reference signals (CSI-RS and/or SSB) identifying the candidate beams for recovery and the associated RA parameters

rsrp-ThresholdSSB L1-RSRP threshold used for determining whether a candidate beam may be used by the UE to attempt contention free Random Access to recover from beam failure. The signalled threshold is applied directly for SSB; a threshold for CSI-RS is determined by linearly scaling singalled value based on Pc_ss corresponding to the CSI-RS resource. (see FFS_Specification, FFS_Section)

ra-prioritization Parameters which apply for prioritized random access procedure for BFR (see 38.321, section 5.1.1).

ra-ssb-OccasionMaskIndex Explicitly signalled PRACH Mask Index for RA Resource selection in TS 36.321. The mask is valid for all SSB resources

rach-ConfigBFR Configuration of contention free random access occasions for BFR

recoverySearchSpaceId Search space to use for BFR RAR. The bwp-Id of the DL BWP on which the search space is configured should be the same as the bwp-Id of the UL BWP on which the BeamFailureRecoveryConfig is configured.

ssb-perRACH-Occasion Number of SSBs per RACH occasion (L1 parameter 'SSB-per-rach-occasion')

BFR-CSIRS-Resource field descriptions

csi-RS The ID of a NZP-CSI-RS-Resource configured in the CSI-MeasConfig of this serving cell. This reference signal determines a candidate beam for beam failure recovery (BFR).

ra-OccasionList RA occasions that the UE shall use when performing BFR upon selecting the candidate beam identified by this CSI-RS. If the field is absent the UE uses the RA occasion associated with the SSB that is QCLed with this CSI-RS.

ra-PreambleIndex The RA preamble index to use in the RA occasions associated with this CSI-RS. If the field is absent, the UE uses the preamble index associated with the SSB that is QCLed with this CSI-RS.

BFR-SSB-Resource field descriptions

ra-PreambleIndex The preamble index that the UE shall use when performing BFR upon selecting the candidate beams identified by this SSB.

ssb The ID of an SSB transmitted by this serving cell. It determines a candidate beam for beam failure recovery (BFR)

| Conditional Presence | Explanation |
| --- | --- |
| CF-BFR | The field is mandatory present, Need R, if CF-BFR is configured. It is optionally present otherwise. |