3GPP TSG-RAN WG2 Meeting NR Adhoc 1807		R2-1810034

Montreal, Canada, 2nd - 6th July 2018

Title:	[DRAFT] LS on BWP usage during initial access

Response to:	-

Release:	Release 15

Work Item:	NR_newRAT

Source:	Nokia [TSG RAN WG2]

To:	TSG RAN WG4

Cc:	TSG RAN WG1

Contact Person:

#### Name:	Tero Henttonen

E-mail Address:	tero.henttonen @nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 has discussed the details of BWP to be used during initial access, i.e. when UE moves from IDLE or INACTIVE to CONNECTED state.

RAN2 has decided that in case gNB indicates something else than the initial BWP as the first active BWP in Msg (i.e. RRCSetup, RRCResume or RRCReestablishment messaged), the UE shall start using the first active BWP before sending Msg5, i.e. the UE will send the Msg5 using the first active BWP.

It is RAN2 understanding that this may require some BWP switching delay, but it is also RAN2 understanding that this switching delay could be assumed to be contained with the RRC processing delay for the Msg4. That means that RAN2 assumes that if UE received the Msg4 at time n, UE would be ready to utilize the first active BWP immediately at n+X, where X is the appropriate RRC processing delay.

2. Actions:

To RAN4 group.

ACTION: 	RAN2 respectfully asks RAN4 to verify if the RAN2 assumption that the BWP switching delay is part of the RRC processing delay is correct, and provide feedback accordingly.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP TSG RAN WG2 #103	20 – 24 Aug 2018		Gothenburg

3GPP TSG RAN WG2#103bis	8 - 12 Oct 2018   	 	Chengdu

3GPPRAN2#104 		12 - 16 Nov 2018    		Spokane