3GPP TSG-RAN WG2 Meeting AH 1807	R2-1810055

# Montreal, Canada, 2nd – 6th July 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0232 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to duplex operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-06-20 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | When the UE is configured with flexible TDD operation the rules for priortizing between uplink grants, PUCCH transmissions, SRS transmissions, and PDCCH occasions are not clear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The UE skips the PDCCH monitoring occasion if the PDCCH monitoring occasion overlaps with PUSCH transmission, PUCCH transmission, or SRS transmission. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE behaviour in case a PDCCH monitoring occasion overlaps with a PUSCH transmission, a PUCCH transmission, or SRS transmission is not clear. Impact analysis: Impacted functionality: PDCCH monitoring Interoperability analysis: If the network implements the CR and the UE does not the network may schedule the UE such that PDCCH monitoring occasions, PUCCH, PUSCH, and SRS transmissions overlap. If the UE implements the CR and the network does not the UE may skip certain PDCCH monitoring occasions which the network expects the UE to read. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

# 5.X	Flexible TDD operation

The MAC entity shall:

1>	if the MAC entity receives an uplink grant to indicate an uplink transmission on a serving cell and if a PDCCH occasion on the same serving cell overlaps with this uplink transmission in time; or

1>	if a PUCCH transmission and a PDCCH occasion on the same serving cell overlaps in time; or

1>	if an SRS transmission and a PDCCH occasion on the same serving cell overlaps in time:

2>	skip the PDCCH occasion.