3GPP TSG-RAN WG2 AH 1807	Tdoc R2-1810069

Montreal, Canada, 2nd – 6th July 2018

Agenda Item:	10.3.1.2

Source:	Ericsson

Title:	BWP Inactivity timer and Measurement Gap

Document for:	Discussion, Decision

# Introduction

BWP inactivity timer is running when a UE is served by an active BWP other than the default (or initial) BWP. If there is data and/or associated control message transmission in the active BWP, the BWP inactivity timer is restarted so that the UE continues to be served in the active BWP. If the BWP inactivity timer expires, the UE should fall back to its default BWP. Considering that the service providing is halted during a measurement gap, it is necessary to investigate the behaviour of BWP inactivity timer in case of a measurement gap. This paper investigates the impact on BWP switch from a measurement gap.

# Discussion

In [1], it was proposed that the BWP inactivity timer should be suspended during a measurement gap and continue after the measurement gap. In such way, the UE may use the non-default BWP for a long time. In the following, we will discuss if this is necessary.

## BWP inactivity timer operation and measurement gap

In 38.321, it is defined that the UE does not monitor PDCCH other than the PDCCH for RAR during a measurement gap:

“During a measurement gap, the MAC entity shall:

1>	not perform the transmission of HARQ feedback, SR, and CSI;

1>	not report SRS;

1>	not transmit on UL-SCH except for Msg3 as specified in subclause 5.4.2.2;

1>	if the ra-ResponseWindow or the ra-ContentionResolutionTimer is running:

2>	monitor the PDCCH as specified in subclauses 5.1.4 and 5.1.5.

1>	else:

2>	not monitor the PDCCH.”

The UE does not transmit/receive service data during a measurement gap.

According to the IE MeasGapConfig in TS 38.331, a measurement gap may last 1.5~6 ms:

MeasGapConfig ::=				SEQUENCE {

gapFR2 						SetupRelease { GapConfig }				OPTIONAL,

...

}

GapConfig ::=					SEQUENCE {

gapOffset 					INTEGER (0..159),

mgl 						ENUMERATED {ms1dot5, ms3, ms3dot5, ms4, ms5dot5, ms6},

mgrp 						ENUMERATED {ms20, ms40, ms80, ms160},

mgta						ENUMERATED {ms0, ms0dot25, ms0dot5},

...

}”

In servingCellConfig IE in TS 38.331, the candidate values of a BWP inactivity timer range from 2 ms to 2560 ms.

bwp-InactivityTimer		ENUMERATED {ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40,ms50, ms60, ms80, ms100, ms200, ms300, ms500, ms750, ms1280, ms1920, ms2560, spare10, spare9, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 } OPTIONAL,	--	Need R

If the BWP inactivity timer continues run during a measurement gap, it can be regarded that the effective value of BWP timer is reduced.

Effective BWP inactivity timer value reduces upon a measurement gap, especially when small timer value is configured.

## Impact from BWP inactivity timer continuation during a measurement gap

According to the reference mentioned above, the BWP inactivity timer value ranges between 2 ms and 2560 ms and it is up to the network to configure which value is used:

If a small value is configured for a BWP inactivity timer, it implies that the gNB just uses the active BWP other than the default (or initial) BWP for short time offloading, for instance, when the default (or initial BWP) cannot provide enough capacity for the data burst due to heavy traffic load or there is a limited operation bandwidth for the default (initial BWP). The UE is expected to use the default BWP in all conditions except the temporary high load situations. This further means that it is acceptable for the UE fall back to the default BWP after a short term in this case. Once offloading is needed, the gNB may then switch a UE back to the non-default BWP. Hence, in this case it doesn’t matter if a UE falls back to the default(or initial) BWP earlier due to a measurement gap.

If a medium or large value is configured for a BWP inactivity timer, a measurement gap is rather small compared to the value of BWP inactivity timer. Hence the effective BWP timer value reduction due to a measurement gap is ignorable.

According to the above analysis, we can observe that:

The impact on BWP inactivity timer value reduction due to a measurement gap is negligible.

Considering the minor impact from measurement gap, suspending the BWP inactivity timer during a measurement gap as in [1] is not necessary. Hence, we propose:

Measurement gap occurrence does not impact the BWP inactivity timer operation.

# Conclusion

In section 2 we made the following observations:

Observation 1	The UE does not transmit/receive service data during a measurement gap.

Observation 2	Effective BWP inactivity timer value reduces upon a measurement gap, especially when small timer value is configured.

Observation 3	The impact on BWP inactivity timer value reduction due to a measurement gap is negligible.

Based on the discussion in section 2 we propose the following:

Proposal 1	Measurement gap occurrence does not impact the BWP inactivity timer operation.

# References

R2-1807571	“Stop BWP inactivity timer during the measurement Gap”, 3GPP RAN2-102