3GPP TSG-RAN WG2 Meeting AH1807	R2-1810080

# Montreal, Canada, 2nd – 6th July 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0236 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TCI state and beam failure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-06-20 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the current version of the specification the TCI state is not reset at beam failure. This causes an ambiguity on how the UE determines its QCL sources during contention-based beam failure recovery. In the current version of the specification, the TCI state of the PDCCH is never reset, which implies that the UE may assume that it is activated also after a handover and after beam failure, despite the fact that the TCI state is clearly not valid. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The PDSCH TCI state is deactivated after a beam failure. The PDCCH TCI state is deactivated upon configuration, after a handover and after beam failure. Impact analysis: Impacted functionality: TCI state Interoperability analysis: If the network implements the CR and the UE does not no interoperability problem is foreseen. If the UE implements the CR and the network does not the beam management operation in the UE will not operate correctly after e.g. beam failure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is an ambiguity on how the UE determines its QCL sources in certain circumstances. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.4, 5.18.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 5.18.4	Activation/Deactivation of UE-specific PDSCH TCI state

The network may activate and deactivate the configured TCI states for PDSCH of a Serving Cell by sending the TCI States Activation/Deactivation for UE-specific PDSCH MAC CE described in subclause 6.1.3.14. The configured TCI states for PDSCH are initially deactivated upon configuration, after a handover, and after beam failure.

The MAC entity shall:

1>	if the MAC entity receives an TCI States Activation/Deactivation for UE-specific PDSCH MAC CE on a Serving Cell:

2>	indicate to lower layers the information regarding the TCI States Activation/Deactivation for UE-specific PDSCH MAC CE.

### 5.18.5	Indication of TCI state for UE-specific PDCCH

The network may indicate a TCI state for PDCCH reception for a CORESET of a Serving Cell by sending the TCI State Indication for UE-specific PDCCH MAC CE described in subclause 6.1.3.15. The configured TCI states for PDCCH are initially deactivated upon configuration, after a handover, and after beam failure.

The MAC entity shall:

1>	if the MAC entity receives a TCI State Indication for UE-specific PDCCH MAC CE on a Serving Cell:

2>	indicate to lower layers the information regarding the TCI State Indication for UE-specific PDCCH MAC CE.