3GPP TSG-RAN WG2 Meeting NR Adhoc 1807	R2-1810148

Montreal, Canada, 2nd - 6th July 2018

Agenda item:	10.4.5.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Dedicated Cell Reselection Priorities

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

This contribution provides stage 3 details for dedicated cell reselection priorities provided in RRCRelease message.

# 2	Discussion

RAN2#101 agreed:

Agreements

1	Working assumption from RAN2#99 is confirmed ("For needs of slicing, appropriate configuration of the dedicated priorities provided from the gNB can be used to control the frequency on which the UE camps. (i.e. reuse of same mechanism as in LTE). ")

2	No additional mechanisms for frequency prioritisation with respect to slicing will be specified for Rel-15

This implies the RRCRelease message providing the frequency list for reselection. The dedicated signalling will serve the purpose to prioritise frequencies according to gNB configuration taking into account slicing policies.

### 2.1 TS38.331 implementation

The TS38.331 baseline that is being used in ASN.1 process, supports the agreement and provides the mean to set priorities. RRCRelease supports signalling of Cell Reselection Priorities as follows:

RRCRelease-IEs ::=		SEQUENCE {

redirectedCarrierInfo				RedirectedCarrierInfo				OPTIONAL,	-- Need N

cellReselectionPriorities			CellReselectionPriorities							OPTIONAL,	-- Need M

suspendConfig						SuspendConfig						OPTIONAL,	-- Need N

deprioritisationReq				SEQUENCE {

deprioritisationType				ENUMERATED {frequency, nr},

deprioritisationTimer				ENUMERATED {min5, min10, min15, min30}

},

lateNonCriticalExtension				OCTET STRING															OPTIONAL,

nonCriticalExtension					SEQUENCE{}																OPTIONAL

}

CellReselectionPriorities ::=		SEQUENCE {

freqPriorityListEUTRA				FreqPriorityListEUTRA			OPTIONAL,		-- Need M

freqPriorityListNR					FreqPriorityListNR				OPTIONAL, 		-- Need M

t320								ENUMERATED {

min5, min10, min20, min30, min60, min120, min180,

spare1}						OPTIONAL,		-- Need R

...

}

### 2.2 TS38.304 implementation

However, it remains unclear if Idle UE procedures for Cell reselection process support the agreement, as the interpretation of priorities is left to UE implementation:

#### 5.2.4.1	Reselection priorities handling

Absolute priorities of different NR frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, a NR frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). If priorities are provided in dedicated signalling, the UE shall ignore all the priorities provided in system information.

NOTE 1:	The prioritization among the frequencies which UE considers to be the highest priority frequency is left to UE implementation.

For slicing needs, there has been coordinated effort made among groups (SA2, RAN3, RAN2) to ensure that the gNB can get the relevant slice information to configure the UE in an appropriate way that support slice prioritisation. To let the gNB remains in control and the UE respects frequencies order provided in the dedicated signalling we propose the following:

Proposal 1: If provided, the UE performs cell reselection evaluation according to frequency priorities provided in dedicated signalling.

# 3	Conclusion

Proposal 1: If provided, UE performs cell reselection evaluation according to frequency priorities provided in dedicated signalling.

Proposal 2: Agree corresponding CR in [1]

# References

- R2-1810149, Dedicated Cell Reselection Priorities supporting Network Slicing, Nokia, Nokia Shanghai Bell