3GPP TSG-RAN WG2 Meeting NR Adhoc 1807	R2-1810150

# Montreal, Canada, 2nd - 6th July 2018													R2-18107173

Agenda item:	10.4.1.8.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Access Category for AS-triggered events

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

In this contribution we discuss options for selecting Access Category for AS-triggered events and their consequences.

# 2	Discussion

### 2.1	Which Access Category?

RAN based notification area in RRC_INACTIVE is managed by RAN. Based on the RAN2#101 agreements RNA update will be an additional event (out of NAS evens for access control) to trigger Access Barring Control in RRC_INACTIVE.

According to RAN2#101bis agreements there are two possible ways to define RNA Update association with Access Category:

1. Access Category 3 MO-signalling for RNA Update is reused. (see Table 6.22.2.3-1 in TS22.261, excerpt below)

Table 1: Access Categories in TS 22.261

| Access Category number | Conditions related to UE | Type of access attempt |
| --- | --- | --- |
| 0 | All | MO signalling resulting from paging |
| 1 (NOTE 1) | UE is configured for delay tolerant service and subject to access control for Access Category 1, which is judged based on relation of UE’s HPLMN and the selected PLMN. | All except for Emergency |
| 2 | All | Emergency |
| 3 | All except for the conditions in Access Category 1. | MO signalling resulting from other than paging |
| 4 | All except for the conditions in Access Category 1. | MMTEL voice |
| 5 | All except for the conditions in Access Category 1. | MMTEL video |
| 6 | All except for the conditions in Access Category 1. | SMS |
| 7 | All except for the conditions in Access Category 1. | MO data that do not belong to any other Access Categories |
| 8-31 |  | Reserved standardized Access Categories |
| 32-63 (NOTE 2) | All | Based on operator classification |

- Standard impacts: if the existing Access Category 3 is selected for the RNA Update, the handling and will not require additional specific coordination with SA1 or maintenance of the Access Category across specifications. The existing characteristic can apply. Extension of the Access Category applicability may be optional outside of RAN2 specifications.

- Access Control impacts: A possible drawback with this approach is lack of possibility to distinguish Access Barring Control for the INACTIVE UEs from IDLE UEs. The same Access Category 3 would cover any case of MO-signalling resulting other from paging. However, split for paging and other MO- signalling originated calls provides already crucial priority that should be sufficient for the system. Furthermore, definition of the RNA Resume cause would serve the purpose to identify AS-triggered attempts specifically and differently, even though not by means of UAC. Thus, a separate Access Category is an optimization.

- SIB impacts: There is no impact to SIB contents. With that approach UAC barring configuration is set only once for MO-signalling connections and RNA.

2. A new Access Category for RNA Update is defined.

- Standard impacts: if a new Access Category is selected for the RNA Update, the handling will require additional coordination with SA1 and definition of the Access Category across specifications. The existing characteristic on SA1 would miss the case .

- Access Control impacts: With this approach, Access Control would allow identifying AS-triggered attempts specifically and differently by two means: establishment cause and UAC. The possibility to control of access attempts depending on their nature, would offer two means and differentiation of the INACTIVE UEs from IDLE already.

- SIB impacts: Selection of a new Access Category will require separate UAC barring configuration in SIB (e.g. different setting of parameters) which impacts signalling load.

Observation: Definition of a new Access Category will have an impact on modelling and broadcast information load. Reuse of the Access Category 3 reveals benefits.

Therefore, for simplicity we propose the following:

Proposal 1: Access Category 3 for MO-signalling is reused for RNA update.

# 3	Conclusions

Observation: Definition of a new Access Category will have an impact on modelling and broadcast information load. Reuse of the Access Category 3 reveals benefits.

Proposal 1: Access Category 3 for MO-signalling is reused for RNA updat