3GPP TSG-RAN WG2 AH 1807	Tdoc R2-1810173

Montreal, Canada, 2nd – 6th July 2018

Agenda Item:	10.3.2.4

Source:	Ericsson

Title:	Out-of-order transmission of RLC PDUs

Document for:	Discussion, Decision

# 1	Introduction

In RAN2#102 meeting, it was discussed how the RLC created RLC PDUs and in which order these packets are transmitted to the MAC layer. This document discusses these procedures.

# 2	Discussion

In [1], it is claimed that the RLC specifications does not explicitly prevent the RLC entity to deliver to lower layers RLC PDUs in “out-of-order”.

In NR, pre-processing results in that PDCP delivers PDCP PDUs to the RLC entity even if the UE does not have a transmission opportunity, the RLC entity allocates SN to the RLC SDU when the PDCP entity delivers a PDCP PDU to the RLC entity and store the RLC SDU/PDUs until MAC requests the data.  According to section 5.2.3.1.1:

[…]

For each RLC SDU received from the upper layer, the AM RLC entity shall:

-    associate a SN with the RLC SDU equal to TX_Next and construct an AMD PDU by setting the SN of the AMD PDU to TX_Next;

-    increment TX_Next by one.

[…]

This implies that the RLC entity allocates SDU SN in consecutive order as PDCP PDUs from higher layers are submitted. I.e., the 3GPP specification does not specify any possibility to adapt and set the SN of the RLC PDU in any other way such as setting the SN according to the order in which the UE RLC would like to send PDUs to lower layers.

It could be argued, however, that the RLC specifications are not clear with that the RLC may deliver to lower layers RLC PDUs in a different order than following a consecutive SN order. Yet, even if this was possible, it opens up the discussion related to the conditions under which the RLC entity would decide to “postpone” the transmission of some RLC SDUs which require segmentation and when the UE would decide to finally transmit and segment those PDUs which were delayed. Such behavior may result in that some UE implementations may wait for an undefined period of time until a future grant provides sufficient resources to transmit an RLC PDU without segmentation.

[…]

Each RLC SDU is used to construct an RLC PDU without waiting for notification from the lower layer (i.e., by MAC) of a transmission opportunity. In the case of UM and AM RLC entities, an RLC SDU may be segmented and transported using two or more RLC PDUs based on the notification(s) from the lower layer.

RLC PDUs are submitted to lower layer only when a transmission opportunity has been notified by lower layer (i.e. by MAC).

NOTE:	The UE should aim to prevent excessive non-consecutive RLC PDUs in a MAC PDU when the UE is requested to generate more than one MAC PDU.

[…]

From the NOTE is becomes obvious that on MAC layer, the UE should aim to prevent non-consecutive RLC PDUs within one MAC PDU (and refers to the case of multiple MAC PDUs). The NOTE thereby refers to behavior within MAC layer and does not relate to the discussion of in which order RLC PDUs are delivered to MAC in the first place.

The behavior suggested in [1] impacts both PDCP and RLC protocols negatively, i.e. comes with the following issues:

For PDCP, the receiver may receive PDCP PDUs in out-of-sequence which will impact PDCP reordering. While reordering is not an issue by itself, reordering always increases the delay/jitter towards higher layers. Reordering, furthermore, typically imposes certain memory requirements. 
Since the conditions under which the UE will transmit certain packets not fitting the grant are unclear, there is an increased risk that the receiving window has moved when late packets (which were not transmitted because the grant did not allow a transmission of the full RLC SDU) are finally transmitted by the transmitter and received by the receiver. This can cause packets losses at the PDCP layer. This may be even more critical in dual connectivity. It is important that, in DC, both legs have a similar data rate to avoid large reordering windows and that the lower side of the window is moved when there are still missing packets, producing packet losses.

For RLC, the impact is more severe, especially for RLC AM. Status reports and retransmissions are impacted by the proposed behaviour since it is unclear when the UE may transmit a certain RLC PDU. That can cause that the t-reassembly expires leading to request status reports and that will consequently lead to retransmissions of packets which were not initially transmitted. Polling is also affected. When a PDU is delivered to lower layers, the POLL_SN is updated to the highest SN transmitted to lower layers. Following the principle in [1], the POLL_SN will be updated to the highest SN value which was transmitted without considering that there may be a number of RLC PDUs waiting still for transmission with a lower SN value, and this may lead to retransmissions for packets which were not initially transmitted.
RLC UM is also affected and the impact is very similar as for PDCP. The risk that the receiving window (for segments) has moved when late packets are finally transmitted by the transmitter and received by the receiver also increases causing packet losses.

The behaviour suggested by [1] and result of doing pre-processing is not desired causing undesired effects in the RLC protocol which will lead to worse performance. Thus, we propose:

Confirm that RLC delivers first in first out, i.e. delivers to MAC in the same order as received from PDCP. This applies to both AM (consecutive SNs) and UM (both non-segmented and segmented).

A CR [2] is available to clarify this behavior in the RLC specification.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Confirm that RLC delivers first in first out, i.e. delivers to MAC in the same order as received from PDCP. This applies to both AM (consecutive SNs) and UM (both non-segmented and segmented).

# References

R2-1807564, Out-of-order RLC transmission

R2-1810174, Clarification on RLC transmission order