3GPP TSG-RAN WG2 AH 1807	Tdoc R2-1810178

Montreal, Canada, 2nd – 6th July 2018

Agenda Item:	10.3.2.3

Source:	Ericsson

Title:	RLC behavior after RLC failure in CA duplication

Document for:	Discussion, Decision

# 1	Introduction

When there is an RLC failure due to the maximum number of retransmissions, the LCP restrictions are lifted. This may lead to that the UE continues re-transmitting PDUs to the MAC layer. This contribution looks at this aspect.

# 2	Discussion

When there is an RLC failure and in case of CA duplication, SCell-RLF is triggered, duplication should be deactivated [1]. Because if CA duplication is deactivated, the logical channel restrictions are lifted and the RLC entity which experienced an RLC failure would still deliver data to MAC to be transmitted over the available cell (which was restricted before).

However, once an RLC entity experiences an RLC failure due to the maximum number of retransmissions, the RLC does not transmit new PDUs to lower layers. In NR, the RLC does not prevent the behavior outlined above:

[…]

When an RLC SDU or an RLC SDU segment is considered for retransmission, the transmitting side of the AM RLC entity shall:

-	if the RLC SDU or RLC SDU segment is considered for retransmission for the first time:

-	set the RETX_COUNT associated with the RLC SDU to zero.

-	else, if it (the RLC SDU or the RLC SDU segment that is considered for retransmission) is not pending for retransmission already and the RETX_COUNT associated with the RLC SDU has not been incremented due to another negative acknowledgment in the same STATUS PDU:

-	increment the RETX_COUNT.

-	if RETX_COUNT = maxRetxThreshold:

-	indicate to upper layers that max retransmission has been reached.

[…]

Note that even after the maximum number of RLC retransmissions, the RLC may still deliver packets to MAC if those PDUs are considered for retransmissions and RLC gets transmission opportunities. Both prerequisites may be fulfilled if data duplication is deactivated. This behavior is not wanted and there should be clearly stated that the RLC should not deliver further PDUs.

When there is an RLC failure due to the maximum number of retransmissions being reached, no more RLC SDUs or RLC SDU segments shall be considered for retransmissions.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	When there is an RLC failure due to the maximum number of retransmissions being reached, no more RLC SDUs or RLC SDU segments shall be considered for retransmissions.

# References

R2-1810252, SCell-RLF discussion