3GPP TSG-RAN WG2 AH 1807	Tdoc R2-1810183

Montreal, Canada, 2nd – 6th July 2018

Agenda Item:	10.3.1.8

Source:	Ericsson

Title:	Handling of SPS and CS Reconfiguration

Document for:	Discussion, Decision

# Introduction

SPS reconfiguration (including SPS DL and Type 2 configured scheduling in UL) may change the periodicity of transmission/reception opportunity occurrence. After SPS reconfiguration, a UE should determine the transmission/reception opportunity occurrence according to the new configuration. This paper discusses the ambiguity for a UE to determine the reception/transmission opportunity upon SPS (or Type 2 configured scheduling) reconfiguration in NR.

# Discussion

In the following, the ambiguity from SPS (or Type 2 configured scheduling) reconfiguration is briefly discussed, then we discuss whether prohibition of SPS reconfiguration in LTE may be reused in NR. Lastly, we discuss if such ambiguity can be conquered in NR.

## SPS reconfiguration ambiguity

For SPS DL, RRC configures a set of parameters such as periodicity, number of HARQ processes and PUCCH resources and the serving gNB determines when to activate the SPS assignment and configures the physical layer parameters such as time-frequency resources and MCS using PDCCH addressed to CS-RNTI. Then the UE determines the periodic reception opportunity occurrences according to the configured parameters and SPS activation time using the following formula defined in Section 5.8.1 of 3GPP TS 38.321:

“After a downlink assignment is configured for SPS, the MAC entity shall consider sequentially that the Nth downlink assignment occurs in the slot for which:

(numberOfSlotsPerFrame × SFN + slot number in the frame) =
[(numberOfSlotsPerFrame × SFNstart time + slotstart time) + N × semiPersistSchedIntervalDL × numberOfSlotsPerFrame / 10] modulo (1024 × numberOfSlotsPerFrame)

where SFNstart time and slotstart time are the SFN and slot, respectively, of the first transmission of PDSCH where the configured downlink assignment was (re-)initialised.”

For uplink SPS, which is referred to as Type 2 configured scheduling in NR, also uses the similar mechanism, i.e. a set of parameters such as periodicity, number of HARQ processes and power control parameters are configured via RRC signalling and the serving gNB determines when to activate the configured UL grant Type 2 and configures the physical layer parameters such as time-frequency resources and MCS using PDCCH. Then the UE determines the periodic transmission opportunity occurrences according to the configured parameters and activation time of configured grant Type 2 according to the following formula defined in Section 5.8.2 of 3GPP TS 38.321:

“After an uplink grant is configured for a configured grant Type 2, the MAC entity shall consider sequentially that the Nth uplink grant occurs associated with the symbol for which:

[(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] =
[(SFNstart time × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + slotstart time × numberOfSymbolsPerSlot + symbolstart time) + N × periodicity] modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)

where SFNstart time, slotstart time, and symbolstart time are the SFN, slot, and symbol, respectively, of the first transmission of PUSCH where the configured uplink grant was (re-)initialised.”

For both downlink SPS and Type 2 configured scheduling, the UE shall determine the transmission/reception opportunity reoccurrences (periodicity) with respect to the respective activation (i.e. (re-) initialization) time. If the periodicity is reconfigured when a downlink SPS grant or configured UL grant Type 2 is active, the UE refers to the activation time in order to determine the (new) reception or transmission reoccurrences respectively.

However, upon reconfiguration of SPS (or Type 2 configured scheduling), the index value (i.e. N) corresponding to the new configuration of SPS (or Type 2 configured scheduling) does not start from 0. Even though the UE may remember the activation time of the ongoing/first/previous downlink SPS (or configured UL grant Type 2), the UE still does not know the index value (i.e. N) of the first available reception/transmission opportunity corresponding to the new SPS (or Type 2 configured scheduling) configuration. Without N value, the UE cannot determine the reception/transmission opportunity according to the new SPS (or Type 2 configured scheduling) configuration using the above formula.

When there is no hyper-frame switch after SPS (or configured uplink grant Type 2) activation, the UE can calculate the past time since activation based on the difference of SFN, slot index and OFDM symbol index between the activation time and the reconfiguration time, and then the UE may determine the N value for the new SPS (or Type 2 configured scheduling) configuration based on the time past. However, when the reconfiguration of SPS (or Type 2 configured scheduling)  is in a hyper-frame other than the hyper-frame during which the SPS assignment (or configured UL grant Type 2) is activated, the UE’s MAC entity cannot determine the past time based on the difference of SFN, slot index and OFDM symbol index between the activation time and the reconfiguration time, which means the UE is not able to derive the correct N value corresponding to the new SPS (or Type 2 configured scheduling) configuration. Figure 1 exemplifies the ambiguity for a UE to determine the index of the first available reception opportunity after SPS reconfiguration. For a detailed analysis, please refer to [1].

Figure 1 illustration of ambiguity after SPS reconfiguration

Similarly, as in LTE, there is ambiguity for a UE to determine the reception / transmission opportunity upon reconfiguration of SPS or Type 2 configured scheduling.

## Necessity of SPS reconfiguration in NR

In LTE, SPS reconfiguration is not allowed to avoid the ambiguity mentioned above except handover occurs according to sps-Config IE (including the configuration for both the SPS DL and SPS UL) 3GPP TS 36.321-e22:

“sps-Config

The default SPS configuration is specified in 9.2.3. Except for handover or releasing SPS for MCG, E-UTRAN does not reconfigure sps-Config for MCG when there is a configured downlink assignment, or a configured uplink grant for MCG (see TS 36.321 [6]). Except for SCG change or releasing SPS for SCG, E-UTRAN does not reconfigure sps-Config for SCG when there is a configured downlink assignment, or a configured uplink grant for SCG (see TS 36.321 [6]).”

In LTE, SPS (for both UL and DL) reconfiguration is prohibited in order to avoid the ambiguity to determine the transmission/reception opportunities.

In LTE, the SPS is designed for services with flat rate such as VoIP, the reconfiguration seems not necessary for a UE if a periodicity 20 ms and transmission duration 1 ms is configured. However, in NR the SPS DL (and configured UL grant type 2) were designed for both flat rate service such as VoIP and URLLC service, a SPS (Type 2 configured scheduling) configuration with a periodicity 20 ms and a transmission duration 1 ms may be OK for eMBB and VoIP but not feasible for URLLC service due to the sparse reception/transmission opportunity occurrences and long transmission duration. If reconfiguration of SPS (or Type 2 configured scheduling) is prohibited as in LTE, it may occur that the preconfigured SPS (or Type 2 configured scheduling) configuration cannot fulfil the QoS requirement of the newly generated session, which is not a desired situation for NR. Hence, there is necessity to support reconfiguration of the SPS (or configured UL grant type 2) according to the QoS requirement.

In NR, SPS assignment (or configured grant Type 2) can be used for services (e.g. VoIP, URLLC) of various QoS requirements.

Single configuration of SPS (or Type 2 configured scheduling) cannot fulfil the data transmission requirement upon service (e.g. eMBB/VoIP   URLLC) change of a UE.

Reconfiguration of SPS and configured grant Type 2 is needed in NR.

## Handling of SPS reconfiguration in NR

Assuming the SPS assignment (or configured grant Type 2) is active when reconfiguration of SPS (or Type 2 configured scheduling) is received, there are the following options which may be considered to mitigate the ambiguity mentioned above:

- Option 1: The UE may derive the elapsed time based on the current time (SFN, slot and OFDM symbol), current N value and the periodicity of the original SPS (or Type 2 configured scheduling) configuration. The UE can calculate the new N value corresponding to the new SPS (or Type 2 configured scheduling) configuration using the total elapsed time. In such way, there is not ambiguity to determine the reception/transmission opportunity occurrence upon reconfiguration of SPS (or Type 2 configured scheduling). For this option, the gNB may not know the exact time that new configuration of SPS (or Type 2 configured scheduling) is applied though the UE can determine the reception/transmission opportunity without ambiguity upon reconfiguration of SPS (or Type 2 configured scheduling).

- Option 2: upon reception of the SPS (or Type 2 configured scheduling) reconfiguration, current SPS assignment (or configured uplink grant of Type 2) is automatically deactivated and the new SPS configuration is applied when a new SPS activation command is received. This option is simple compared to Option 2 though the SPS assignment need to be reactivated after SPS configuration. Furthermore, the gNB has no ambiguity to determine when the new SPS (Type2 configured scheduling) configuration is applied.

- Option 3:  upon reception of the SPS (or Type 2 configured scheduling) reconfiguration, a UE continues to run current SPS assignment (or configured uplink grant of Type 2) according to the old configuration of SPS (or Type 2 configured scheduling) until a new activation/reactivation command is received. If a new activation/reactivation command is received, the UE determines the SPS assignment (or configured uplink grant of Type 2) according to the new activation time and the new SPS (or Type 2 configured scheduling) configuration. Compared to Option 1, this method avoids the ambiguity during the transition period. Compared to Option 2, this option avoids the unavailability of SPS ((or configured uplink grant Type 2) during the transition period. One minor complexity is that a UE shall store the old configuration during the transition period.

NR allows for reconfiguration of SPS and configures grant Type 2

Considering the complexity and feasibility, option 3 seems more preferred than Option 1 and Option 2.

Upon reconfiguration of SPS (or Type 2 configured scheduling), a UE shall

  - continue to maintain the active SPS assignment (or configured uplink grant Type 2) based on the old configuration until a new activation/reactivation command is received;

  - apply the new configuration when a new activation/reactivation command is received.

RAN2 supports the correction in [2]

# Conclusion

In section 2 we made the following observations:

Observation 1	Similarly, as in LTE, there is ambiguity for a UE to determine the reception / transmission opportunity upon reconfiguration of SPS or Type 2 configured scheduling.

Observation 2	In LTE, SPS (for both UL and DL) reconfiguration is prohibited in order to avoid the ambiguity to determine the transmission/reception opportunities.

Observation 3	In NR, SPS assignment (or configured grant Type 2) can be used for services (e.g. VoIP, URLLC) of various QoS requirements.

Observation 4	Single configuration of SPS (or Type 2 configured scheduling) cannot fulfil the data transmission requirement upon service (e.g. eMBB/VoIP   URLLC) change of a UE.

Observation 5	Reconfiguration of SPS and configured grant Type 2 is needed in NR.

Based on the discussion in section 2 we propose the following:

Proposal 1	NR allows for reconfiguration of SPS and configures grant Type 2

Proposal 2	Upon reconfiguration of SPS (or Type 2 configured scheduling), a UE shall

a.	continue to maintain the active SPS assignment (or configured uplink grant Type 2) based on the old configuration until a new activation/reactivation command is received;

b.	apply the new configuration when a new activation/reactivation command is received.

Proposal 3	RAN2 supports the correction in [2]

# References

R2-115891, Details on SPS reconfiguration, 3GPP TSG-RAN WG2 Meeting #76

R2-1810244, Correction of SPS Reconfiguration, Ericsson