3GPP TSG-RAN WG2 AH 1807	Tdoc R2-1810187

Montreal, Canada, 2nd – 6th July 2018

Agenda Item:	10.3.1.7

Source:	Ericsson

Title:	URLLC MCS table

Document for:	Discussion, Decision

# Introduction

At RAN1#93, RAN1 has made some agreements concerning the new MCS table due to the fact the existing MCS tables are insufficient for users at the cell edge.  These RAN1 agreements are highlighted as below

Agreements:

- For URLLC, for grant-based transmissions, introduce one RRC parameter for configuring a new RNTI.

  - When the new RNTI is not configured, existing RRC parameter mcs-table is extended to select from 3 MCS tables (existing 64QAM MCS table, existing 256QAM MCS table, new 64QAM MCS table).

    - When mcs-table indicates the new 64QAM MCS table:

      - For DCI format 0_0/1_0 in CSS, existing 64QAM MCS table is used.

      - For DCI formats 0_0/1_0/0_1/1_1 in USS, new 64QAM MCS table is used.

    - Otherwise, follow existing behaviour.

    - Note: the configuration for DL and UL is separate

  - When the new RNTI (via RRC) is configured, RNTI scrambling of DCI CRC is used to choose MCS table:

    - If the DCI CRC is scrambled with the new RNTI, the new 64QAM MCS table is used; otherwise, follow existing behaviour.

Agreements:

For both initial transmission and re-transmissions for GF scheduling for URLLC,

- For UL configured grant, the MCS table is configured by the existing parameter associated with the RRC configured grant configuration, which is extended to include the new 64QAM MCS table.

- For DL SPS, RRC indicates whether or not the new 64QAM table is configured. The indication for the new MCS table for DL SPS is separate from the one for grant-based DL scheduling.

Based on above agreements, it is observed that

- For dynamic scheduling, the gNB indicates to a UE that the new MCS table shall be used for PUSCH via RRC signalling or a DCI. For latter case, an additional RNTI is defined accordingly.

- For GF transmission, the gNB indicates to a UE that the new MCS table shall be used for PUSCH via RRC signalling.

The gNB indicates the new MCS table that shall be used for PUSCH transmission via RRC signaling or a DCI addressed to an additional RNTI.

At the same time, in RAN1 discussions, companies have agreed that the gNB scheduler can schedule a transmission at whatever BLER target as it chooses meaning that the new MCS table is not coupled with the BLER target 10-5. For cell center UEs, the existing 64QAM MCS table is sufficient for URLLC services, thus there is no need to use the new MCS table. Instead, the existing 64QAM MCS table can be scheduled with the BLER target 10-5. While for cell edge UEs, the gNB scheduler may choose to use the new MCS table since the existing MCS table is insufficient in this case.

The new MCS table is not coupled with the BLER target 10-5.

At RAN2#102, RAN2 has discussed whether the existing LCP procedure needs to be updated considering the impact of the new MCS table [1]. However, there is no conclusion on this issue since RAN2 would like to wait for RAN1 agreements.

Therefore, considering the progress made by RAN1 on this issue, this paper further analyses the potential impact of the new MCS table on MAC including aspects

1.	How does the MAC spec adopt the new RNTI?

2.	Does the LCP procedure need to be updated accordingly?

# Discussions

## Impact of the new RNTI on MAC

Upon reception of a DCI addressed to the new RNTI, the UE can determine the transport block size and the MCS for the transmission by searching in the new MCS table. After that, the PHY layer informs the MAC layer of the determined information, the MAC layer triggers the LCP procedure and prepares the MAC PDU accordingly. In the actual transmission, the ordinary C-RNTI is used to scramble the CRC of the data. The procedure for the data transmission is the same regardless whether the new MCS table or the existing MCS table is used for PUSCH transmission. Given that the only intention of the new RNTI is to indicate usage of the new MCS table, the new RNTI is unnecessary to be visible at the MAC layer. It is sufficient that the PHY layer is aware of the new RNTI.

The MAC spec doesn’t need to capture the new RNTI.

It is noteworthy that visibility of an RNTI at MAC would have had large specification impact (many references to e.g. “C-RNTI” in many sections in the specification), which should be avoided, unless clearly motivated.

## Impact of the new MCS table on LCP

In [1], a concept on the uplink grant class was proposed for LCP procedure to reflect the restriction of the transmission reliability. The outcome from that discussion was to wait for RAN1 agreements. From our analyses, it is expected that neither the new MCS table nor the new RNTI can be used to represent the transmission reliability, e.g., the BLER target. Therefore, they cannot be considered as additional LCP restriction parameters.

Neither the new MCS table nor the new RNTI shall be used as additional LCP mapping restriction for transmission reliability.

An alternative cleaner solution to enable grants only of a specific service (e.g. URLLC) would be to introduce a service specific grant itself. The grant by itself can indicate the intended set of LCHs for PUSCH transmission. However, such solution requires more standardization efforts and shall be carefully investigated in future releases.

Meanwhile, the existing LCP mapping restrictions such as numerology, and PUSCH transmission duration can be used to achieve the mapping restriction for transmission reliability. For example, the network can configure eMBB requiring the BLER target 10-1 to be mapped with a low SCS, and a long PUSCH duration so that eMBB is not allowed to use the grant which is associated with the BLER target 10-5 and a short PUSCH duration. Such option may lose flexibility to some extent; however, it is sufficient for Rel-15 considering the limited time left to finalize Rel-15. Therefore, we think no further enhancements on the LCP procedure are needed for transmission reliability in the current release.

Enhancement on the existing LCP procedure can be considered in future releases.

# Conclusion

We have observed:

Observation 1	The gNB indicates the new MCS table shall be used for PUSCH transmission via RRC signaling or a DCI addressed to an additional RNTI.

Observation 2	The new MCS table is not coupled with the BLER target 10-5.

We propose:

Proposal 1	The MAC spec doesn’t need to capture the new RNTI.

Proposal 2	Neither the new MCS table nor the new RNTI shall be used as additional LCP mapping restriction for transmission reliability.

Proposal 3	Enhancement on the existing LCP procedure can be considered in future releases.

# References

- R2-1808575	On impacts of new MCS table for URLLC on MAC procedures,	Qualcomm Incorporated