3GPP TSG-RAN WG2 Meeting NR Adhoc 1807	R2-1810285

Montreal, Canada, 2nd - 6th July 2018

Agenda item:	10.4.1.3.7

Source:	Nokia, Nokia Shanghai Bell

Title:	Wait time in RRC Release

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

In RAN2#101bis meeting the following was agreed:

Agreements

1	As in LTE, the maximum waitTime value in REJECT kind of message in response to RRCResumeRequest over SRB0 is 16 seconds.

2	For Rel-15, we do not support RRCREJECT over SRB1.

FFS Whether a wait timer is needed in RRCRelease

In this contribution we discuss further the need for wait time in the RRC release.

# 2	Discussion

The following requirement is specified in TS 23.501 [1]:

#### 5.19.5.2	AMF Overload Control

…

Using the overload start procedure, the AMF can request the 5G-AN node to:

…

c)	release 5G-AN signalling connection where the Requested NSSAI at AS layer only includes the indicated S-NSSAI(s) in the N2 overload control message.

…

When releasing a 5G-AN signalling connection for the case c) above, the 5G-AN indicates to the UE an appropriate wait timer that limits further 5G-AN signalling connection requests which are related only to the S-NSSAIs in the Requested NSSAI until the wait timer expires.

According to above requirement wait timer is required in the RRC Release message. Therefore, we propose the following:

Proposal 1: Wait time can be included in the RRC Release message.

# 3	Conclusion

Based on the discussion the following is proposed:

Proposal 1: Wait time can be included in the RRC Release message.

# References

- 3GPP TS 23.501: " System Architecture for the 5G System".