3GPP TSG-RAN WG2 Meeting NR Adhoc 1807	R2-1810295

Montreal, Canada, 2nd - 6th July 2018	Revision of R2-1808521

Agenda item:	10.4.1.6.1

Source:	Nokia, Nokia Shanghai Bell

Title:	SI contents for NSA and SA/NSA network sharing

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

During the discussions on System Information contents during the RAN2#102 meeting, it was decided to make TAC optional in SIB1. This is justified by the fact that TAC has no meaning in case the NR cell supports only EN-DC operation and would not make sense to configure and broadcast it. In relation to this topic, this paper discusses some aspects related to standalone/non-standalone (SA/NSA) network sharing deployments.

# 2	Discussion

SIB1 may be provided in NSA deployments for the sake of ANR procedure. The need for an indication of whether the reported cell support SA and/or NSA was discussed as part of an e-mail discussion on ANR [1]. It should be however noted that a NR cell, which is purely a NSA cell does not need to broadcast Tracking Area Code in SIB1 as there is no use of this field for en-gNB. Furthermore, lack of tracking area code (TAC) in SIB1 for a certain PLMN ID may be translated into this cell being deployed as NSA cell for this PLMN.

Proposal 1: Lack of trackingAreaCode in the PLMN-IdentityInfoList for a certain PLMN(s) can be translated as “this cell is an NSA cell for those PLMNs”. No additional SA/NSA indication is required for ANR purposes in CGI reporting procedure.

It should be also noted that in case network sharing is applied, some operators sharing the gNB might want to use it as en-gNB while other operators may want to use the same gNB in a standalone mode depending on their network deployments (e.g. 5GC availability). In such situations SIB1 would contain all the information required by the UE to access the cell including RACH configuration and other common serving cell configuration relevant for SIB. This may lead to a situation where standalone operation capable UE with a selected PLMN that uses some specific cells in NSA mode only, may attempt to access this cell in SA mode. Such connection attempt would be then rejected by the network, but unnecessary signalling would have to be already exchanged between the gNB and UE and Core Network. Therefore, such UEs should be prevented from attempting to access the NSA cell. Since TAC should not be provided for the PLMN(s) which utilize the cell in NSA mode only, lack of TAC can also be used by the UE to know that it should not attempt to perform initial access in such cell.

Observation 1: Lack of TAC within SIB1 for a certain PLMN can be used by the UE to know that it should not attempt to perform initial access in such cell.

Based on that we propose to clarify in the field description of TAC that if TAC is not signalled for a PLMN in SIB1, UE shall not perform initial access in this cell when the UE is attached to this PLMN.

Proposal 2: Capture the UE behaviour in the specification (e.g. in TAC field description) that if TAC is not signalled for a PLMN in SIB1, UE shall not perform initial access in this cell when the UE is attached to this PLMN.

The related TP is provided in the Annex.

# 3	Summary

In this paper we discussed a scenario where a NR cell may be used as NSA cell in some PLMNs while it may be used as a SA cell in other PLMNs where multiple PLMNs share the gNB in a deployment where the RAN is shared. Based on the discussion, the following is proposed:

Proposal 1: Lack of trackingAreaCode in the PLMN-IdentityInfoList for a certain PLMN(s) can be translated as “this cell is an NSA cell for those PLMNs”. No additional SA/NSA indication is required for ANR purposes in CGI reporting procedure.

Observation 1: Lack of TAC within SIB1 for a certain PLMN can be used by the UE to know that it should not attempt to perform initial access in such cell.

Proposal 2: Capture the UE behaviour in the specification (e.g. in TAC field description) that if TAC is not signalled for a PLMN in SIB1, UE shall not perform initial access in this cell when the UE is attached to this PLMN.

# References

- R2-1807361 Report of email discussion [101bis#47][NR] ANR, Vivo

# Annex

#### –	PLMN-IdentityInfoList

Includes a list of PLMN identity information.

PLMN-IdentityInfoList information element

-- ASN1START

-- TAG-PLMN-IDENTITY-LIST-START

PLMN-IdentityInfoList ::=				SEQUENCE (SIZE (1..maxPLMN-Info)) OF PLMN-IdentityInfo

PLMN-IdentityInfo ::=					SEQUENCE {

plmn-IdentityList							SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity,

trackingAreaCode							TrackingAreaCode OPTIONAL,  -- Cond non-ENDC

cellIdentity								CellIdentity,

cellReservedForOperatorUse 					ENUMERATED {reserved, notReserved},

...

}

-- TAG-PLMN-IDENTITY-LIST-STOP

-- ASN1STOP

PLMN-IdentityInfoList field descriptions

trackingAreaCode Indicates Tracking Area Code to which the cell indicated by cellIdentity field belongs. If it is absent, the UE should not attempt to access this cell.

| Conditional Presence | Explanation |
| --- | --- |
| Non-ENDC | The field is mandatory present, need S, if the cell supports initial access for the UEs with the selected PLMN which it relates to. Otherwise it is absent. |