3GPP TSG RAN WG2 Meeting #AH                                                            R2- 1810410 
Montreal, Canada, 2nd – 6th July 2018

Title:	(Draft) LS on fallback interpretation of UE capability fields

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	Intel Corporation

To:	RAN1, RAN4

Contact Person:

#### Name:	Naveen Palle

Tel. Number:	+1 224 715 6778

E-mail Address:	naveen [DOT] palle [AT] intel [DOT] com

# 1	Overall description

RAN2 has agreed for NR capability reporting, the UE is not needed to report lower order CA BCs with the assumption that the UE should support the fallback configurations of higher order CA BCs. In this aspect, RAN2 would like RAN1/RAN4 to confirm on the below understanding of the fallback for certain capability fields.

## 1.1	Fallback interpretation of FeatureSetDownlinkPerCC and FeatureSetUplinkPerCC fields

The FatureSetDownlinkPerCC and FeatureSetUplinkPerCC structures are linked from FeatureSetDownlinkPerBand and FeatureSetUplinkPerBand (respectively) which are in turn linked from the bands of a band combinations supported by the UE.

As part of fallback definition, it is RAN2’s understanding that for MIMO layers and modulation order, the UE is expected to support lower layers and lower modulation order respectively, as allowed by the RAN1/RAN4 specifications than the advertised value in the capability,.

However, when MIMO layer and modulation order are combined with sub-carrier spacing and bandwidth as a UE supported configuration, RAN2 is not sure if the above fallback logic can be applied and requests guidance.

## 1.2	Fallback interpretation of FeatureSetDownlinkPerBand and FeatureSetUplinkPerBand fields

For the parameters that are signalled for each band, RAN2 sees that most of them indicate the support of sub-features through a boolean, or provide the ‘maximum’ supported quantity of ports or reports or resources,  where the ‘maximum’ implies that the UE is expected to also support a configuration with lower number of reported capability (as allowed by the specification). For these RAN2 expects the UE to support lower values of the reported maximum capabilities as allowed by the specification.

## 1.3 Questions to RAN1/RAN4

Q1. RAN2 would like to request RAN1/RAN4 on whether the UE is expected to support values lower than the UE advertised capability for the following (as allowed by the RAN1/RAN4 specifications):

- MIMO layers

- Modulation order

- Bandwidth

- Subcarrier spacing

As an example, if the UE advertises in a particular FeatureSetDownlinkPerCC a capability configuration :{ 4 layer MIMO, 256 QAM, 100 MHz, 30 KHz }, is the UE expected to also support all the configurations where any of the fields contain lower values than advertised:

- { 4 layer MIMO, 64 QAM, 100 MHz, 30 KHz }  (Modulation order has lower value)

- { 2 layer MIMO, 256 QAM, 100 MHz, 30 KHz }  (MIMO layers has lower value)

- { 4 layer MIMO, 256 QAM, 80 MHz, 30 KHz }  (BW has lower value)

- { 4 layer MIMO, 256 QAM, 100 MHz, 15 KHz }  (SCS has lower value)

- { 2 layer MIMO, 64 QAM, 100 MHz, 15 KHz }   (MIMO layers, modulation order and SCS are of lower value)

- Etc…

- RAN2 considers that the following parameters reported from FeatureSetDownlinkPerBand and FeatureSetUplinkPerBand are consider as the maximum supported values by the UE and that the UE can support a configuration where the values are lower than the UE reported values (as allowed by the RAN1/RAN4 specifications). RAN2 requests RAN1/RAN4 to take this into consideration and to report if the RAN2 interpretation on any of the parameters below is incorrect.

UL

CSI-ReportFramework

maxNumberPeriodicCSI-ReportPerBWP

maxNumberAperiodicCSI-ReportPerBWP

maxNumberSemiPersistentCSI-ReportPerBWP

simultaneousCSI-ReportsAllCC

SRS-Resources

maxNumberAperiodicSRS-PerBWP-PerSlot

maxNumberPeriodicSRS-PerBWP

maxNumberPeriodicSRS-PerBWP-PerSlot

maxNumberSemiPersitentSRS-PerBWP

maxNumberSP-SRS-PerBWP-PerSlot

maxNumberSRS-Ports-PerResource

DL

CSI-RS-IM-ReceptionForFeedback

maxNumberNZP-CSI-RS-PerCC

maxNumberPortsAcrossNZP-CSI-RS-PerCC

maxNumberCS-IM-PerCC

maxNumberSimultaneousCSI-RS-ActBWP-AllCC

totalNumberPortsSimultaneousCSI-RS-ActBWP-AllCC

TypeI-SinglePanelCodebook

maxNumberTxPortsPerResource

maxNumberResources

totalNumberTxPorts

maxNumberCSI-RS-PerResourceSet

TypeI-MultiPanelCodebook

maxNumberTxPortsPerResource

maxNumberResources

totalNumberTxPorts

maxNumberCSI-RS-PerResourceSet

TypeII-Codebook

maxNumberTxPortsPerResource

maxNumberResources

totalNumberTxPorts

maxNumberCSI-RS-PerResourceSet

TypeII-CodebookPortSelection

maxNumberTxPortsPerResource

maxNumberResources

totalNumberTxPorts

maxNumberCSI-RS-PerResourceSet

# 2	Actions

ACTION to RAN1 and RAN4:

RAN2 respectfully asks inputs on Q1 and Q2.

# 3	Date of Next RAN2 Meetings:

RAN2#103			20 - 24 Aug 2018 	Gothenburg, Sweden

RAN2#103bis		8 - 12 Oct 2018   Chengdu, China