3GPP TSG-RAN WG2 Meeting #AH-1807	R2-1810456

Montreal, Canada, 2nd – 6th Jul 2018

Agenda item:	10.3.1.3

Source:	Qualcomm Incorporated

Title:	Corrections on SP CSI reporting on PUCCH Activation/Deactivation MAC CE

WID/SID:	NR_newRAT-Core – Release 15

Document for:	Discussion and Decision

# Introduction

This contribution addresses one correction request on the text description and MAC CE format of the SP CSI reporting on PUCCH Activation/Deactivation MAC CE from the latest TS 38.321 [1]. A text proposal is provided to capture the corrections in the Annex.

# Discussion

In the latest TS 38.321 [1], the SP CSI reporting on PUCCH Activation/Deactivation MAC CE is defined with the field of Serving Cell ID, BWP ID, Si and R. Among them, the Si field is used to indicate the activation/deactivation status of the Semi-Persistent CSI report configuration within csi-ReportConfigToAddModList, as specified in TS 38.331 [2]. The Semi-Persistent CSI report consists of two reporting types, one is “semiPersistentOnPUCCH” and the other is “semiPersistentOnPUSCH” [3]. As introduced in [1], Si refers to the report configuration within the list with type set to “semiPersistentOnPUCCH”, and S0 refers to the first report, and S1 refers to the second report and so on. In the Figure 6.1.3.16-1 of TS 38.321 [1], the MAC CE format has 8 Si bitmap to present the status of report configuration within the list with type set to “semiPersistentOnPUCCH”.

The possible ambiguous issue on this MAC CE is whether the up to 8 Si bitmap is needed or not for the SP CSI reporting on PUCCH indication. Given the latest TS 38.331 [2], the csi-ReportConfigToAddModList is defined under CSI-MeasConfig and it can be configured with maximum number maxNrofCSI-ReportConfigurations of CSI-ReportConfig. According to [2], four report config types (i.e. “periodic”, “semiPersistentOnPUCCH”, “semiPersistentOnPUSCH” and “aperiodic” in reportConfigType) should share the maxNrofCSI-ReportConfigurations of CSI-ReportConfig.

It should be noted that RAN1 has sent an LS to RAN2 in last meeting on the updated NR feature list (from RAN1 point of view) [4] [5]. In the #2-35 of table of [5], the candidate values of CSI report setting for each report type are provided. The table shows that for the periodic and aperiodic CSI report setting per BWP, the maximum number is 4 separately, and the candidate values of the number of SP CSI report setting per BWP are {0, 1, 2, 3, 4}. The exact value depends on the UE capability. So, considering the SP CSI report on PUCCH resource per BWP, 4 bits are enough to express the SP CSI reporting on PUCCH status.

Taking all the above analysis into account, we propose to use 4 bits to indicate the activation/deactivation status of the Semi-Persistent CSI report configuration within the list with type of "semiPersistentOnPUCCH" in SP CSI reporting on PUCCH Activation/Deactivation MAC CE.

Proposal 1: Use 4 bits to indicate the activation/deactivation status of the Semi-Persistent CSI report configuration within the list with type of "semiPersistentOnPUCCH" in SP CSI reporting on PUCCH Activation/Deactivation MAC CE.

The proposed correction is shown in the TP provided in Annex. The only change is the MAC CE format compared to [1], and four reserved bits are filled in the second octet.

Proposal 2: RAN2 adopts the text proposal and new MAC CE format in Annex.

# Summary

Based on the above discussions, we recommend RAN2 to discusses the following proposals:

Proposal 1: Use 4 bits to indicate the activation/deactivation status of the Semi-Persistent CSI report configuration within the list with type of "semiPersistentOnPUCCH" in SP CSI reporting on PUCCH Activation/Deactivation MAC CE.

Proposal 2: RAN2 adopts the text proposal and new MAC CE format in Annex.

# References

[1] TS 38.321, Medium Access Control (MAC) protocol specification, 15.2.0.

[2] TS 38.331, Radio Resource Control (RRC) protocol specification, 15.2.0.

[3] R1-1807958, CR to 38.214 capturing the RAN1#92bis and RAN1#93 meeting agreements.

[4] R1-1807896, LS on NR UE feature list.

[5] R1-1807891, Rel-15 NR UE feature list.

# Annex – Text Proposal

#### 6.1.3.16	SP CSI reporting on PUCCH Activation/Deactivation MAC CE

The SP CSI reporting on PUCCH Activation/Deactivation MAC CE is identified by a MAC PDU subheader with LCID as specified in Table 6.2.1-1. It has a fixed size of 16 bits with following fields:

-	Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits;

-	BWP ID: This field contains BWP-Id, as specified in TS 38.331 [8], of a downlink bandwidth part for which the MAC CE applies. The length of the BWP ID field is 2 bits;

-	Si: This field indicates the activation/deactivation status of the Semi-Persistent CSI report configuration within csi-ReportConfigToAddModList, as specified in TS 38.331 [8].  S0 refers to the first report configuration within the list with type set to "semiPersistentOnPUCCH", S1 to the second report configuration within the list with type set to "semiPersistentOnPUCCH" and so on. The Si field is set to "1" to indicate that the Semi-Persistent CSI report configuration i shall be activated. The Si field is set to "0" to indicate that the Semi-Persistent CSI report configuration i shall be deactivated. If there is no Semi-Persistent CSI report configuration i within the list, MAC entity shall ignore this field;

-	R: Reserved bit, set to "0".

Figure 6.1.3.16-1: SP CSI reporting on PUCCH Activation/Deactivation MAC CE