3GPP RAN WG2 NR Ad-Hoc	     																				R2-1810475

Montreal, Canada, 2nd – 6th July 2018

Agenda item:		10.4.4.1

Source:		Intel Corporation

Title:			Update of NR UE feature list - RAN2

Document for:	Discussion and decision

# It is to update NR UE feature list – RAN2 accroding to the discussion and decision from latest RAN (RP-181387 and RP-181397). The intention is to maintain NR UE feature list – RAN2 for both EN-DC and SA in order to track the decisions easily.

# Note if new UE capabilities for SA are decided, it will be indicated in the column of “Note” indicating it is for SA.

| Features | # | Feature group | Components | Prerequisite feature groups (listed in this sheet only) | Need for gNB to know whether the feature is supported by the UE (what happens if eNB/gNB does not know?) | Consequences if the feature is not supported by the UE | Type (see R2-1712078) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | RAN5 implication | Note | Responsible WG | RAN WG recommendation | TSG-RAN decision |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0. General (including supported bearer types) | 0-1 | Access stratum release | Access stratum release |  | Yes | Configuration specific to a release is not possible | Type 4 | No | No |  |  | RAN2 | Optional with capability signaling and candidate value set is {Rel-15, spare7, … , spare1} |  |
|  | 0-2 | SRB | 1) Split SRB with one UL path 2) SRB3 |  | Yes | 1) Configuration of split SRB with one UL path is not possible 2) Configuration of SRB3 is not possible | Type 4 | Yes | No |  |  | RAN2 | 1) Optional with capability signaling 2) Mandatory with capability signaling |  |
|  | 0-3 | DRB | 1) Maximum number of DRBs 2) Split DRB with one UL path 3) Split DRB with both UL MCG and SCG paths |  | Yes | 1) Configuration of DRB may be failed 2) Configuration of split DRB with one UL path is not possible 3) Configuration of split DRB with both UL MCG and SCG paths is not possible | Type 4 | Yes | No |  | 2) 8 DRBs are supported regardless of bearer types | RAN2 | 1, 2) Mandatory without UE capability signaling 3) Mandatory with capability signaling |  |
|  | 0-4 | Direct SN addition in the first RRC connection reconfiguration after RRC connection establishment | Direct SN addition in the first RRC connection reconfiguration after RRC connection establishment |  | Yes | SN addition in the first RRC connection reconfiguration may be failed | Type 4 |  |  |  |  | RAN2 | Mandatory without UE capability signaling |  |
|  | 0-5 | IMS voice | 1) IMS voice over NR PDCP on MCG bearer 2) IMS voice over NR PDCP on SCG bearer |  | Yes | IMS voice over NR PDCP is not supported | Type 4 | Yes | No |  |  | RAN2 | Conditional mandatory with capability signaling if UE is IMS voice capable in NR SA. Otherwise optional with capability signaling. |  |
| 1. PDCP | 1-1 | ROHC context | 1) Maximum number of ROHC context sessions 2) Supported ROHC profiles |  | Yes | Configuration of ROHC context sessions may be failed | Type 4 | No | No |  |  | RAN2 | Optional with capability signaling and candidate value set is: 1) {cs2, cs4, cs8, cs12, cs16, cs24, cs32, cs48, cs64, cs128, cs256, cs512, cs1024, cs16384, spare2, spare1} 2) {0x0000, 0x0001, 0x0002, 0x0003, 0x0004, 0x0006, 0x0101, 0x0102, 0x0103, 0x0104} |  |
|  | 1-2 | ROHC context continuation operation | ROHC context continuation operation |  | Yes | Configuration of DRB ROHC continuation is not possible. | Type 4 | No | No |  |  | RAN2 | Optional with capability signaling |  |
|  | 1-3 | Uplink only ROHC Profiles | Uplink only ROHC Profiles |  | Yes | Configuration of uplink only ROHC operation is not possible. | Type 4 | No | No |  |  | RAN2 | Optional with capability signaling |  |
|  | 1-4 | Out of order delivery | Out of order delivery |  | Yes | Configuration of out of order delivery is not possible | Type 4 | No | No |  |  | RAN2 | Optional with capability signaling |  |
|  | 1-5 | Short SN | Short SN |  | Yes | Configuration of short SN is not possible | Type 4 | No | No |  |  | RAN2 | Mandatory with capability signaling |  |
| 2. RLC | 2-1 | RLC AM with short SN | RLC AM with short SN |  | Yes | Configuration of RLC AM with short SN is not possible | Type 4 | No | No |  |  | RAN2 | Mandatory with capability signaling |  |
|  | 2-2 | RLC UM with short SN | RLC UM with short SN |  | Yes | Configuration of RLC UM with short SN is not possible | Type 4 | No | No |  |  | RAN2 | Mandatory with capability signaling |  |
|  | 2-3 | RLC UM with long SN | RLC UM with long SN |  | Yes | Configuration of RLC UM with long SN is not possible | Type 4 | No | No |  |  | RAN2 | Mandatory with capability signaling |  |
| 3. MAC | 3-1 | LCP restriction | LCP restriction |  | Yes | Configuration of mapping restrictions for each LCH is not possible | Type 4 | No | No |  |  | RAN2 | Optional with capability signaling |  |
|  | 3-2 | LCH SR delay timer | LCH SR delay timer |  | Yes | Configuration of LCH SR delay timer is not possible | Type 4 | Yes | No |  |  | RAN2 | Optional with capability signaling |  |
|  | 3-3 | DRX | 1) DRX with long DRX cycle 2) DRX with short DRX cycle |  | Yes | Configuration of DRX is not possible | Type 4 | Yes | No |  |  | RAN2 | Mandatory with capability signaling |  |
|  | 3-4 | Configured grants | Maximum number of configured grant configurations per cell group |  | Yes | Configuration of configured grant may be failed | Type 4 | Yes | No |  |  | RAN2 | Optional with capability signaling |  |
|  | 3-5 | SR | Multiple SR configurations |  | Yes | Configuration of per cell group is not possible | Type 4 | Yes | No |  |  | RAN2 | Optional with capability signaling |  |
|  | 3-6 | Skipping UL transmission | 1) Skipping UL transmission for dynamic UL grant 2) Skipping UL transmission for configured UL grant |  | Yes | Configuration of skipping UL transmission is not possible | Type 4 | 1) Yes 2) No | No |  | 1) Mandatory with capability signaling from Rel-16 | RAN2 | 1) Optional with capability signaling 2) Conditional mandatory if the UE supports configured grant |  |
| 4. Measurements | 4-1 | Intra-NR measurements and reports | 1) Intra-frequency and inter-frequency measurements and reports 2) Event A-based measurement and measurement report |  | Yes | 1) Configuration for intra-frequency and/or inter-frequency is/are not possible 2) Configuration for event A-based measurement is not possible | Type 4 | Yes | No |  |  | RAN2 | Mandatory with capability signaling |  |
|  | 4-2 | Inter-NR measurement and reports while in LTE connected | 1) NR measurement and reports while in LTE connected 2) Event B1-based measurement and reports while in LTE connected |  | Yes | 1) Configuration for NR measurement is not possible 2) Configuration for event B1-based measurement is not possible | Type 4 |  |  |  |  | RAN2 | Mandatory without capability signaling |  |
|  | 4-3 | SFTD measurements | 1) SFTD measurements between PCell and PSCell 2) SFTD measurements between PCell and NR Cell |  | Yes | Configuration for SFTD is not possible | Type 4 | Yes | No |  |  | RAN2 | Optional with capability signaling |  |
|  | 4-4 | Measurement gaps | Additional measurement gap configurations |  | Yes | Configuration for additional measurement gap is not possible | Type 4 | No | No |  |  | RAN2 | Optional with capability signaling and candidate value set is: BIT STRING (SIZE (22)) |  |