3GPP TSG-RAN WG2 Meeting NR Adhoc 1807	R2-1810600

Montreal, Canada, 2nd - 6th July 2018

Agenda item:	10.3.1.2

Source:	Nokia, Nokia Shanghai Bell

Title:	DL BWP for contention resolution

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

In the context of BWP discussion, the following was added in the last meeting in the BWP section assuming DL BWP for RACH always from SpCell [1]:

1>	perform the Random Access procedure on the active DL BWP of SpCell and active UL BWP of this Serving Cell .

It is true that RAR for RA from SCell is scheduled from SpCell, but it is not clear it should be also applicable to PDCCH for contention resolution.

# 2	Discussion

In the Contention Resolution section in current specification [2], it has been assumed PDCCH from any cell can serve the purpose of Contention Resolution. Since the PDCCH addressed to C-RNTI is on UE specific search space, there is no reason to restrict it only to SpCell.

### 5.1.5 Contention Resolution Contention Resolution is based on either C-RNTI on PDCCH of the SpCell or UE Contention Resolution Identity on DL-SCH. Once Msg3 is transmitted, the MAC entity shall: 1> start the ra-ContentionResolutionTimer and restart the ra-ContentionResolutionTimer at each HARQ retransmission in the first symbol after the end of the Msg3 transmission; 1> monitor the PDCCH while the ra-ContentionResolutionTimer is running regardless of the possible occurrence of a measurement gap; 1> if notification of a reception of a PDCCH transmission is received from lower layers:

Similar issue has been discussed for LTE as well [3]. It was concluded to keep MAC spec as it is which allows PDCCH from any cell as Contention Resolution. Same should be applicable to NR.

Proposal 1: PDCCH for Contention Resolution can be from any cell.

Which cell for each step of the RA procedure should be clear in the RACH section, hence no need to mention it in BWP section. BWP section only needs to take care of the necessary switching.

Proposal 2: Keep the Contention Resolution section unchanged and the BWP section should be modified accordingly.

Upon initiation of the Random Access procedure on a Serving Cell, the MAC entity shall for this Serving Cell: 1> if PRACH occasions are not configured for the active UL BWP: 2> switch the active UL BWP to BWP indicated by initialUplinkBWP; 2> if the Serving Cell is a SpCell: 3> switch the active DL BWP to BWP indicated by initialDownlinkBWP. 1> else: 2> if the Serving Cell is a SpCell: 3> if the active DL BWP does not have the same bwp-Id as the active UL BWP: 4> switch the active DL BWP to the DL BWP with the same bwp-Id as the active UL BWP. 1> perform the Random Access procedure as specified in subclause 5.

# 3	Conclusion

PDCCH for contention resolution is discussed in this contribution with the following proposals proposed and corresponding CR provided in [4].

Proposal 1: PDCCH for Contention Resolution can be from any cell.

Proposal 2: Keep the Contention Resolution section unchanged and the BWP section should be modified accordingly.

# References

- R2-1808830, Corrections on BWP Switching Upon Initiation of Random Access Procedure, Samsung

- 3GPP TS38.321 v15.2.0, NR; Medium Access Control (MAC) protocol specification

- R2-105201, RAN2 #71 LTE UP minutes, Vice-Chairman

- R2-1810601, CR on DL BWP for contention resolution, Nokia, Nokia Shanghai Bell